Student’s Name: Shanice Fezeu

Student’s Major: Biology

Faculty Mentor’s Name: Dr. Changyong Qin

Presentation Category: Chemistry

Poster Presentation Only

Exfoliation of Graphite through Electrochemical Intercalation in Molten Carbonate

Graphene is a two-dimensional single atomic layer of carbon and has many unique physical and chemical properties. So far, graphene has been applied to many fields such as semiconductor, electronics, energy conversion, catalysis and so on. However, graphene is tough in terms of large-scale production like many other nanomaterials. The Hummer’s method is mostly used, but the procedure is too long and also it produces significant amount of waste. A new green method for graphene production is highly demanded.

In this project, we have developed a new electrochemical method to expand the graphite through the intercalation of molten carbonate salts. Such electrochemical intercalation is simple and of high efficiency. In the experiment, molten carbonate salts, Li2CO3, Na2CO3, and K2CO3, were mixed at a ratio of 3.2:3.32:3.48 (g), respectively. A two-electrode system was set up using silver and graphite rod. The mixed carbonate was melted at 500oC. The cyclic voltammetry method was used to find the potential range. Then, a potentiostatic method was used to begin the reaction until the current change was obvious. The potential range was found to be approximately between 0.7 and 2.4 voltages. In this range, there is a peak which in turn means there is a chemical reaction taking place. Potential voltage of 2.0V was then selected for the potentiostatic method. Subsequently, a change was observed during 540-600 seconds. Next, the sheets of graphene in the molten carbonate will be collected and characterized for different physical and chemical properties.

Overall, the current project went very well and the future work should be concentrated on the temperature of the reaction and the applied voltage.