Student’s Name :  Rose Blandith Exantus

Student’s Major :  Environmental Health Science

Faculty Mentor’s Name:  Dr. Milton Morris

Presentation Category:  Environmental Engineering & Science

Poster only

Title : Influence of land slope and soil texture on flooding.

Abstract

This study focuses on the effects of differents types of soil textures contributions to flooding.  The aim is to demonstrate how the slope of the land and the types of soil will make flooding more or less likely to occur. Soil is the material found on the surface of the earth, also is made of both living and dead plants, animals and mineral particles such as Sand, Silt and Clay. Flood simulation based on soil textures in the lab, we placed 10 ml of each soil texture into 40 ml beaker slowly extracted and pipetted water. Carefully recorded the amount of water needed just to cover 10 ml, to reach twice and to reach triple times the height of the soil. Based on the field work, we analyzed site to locate area with continuous slope. We Calibrated transit to insure 360 degree level reading. We measured 1200 inches of vertical land distance and 600 inches of horizontal land distance and calculated the slope of the land as a percentage of surveyed line using mathematical analysis also conducted runs by pouring 1 gallon of water at consistent location and we determined slope impact by measuring and recording vertical and horizontal distances traveled by water. Soils with larger particle size such as sand will absorb more water and thus reduce the extent of flooding, therefore flooding will occur sooner in a clay soil because the particles are tightly close together.

Key Words : Flood, Soil Texture, Land slope, Sand, Silt, Clay.