Student’s Name: Hector Hinestroza

Student’s Major: Computer Engineering

Faculty Mentor’s Name: Dr. Jessica Furrer

Presentation Category: Environmental Engineering & Science

Oral/poster Presentation: Poster

Implementation of EPS Dynamics In Computational Model

The purpose of this research is to investigate and quantify bacterial extracellular polymeric substance (EPS, or biofilm) properties and to incorporate them into a Lattice Boltzmann model for pore-scale fluid dynamics. EPS is produced by soil bacteria and has been shown to alter soil moisture retention. It has the ability to take up water, resulting in swelling, and it retains and then releases water to the soil, resulting in shrinking. We took an existing multiphase Lattice Boltzmann code and added EPS as a 4th phase (solid, liquid, vapor, EPS). According to studies, EPS is capable of retaining up to 70 kg water/kg of EPS (Chenu, 1993; Rosenzweig et al., 2011). We used data from Rosenzweig et al., scaled their results to fit Lattice Boltzmann units and found a new power rule in order to find EPS potential, which we use to calculate and implement water exchange between EPS and fluid phase. Preliminary testing of the model showed exchange of water between EPS and fluid based on the concentration of EPS inside the model. Experiments of drying flux were conducted with both plain EPS, and plain water inside capillary tubes to parametrize the model. Ongoing work includes testing EPS inside complex soil geometries and parameterizing its behavior to real EPS behavior.