Name: _________________________________________________________________

Activity 1.1.1 Environmental Problems

Purpose

Clean water is necessary for healthy ecosystems. You may or may not consider rain an environmental problem, but when pollutants alter the physical or chemical properties of water, water can harm organisms and ecosystems.

The effects of environmental problems do not follow county, state, or national boundaries. An activity in your area may cause a problem somewhere else. A local environmental problem may originate elsewhere. Increasing carbon dioxide, sulfur dioxide, and nitrogen dioxide concentrations in the atmosphere are good examples of worldwide problems with localized effects. These gases can affect soils and marine life when dissolved in water. Since all humans depend on soil and water for food, the problem is local as well as global.

Human activities, such as burning fossil fuels, contribute carbon dioxide and other gases into the atmosphere. The concentration of carbon dioxide in the atmosphere has increased steadily from 315 parts per million in 1960 to 400 parts per million in 2015. As carbon dioxide drifts through the atmosphere, it mixes with water, forming carbonic acid. Other combustion gases, including sulfur dioxide and nitric oxide, also form acids in water. Precipitation, surface water, and the oceans mix with gases and acidify, producing acid rain, acid runoff, and ocean acidification.

The pH scale indicates the acidity or alkalinity of a substance. Rain is naturally slightly acidic with a common pH range of 5 to 6 and does not generally cause any problems. As rainwater becomes acidified, the pH decreases. When pollutants mix with water in the atmosphere, the acidity will increase and the pH will decrease. High concentrations of gases released by the by-products of combustion can reduce the pH further to 4.

| Figure 1 shows the pH of rainwater across the United States. Most acidic rain occurs over and downwind of heavily populated and industrialized areas, especially the northeastern U.S. Acid rain can cause many environmental problems. Acidified water percolating through acidic soils often removes nutrients, reducing soil fertility. Nutrients leached from soil and into aquatic systems can be toxic to aquatic life. |  |
| --- | --- |
|  | Figure 1. Rainwater Acidity in the United States. |

Acid rain does not always reduce fertility right away. Alkaline soils act as buffers that neutralize acid rain. Over time, the buffering capacity of a soil is decreased.

Measuring the electrical conductivity of water indicates the amount of dissolved chemical in the water. Dissolving chemicals in water increases the conductivity. As acid rainwater removes nutrients from the soil, the concentration of dissolved chemicals in runoff is increased, and the runoff water conductivity increases.

How does carbon dioxide in the air affect the acidity of water? How does acid rain impact local soil and water?

Materials

| Per pair of students: 100ml of pH buffer solution 4 100ml beakers 3 250ml beakers 100ml graduated cylinder Distilled water (dH2O) LabQuest2 pH sensor Conductivity sensor Rinse bottle Alcohol burner with ethanol Butane lighter Large beaker tongs | ½ gallon wide-mouth mason jar with lid Marking pen Laboratory tape Small funnel 2 coffee filters Local soil Plastic spoon Per student: Personal protective equipment (PPE) Pencil |
| --- | --- |

Procedure

Work with a partner to simulate the formation of acid rain and observe how it affects your local soil. It is important to work efficiently with the acid rain as the pH may change over time. Have all materials ready at your workstation. Be sure to read all procedures before starting. Review the ESI Laboratory Safety Manual and use the personal protective equipment provided appropriately.

Part One – Producing Acidified Water

Use the graduated cylinder to measure 100ml of distilled water (dH2O).

Remove the lid from the ½ gallon wide-mouth mason jar.

Place the alcohol burner in a safe location away from your other lab materials and light it with the butane lighter.

Invert the mason jar over the burner. Use the beaker tongs to hold the jar. Keep the jar opening below the level of the flame.

Hold the jar in place until the flame goes out. Keeping the jar inverted, move it away from the burner.

With the jar still inverted, place the lid on the jar and twist tight. Place the jar upright on the tabletop.

Lift the lid up slightly, and pour 100ml of dH2O from the graduated cylinder into the jar. Reseal the jar.

Shake the jar vigorously for 30 seconds. Set the jar on the table and keep it sealed.

Part Two – Measuring Acidity of Water

- Use the marking pen and laboratory tape to label three 250ml beakers as follows.

dH2O

Soaking Solution

Rinse

Use the marking pen and laboratory tape to label four 100ml beakers as follows.

dH2O

Acid rain

Runoff from dH2O

Runoff from acid rain

Use a graduated cylinder to measure 100ml of dH2O and pour into the 250ml dH2O beaker.

Use a graduated cylinder to measure 100ml of pH soaking solution and pour into the Soaking Solution beaker.

Set up the LabQuest2 and sensors.

Turn on the LabQuest2. Plug the pH sensor into Channel 1.

Remove the pH sensor from the storage vial and place in the beaker with pH soaking solution.

Plug the conductivity sensor into Channel 2 on the LabQuest2.

Remove the cap from the conductivity sensor and place the sensor in the beaker with dH2O.

The conductivity sensor cable incorporates a range setting switch. Set the switch to 0-200 µS.

Pour 100ml dH2O into the 100ml beaker labeled dH2O.

Remove the pH sensor from the soaking solution and rinse the end with dH2O using the rinse bottle over the rinse beaker.

Place the pH sensor in the 100ml dH2O beaker, and wait 30 seconds for the reading to stabilize.

Record the pH in the Distilled Water row of Table 1 on the student worksheet.

Rinse the pH sensor with the rinse bottle over the rinse beaker and return to the soaking solution.

Remove the conductivity sensor from the 250ml dH2O beaker.

Place the conductivity sensor in the 100ml dH2O beaker, and wait 30 seconds for the reading to stabilize.

Record the conductivity in the Distilled Water row of Table 1.

Rinse the sensor with the rinse bottle and return the sensor to the 250ml dH2O beaker.

Working quickly, remove the cap from the jar and pour the acidified water into the Acid Rain 100ml beaker.

Repeat Steps 7 – 14 for the acidified water. Record results in the Acidified Water row of Table 1.

While you are collecting data for the acidified water, have your partner set up Steps 1 – 2 of Part Three.

Part Three –Soil Runoff

- Place a coffee filter in the funnel and add four heaping spoonfuls of soil to the funnel.

- Set the funnel on the 100ml beaker labeled Runoff from Acidified Water.

- Pour 100ml of acidified water through the soil in the funnel and collect the runoff in the beaker.

- If less than 50ml of runoff collects in the beaker, slowly add distilled water through the funnel until the beaker contains 50ml of runoff.

- Repeat Steps 7 – 14 of Part Two for the runoff from acidified water. Record results in Table 1.

- Dispose of the used soil and coffee filter as instructed.

- Rinse the funnel with water.

- Repeat Steps 1 – 6 of Part Three replacing acidified water with dH2O.

- Rinse the sensors with dH2O over the rinse beakers and store the sensors.

- Dispose of the solutions as instructed.

- Answer the analysis questions on the student worksheet.

Conclusion

- How does acidified rain affect soil when compared to distilled water?

- How can a distant pollution source affect your local soil and water?

What problem(s) does acid rain pose for the environment?

Why is it important to understand environmental problems, such as acid rain, in order to solve them?

Name: ___________________________________________________________________

Activity 1.1.1 Student Worksheet

| Table 1. pH and Conductivity Data |  |  |
| --- | --- | --- |
| Control Test | pH | Conductivity (µS) |
| dH2O |  |  |
| Runoff from dH2O |  |  |
| Change in DH2O |  |  |
| Variable Test | pH | Conductivity (µS) |
| Acidified Water |  |  |
| Runoff from Acidified Water |  |  |
| Change in Acidified Water |  |  |

Analysis Questions

Describe the pH differences between distilled water and acidified water. What process is producing this change?

Discuss the pH change in runoff from distilled water and acidified water. How did soil filtration affect acidity?

Describe the conductivity changes that occur when the acidified water percolates through the soil and becomes runoff. What is happening to the soil nutrients? What is happening to soil fertility?

Describe any other changes you observe.