Activity 2.1.3 Geographic Information

Purpose

When researching and recording environmental data, it is important to know where the data was collected. What types of technology can you use to identify the location of environmental data?

GPS (global positioning system) technology allows you to pinpoint the exact location on this earth. Each location on the earth has its own physical features which can be measured and analyzed. GPS locations are identified using a latitude and longitude. Information about the location is endless. Soil type, water quality, and soil moisture are few examples researchers can correlate with a location.

Soil moisture and temperature are indicators of biodiversity in the soil. Micro-organisms and plant life are dependent upon water for survival. Soil moisture prevents fluctuations in soil temperature, which increases the reproduction of plants and microbes. An adequate soil moisture percentage is needed to increase the potential for biodiverse microbial growth.

How can you use geographic locations with temperature and soil moisture data to measure biodiversity?

Materials

| Per team of three students: LabQuest2 Soil moisture sensor Temperature sensor Garden trowel | Per student: ESI Notebook Laboratory Notebook Pencil |
| --- | --- |

Procedure

Work with a team of three to collect soil moisture data from the quadrat locations marked in Activity 2.1.1 Biodiversity Data. Use your data to determine the biodiversity of the soil.

Part One – GPS Data

- Prepare the LabQuest2 to measure soil moisture and temperature data.

Plug the soil moisture sensor into Channel 1.

Plug the temperature sensor into Channel 2.

Tap Mode. In the Mode menu, select Events with Entry. In the Number of Columns menu, choose 1.

Tap Name. Type in “Location”. Tap Done. Do not enter a unit.

Check Average over 10 seconds.

Tap OK. The Latitude, Longitude, Temperature, and Soil Moisture values should reappear on the screen.

On your sketch, identify the locations marked with flags.

Go to the A flag marked with your assigned number from Activity 2.1.1 Biodiversity Data.

| At the location, use the garden trowel to make a trench in the ground as seen in Figure 1. Place the soil moisture sensor horizontally in the ground as seen in Figure 2. Insert the temperature sensor vertically into the trench. Compact the soil around the temperature and soil moisture sensor so the compaction is the same as the soil around the trench. See Figure 3. Allow the sensors to stabilize for 30 seconds. Tap the green arrow on the screen to prepare LabQuest2 for data collection. Tap Keep to collect data for the location. Data is collected over 10 seconds. When the Location screen appears, enter the number and letter on the flag (ex. 1A) and tap OK. When the Meter screen appears, tap the Table icon in the upper right corner of the screen. The table should have the location number, latitude, longitude, temperature, and soil moisture data. Tap the meter screen icon. Remove the sensors from the ground. Caution: Loosen the soil around the sensors and do not pull sensors by the wires. Go to the B flag at your site. Repeat Steps 8 – 17 for the location entering the corresponding location number. Repeat Step 8 – 17 for your C flag and then each group of remaining flags. Tap the red stop square in the lower left of the screen to stop data collection. Tap File and choose Save. Type the file name as 2.1.3YourInitials and tap Done. Part Two – Data Recording and Analysis - Tap the Table icon at the top of the LabQuest2 screen. |  |
| --- | --- |
|  | Figure 1. Trowel in Trench |
|  | Figure 2. Horizontal Soil Sensor |
|  | Figure 3. Sensors in Ground |

- Copy the data table into your Laboratory Notebook. You do not need to copy the latitude and longitude coordinates.

Based upon your data, which quadrats have greater potential to be biodiverse? Explain.

What was the relationship between soil moisture and temperature?

How could you compare the soil data with your biodiversity calculations?

Conclusion

- How can GPS be used to measure biodiversity?

- What type of environmental data could you collect using GPS technology?