Activity 2.1.5 Technological Display

Purpose

Researchers present data in many forms. Throughout the CASE courses you have taken, you have developed charts and graphs to analyze data. How could you analyze qualitative and quantitative data represented on a map?

In Activity 2.1.3 Geographic Information, you used a GPS sensor to collect data. Your data can be displayed and analyzed using a Geographic Information System (GIS). GIS software visually displays data sets in layers. A layer contains a set of spatial data such as roads, soils, or waterways. GIS maps display multiple layers for comparison, analysis and reference. A base layer is the first layer used. A base layer is a map with physical features a researcher can use for reference when analyzing his or data.

Each layer contains spatial data called an attribute. Soil moisture is an example attribute you have collected and can display on map. Attributes are visually represented as shapes or colors on a map. Visual images on the map enable researchers to analyze and compare multiple attributes at the same time.

How is data displayed in a GIS map?

Materials

| Per team of three students: LabQuest2 Computer with Logger Pro® and GIS software | Per student: Pencil ESI Notebook Laboratory Notebook |
| --- | --- |

Procedure

Work with your team to develop a map of the biodiversity data you collected in Activity 2.1.1 Biodiversity Data and Activity 2.1.3 Geographic Information.

Part One – Data Import

Go to a computer with Logger Pro® software.

Open Logger Pro® and attach LabQuest2 to the computer using a USB cord.

Choose File, LabQuest Browser, and select Open.

Open the file entitled 2.1.3YourInitials you saved during Activity 2.1.3 Geographic Information.

Click on Data and choose New Manual Column.

Change the Name of column as Biodiversity and change the Short Nm as Bio.

Click Done.

In the Biodiversity column of the data table, enter the Simpsons Index number for each location from Activity 2.1.2 Calculating Biodiversity.

Click and File and choose Save.

Save file as 2.1.5YourInitials.

Choose File, Export As, and select GIS Format.

Type the file name as 2.1.5YourInitials and click Save.

Exit Logger Pro®.

Part Two – Data Upload

- Open a web browser and go to www.arcgis.com.

Log in using a username and password provided by your teacher.

Click on Map at the top of the screen. You will see a map of the United States.

Click on Basemap and choose Imagery.

Click on Add and choose Add Layer from File.

Click Choose file and open 2.1.5YourInititals.

Click Import Layer. Your will see a large dot on your screen. Zoom in on the map to view all dots.

Click on Save and select Save As.

Type in 2.1.5YourInitials and click Save Map.

Part Three – Layers and Attributes

- Click on Details, select Content, More Options (…).

Choose Rename and change the name to Temperature.

Click OK. You now have a Temperature layer for your map.

Under the Temperature layer listed in the Contents, click on More Options (…).

Click on Copy

Rename the copy Soil Moisture.

You now have a soil moisture layer for your map.

| Under the Temperature layer listed in the Contents, click on More Options (…). Click on Copy Rename the copy Biodiversity. You now have a biodiversity layer for your map. Your contents should appear similar to Figure 1. Under the Biodiversity layer click on Change Style. Change the attribute to Biodiversity. Under 2 Select a drawing style, Select Heat Map. Click Done. Under the Soil Moisture layer click on Change Style. Change the attribute to Soil Moisture. Under 2 Select a drawing style, Select Counts and Amounts (size). Click Done. |  |
| --- | --- |
|  | Figure 1. Contents |

Under the Temperature layer click on Change Style.

Change the attribute to Temperature.

Under 2 Select a drawing style, Select Counts and Amounts (Color).

Click Done

Answer the analysis questions in your Laboratory Notebook.

Click on the shapes of the map. What do the shapes and colors on the map represent?

Click on Legend. How is the visual appearance of each dataset different?

Click on the check marks beside the layers in the Contents to turn them on and off on the map. How does this change the appearance of the map?

Click on and move the layers in the Contents to change the order. How does the layer order change the appearance of the map?

Based upon your map, which quadrats are the most biodiverse? Why do you believe so?

Conclusion

- How can GIS be used to display your data?

- What types of qualitative data did you add to your map?

- How is a GIS map different from a data table or graph?

- How can you use colors and shapes to determine the biodiversity of the area?