Name: _______________________________________________________________

Activity 3.1.2 The Sun is Shining

Purpose

Interest in sustainable energy in the U.S. has significantly increased over the past few decades. One of the pieces of this sustainability puzzle is the use of home rooftop photovoltaic (PV) systems, which create far less pollution than the conventional fuel mixes of coal, oil, nuclear, and gas. Solar arrays are becoming more common on homes, farms, and businesses across the country.

Solar arrays consist of solar panels containing photovoltaic cells. The cells convert sunlight into electrical power. Solar arrays can be cost effective because solar energy is free and renewable; however, they only operate when the sun is shining. Therefore, there must be enough sunshine at the location each year to produce electricity. Although solar arrays are rated to produce a certain amount of power, many variables, such as cloudiness and tilt, affect efficiency.

A residential 5kW solar array typically costs around $30,000. If you paid the $30,000 in monthly installments over a 15-year period, the monthly payment would be about $167 per month.

$30,000180 months=$166.666667month

Since a solar array does not operate during dark hours, electricity used during those hours would have to be purchased from the electric company or produced through other means.

| 1 watt (W) | = | unit of power | Many home solar array systems are rated at 5kW; enough to significantly reduce the amount of electrical energy that a home would otherwise draw from the traditional electric grid. A home 5kW solar array operating at maximum capacity could illuminate 83 60W light bulbs. (5,000W ÷ 60W = 83). |
| --- | --- | --- | --- |
| 1,000W | = | 1 kilowatt (kW) |  |
| 1,000kW | = | 1 megawatt (MW) |  |
| Figure 1. Energy Labels |  |  |  |

A pyranometer is used to measure solar radiation and can be used to determine suitability for solar array installation and operation. The data collected using a pyranometer includes irradiance in watts per square meter.

Does your location produce enough solar energy for a solar system to be viable? How does a solar array change household energy costs?

Materials

| Per group of four students: LabQuest2 LabQuest2 power supply Vernier pyranometer sensor Electrical tape Computer with internet access | Per student: Pen Calculator Laboratory Notebook ESI Notebook |
| --- | --- |

Procedure

In a group of four, calculate solar array specifications, then set up the pyranometer and solar panel apparatus to collect data over a full day. Analyze the data and determine if the goal was met for the array location. Record all research and observations in your Laboratory Notebook.

Part One – Current Electricity Use Estimation

Visit http://insideenergy.org/2014/05/22/using-energy-how-much-electricity-do-you-use-each-month/ and research the average electric use and cost per month for your state.

- How many kWh, on average, are used by an average household each month in your state?

- How much does the average household pay for electricity each month, in your state?

- Given the average electricity use and the average electrical bill, what is the average cost, per kilowatt hour, of electricity in your state? Use the formula below.

average monthly electricity bill$average monthly electricity use (kWh) = $/kWh

- Do you think a solar array would change your household energy cost? Why or why not?

Your teacher will direct you to the data collection site. Observe the site. Include the following in your observations and predictions.

Will there be shade at any time during the day?

What time of day do you expect the solar radiation to be highest? Lowest? Why?

What season of the year are you collecting data during?

What season will your region have the greatest number of sunlight hours?

Part Two – Data Collection

You will collect data over a six-hour period of daylight. Your teacher will provide a location for data collection. Prepare the apparatus on the first day, set up and collect data over the next entire school day, and analyze the data on a third day of class. NOTE: Part Two will be completed outside of class time in order to collect six full hours of solar energy data.

- In the morning, retrieve the pyranometer and LabQuest2 apparatus and transport to the outdoor location specified by your teacher.

Place the apparatus on a level surface in the area specified by your teacher and secure with electrical tape.

Turn the LabQuest2 on. Connect the LabQuest2 to a power source if directed to do so by your teacher.

Connect the pyranometer sensor to the LabQuest2. Set up data collection parameters.

Open the Sensors menu and select Data collection.

Change the Duration to hours, and enter 6 hours for the Duration.

Set the rate to 1 sample/h.

Click OK.

Tap Start to begin data collection.

Record observations about cloud cover and shade, then leave the apparatus until the end of the six-hour data collection period.

At the end of the data collection period, return to the site and save the collected data on the LabQuest2. Do not leave the LabQuest2 outside overnight.

Record additional observations about cloud cover and shade.

Collect the apparatus and transport it back to your classroom.

Part Three – Analyze Data

- In your group of four, access the stored data from your LabQuest2.

- Draw a table similar to Table 1 with rows for 6 hours in your Laboratory Notebook.

| Table 1. Investigation Results |  |  |  |
| --- | --- | --- | --- |
| Time of day (hour) |  | Average Wh/m2 | Average kWh/m2 |
|  |  | (a) | (b) |
|  | - am/pm |  |  |
| Total Six Hour Solar Energy Incident: |  |  |  |

Calculate energy production for your location using the steps below.

Record the six hours for which data was collected in the Time column in Table 1.

Record the integrals from the Irradiance vs. Time chart on the LabQuest as Wh/m2 in column (a).

Convert watt hours to kilowatt hours and record in column (b). Use the formula below.

Whm2 x1kW1,000W=kWhm2

Add the values in column b to calculate the total solar energy incident on the panels for six hours.

Complete the following calculations.

| Multiply the total single day solar energy incident (TSDSEI) by 45m2 which is about the size of a 5kWh solar array. | total six hour solar energy incident x 46.5m2 = kWh daily |
| --- | --- |
| Multiply the amount of energy produced by a 5kWh solar array in one day by 30 to estimate total production for the month. | daily x 30 days=kWh monthly |
| Calculate the cost value of monthly production, if the monthly cost is $167. | $167kWh monthly = $/kWh |
| How does the above figure compare to your state’s average electricity cost determined in Part One? | production cost - avg. electricity cost = $/kWh |
| Estimate how much additional electricity would you have to purchase, assuming your household electricity use is the same as the average for your state (data from Part One) if you installed a 5kW solar array. | avg. monthly use - monthly production = kWh |
| Compare the average monthly electricity bill for your state (data form Part One) to the monthly payment for a 5kWh solar array ($167/mo.). | avg. monthly electricity bill - $167 = $ |

Answer the following analysis questions.

What factors would increase electrical production?

Will monthly production be the same during all twelve months of the year? Why or why not?

Based on your calculations, does it make sense to install a solar array in your region? Why or why not?

Would you be willing to pay more for a renewable energy source, such as a solar array? Why or why not?

Conclusion

- How does the consumer cost of a technology influence consumer adoption and use of photovoltaic systems?

- Based on this experience, would you install a solar array on your home? Why or why not?