Name: ___________________________________________________________

Activity 3.1.3 A Lifetime of Emissions

Purpose

In 2015 the earth experienced its highest average temperature. Earth’s temperature has been increasing due to greenhouse gases (GHG) released during human activities. GHGs trap heat near the earth’s surface, and as the concentration of these gasses increase, so does the earth’s average temperate. A major source of GHGs is electricity generation. GHG pollutants emitted during electricity generation are difficult to collect and process as they escape into the atmosphere.

Utilities use many types of energy to generate electricity. Every phase of electricity generation can release pollutants into the environment. Utilities burn nonrenewable fuels, such as coal or oil, to convert water to steam. Steam pressure spins a turbine generating electricity. Nuclear and geothermal generators function in a similar fashion, using nuclear energy or earth’s energy as a heat source. Wind and hydroelectricity use the sun’s energy indirectly. Energy from the sun causes wind and transports water in the water cycle. Solar energy uses energy from the sun directly.

When you calculated your carbon footprint in Natural Resources and Ecology, you observed how some activities produce more GHGs than others. This is true for different sources of electricity generation. To compare the GHGs released during electricity generation, each generation phase is evaluated. The phases vary, but include construction of the facility, mining or extracting the fuel, transporting the fuel, and combusting the fuel.

Greenhouse gases released during each phase are estimated. The total GHGs emitted during electricity generation are called life cycle emissions (LCE). The unit for LCEs is grams of carbon dioxide released per kilowatt-hour of energy produced (gCO2/kWh).

What are the sources of GHGs during electricity generation? How do the LCE of different sources of electricity generation compare?

Materials

| Per student: Computer with internet Access to printer Calculator | Laboratory Notebook Pen ESI Notebook |
| --- | --- |

Procedure

Work in a group of four to plot, analyze, and compare LCE from different electricity generation sources.

Part One – Individual Analysis

Each member of the group should choose one of the following energy source pairs to analyze.

Oil and Wind

Coal and Solar

Liquefied Natural Gas (LNG) and Hydropower

Nuclear and Geothermal

Research each source and classify it as a renewable or a nonrenewable energy source. Record your findings in your Laboratory Notebook. Provide reasoning for each classification.

Using the data in Table 1, make two pie charts, one for each of your energy sources. Use the website www.piecolor.com to make pie charts following the instructions below. The pie charts should be neat, easily read, and visually attractive.

Determine how many slices the pie chart will have. Include each process contributing to the LCE. Enter that value and click on the button labeled Add this number.

Convert the LCE values for each process related to your source to percentages and enter the values in the column labeled Values on the website. Total percentages need to equal 100.0%.

Select contrasting colors for pie slices from the color wheel menu for each value to make the data visible.

Enter the processes in the column labeled Names.

Enter 300 px for the pie chart size.

Enter the chart title. Use the source and total LCE as the title for the pie chart. Ex: Coal LCE, 922.4 gCO2/kWh.

Select a coordinating font color for the title.

Leave the title font size at 16.

Select white for the pie chart background from the menu.

Select an appropriate, easily readable font style from the menu.

Select either 2D or 3D, centered, for the pie chart style from the menu.

Click on the button labeled Generate Pie Chart. Download the chart and save for use.

Repeat for the second source.

| Table 1. LCE for Electricity Generation Sources (gCO2/kWh) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source | Construction | Extraction | Transport | Combustion | Other Operations | Total |
| Coal | 3.6 | 9.7 | 15.6 | 886.8 | 6.7 | 922.4 |
| Geothermal | 5.3 | 0.0 | 0.0 | 0.0 | 9.7 | 15.0 |
| Hydropower | 9.3 | 0.0 | 0.0 | 0.0 | 1.9 | 11.2 |
| Natural Gas | 2.9 | 93.8 | 19.4 | 477.9 | 4.5 | 598.5 |
| Nuclear | 2.8 | 20.9 | 0.0 | 0.0 | 3.9 | 28.3 |
| Oil | 2.3 | 11.1 | 6.7 | 704.3 | 17.4 | 741.8 |
| Solar | 33.7 | 0.0 | 0.0 | 0.0 | 10.1 | 43.8 |
| Wind | 21.2 | 0.0 | 0.0 | 0.0 | 8.3 | 29.4 |

Hondo, H. (2004). Life cycle GHG emission analysis of power generation systems: Japanese case. Retrieved from http://www.univie.ac.at/photovoltaik/umwelt/LCA_japanstudy.pdf.

Download and print the pie charts.

Answer the following analysis questions in your Laboratory Notebook. Refer to your two pie charts as applicable.

What differences do you observe between the data in the two pie charts?

How do the total LCE compare for the two energy sources? Include data.

Which process contributes the greatest share of GHGs to LCE? Include data.

How do visual tools, such as pie charts, help organize and analyze data?

Part Two – Group Analysis

- Share your pie charts with your group. Discuss the similarities and differences among the energy sources.

Individually, refer to the pie charts as you answer the following analysis questions in your Laboratory Notebook.

How do all energy sources rank in terms of total LCE, from greatest to least?

How do the nonrenewable energy sources rank for each process (Construction, Extraction, Transport, Combustion, and Other Operations)? Rank the energy sources based on contribution to total LCE from largest to smallest.

Which two energy sources are you most in favor of developing or using? Provide reason(s) for each source chosen.

Conclusion

- Compare the processes that produce GHGs for renewable and nonrenewable energy sources.

- Why do renewable energy resources used for electricity generation have smaller LCE values than nonrenewable sources?

- Why would replacing nonrenewable energy sources with renewable energy sources slow the increase in GHGs in the atmosphere?