Activity 3.2.1 Watts Up?

Purpose

Choosing the source of electrical energy is not always an option for consumers. However, as a consumer you can choose how much energy you use.

A hardware store has many choices for lighting in your home. LED, fluorescent, and incandescent light bulbs are commonly found in homes today. The most common type of light bulb is an incandescent bulb. Incandescent bulbs use electricity to heat a filament emitting light. As technologies improved, fluorescent light bulbs were developed. Fluorescent bulbs produce light by sending electricity through ionized gas. Fluorescent bulbs last longer, but cost more to produce and contain toxic metals, such as mercury. Light emitting diode (LED) bulbs contain semiconductors that conduct electricity and produce light. Each light bulb consumes a different amount of energy.

When you turn on an appliance, such as a refrigerator or light, you are consuming electrical energy measured in kilowatt-hours. Energy companies calculate kilowatt-hours by multiplying the number of hours an appliance uses electricity by the kilowatts, which is the work done by the appliance. The work done by the light bulb is the emitted light. Because light bulbs are not 100% efficient, energy is lost in the form of heat while being used.

What types of light bulbs are more efficient? How do the costs of light bulbs differ? How can your personal consumer choices affect energy usage?

Materials

| Per pair of students: LabQuest2 IR thermometer Kill A Watt® Fluorescent light bulb Incandescent light bulb LED light bulb Lamp base Ring stand Utility clamp Hot hand protector | Per student: Pencil ESI Notebook Laboratory Notebook |
| --- | --- |

Procedure

Work with your partner to find the energy costs of three types of light bulbs.

Part One – Light Bulb

Copy Table 1 into your Laboratory Notebook.

| Table 1. Light Bulb |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Light Bulb | Initial Temp. | Ending Temp. | Average Watts | Average Kilowatts | Time (hours) | Kilowatt-hours | Cost/kilowatt hour | Total Cost |
| Incandescent |  |  |  |  | 0.05 |  | $ 0.12 |  |
| Fluorescent |  |  |  |  | 0.05 |  | $ 0.12 |  |
| LED |  |  |  |  | 0.05 |  | $ 0.12 |  |

| Set up the LabQuest2. Turn on the LabQuest2. Plug the infrared (IR) temperature sensor into Channel 1. Tap on Rate and change the Duration to 0.05 hours. Change Rate to 3600 samples/h. Tap on the Channel 1 on the meter screen and Change the Units to °C. Tap OK. Review Figure 1 for setting up light and IR temperature sensor. Attach the clamp to the bottom of the ring stand. Attach the IR temperature sensor to the clamp with the sensor pointing upward. Fasten the incandescent light bulb to the lamp. Clamp the lamp on the ring stand with the light facing the temperature sensor as seen in Figure 1. |  |
| --- | --- |
|  | Figure 1. Lamp and Temperature Sensor |

Press the infrared button on the sensor. CAUTION: The laser could cause serious injury to your eyes. DO NOT point the laser at anyone or look directly into the laser. Adjust the distance of the sensor so a circle approximately 1″ in diameter projects on the center of the light bulb in the lamp.

Shut off the laser on the infrared temperature sensor.

Plug the Kill A Watt® into an electrical outlet.

Plug the lamp into the Kill A Watt®.

Press the watt button on the Kill A Watt®.

Turn on the lamp.

Press the Start button on the LabQuest2 to begin recording data.

Once data collection stops, shut off the lamp.

On the LabQuest2, tap Analyze and choose Statistics, then select Temperature.

Record the low and high temperature of the light bulb in Table 1.

Record the watts used by the lamp in Table 1.

Unplug the Kill A Watt® and use a hand protector to remove the light from the lamp base

Repeat Steps 6 – 15 with a fluorescent lightbulb and LED lightbulb.

Convert watts to kilowatts by dividing watts by 1000. Record the kilowatts of energy used by each light bulb in Table 1.

Find the kilowatt-hours used by each light by multiplying the Average Kilowatts by Time. Record the Kilowatt-hours in Table 1.

Find the cost of using each light by multiplying Kilowatt-hours by the Cost/Kilowatt-hour ($ 0.12). Record the Total Cost in Table 1. Round to nearest thousandth. ($0.000)

Answer the following analysis questions.

How does energy used by each light differ?

What forms of energy were emitted from each light?

How was energy lost?

Part Two – Cost Calculations

Use the information in Table 2 and the kilowatts used by each bulb found in Part One to compare the total cost of using an incandescent, fluorescent, and LED light bulb for an entire year if a bulb is used for 2400 hours. Calculate the total cost by adding the cost of light bulbs used in a year to the total energy costs. Record calculations in your Laboratory Notebook and answer the analysis questions.

| Table 2. Light Bulb Data |  |  |  |
| --- | --- | --- | --- |
|  | Fluorescent | Incandescent | LED |
| Cost per bulb | $2.00 | $1.00 | $4.00 |
| Average Life Span | 8000 hours | 1200 hours | 50,000 hours |

Analysis Questions

What is the difference in total cost between the three bulbs?

What other factors do you think consumers consider before choosing a bulb?

Part Three – Environmental Cost

Use the websites listed below to compare advantages and disadvantages of each light bulb. Record the advantages and disadvantages in your Laboratory Notebook and answer final analysis questions.

Internet sources

Diffen: Fluorescent vs Incandescent Bulbs: http://www.diffen.com/difference/Fluorescent_Bulbs_vs_Incandescent_Bulbs

Consumer Energy Center: http://www.consumerenergycenter.org/lighting/bulbs.html

Analysis Questions

What are the environmental tradeoffs of fluorescent, incandescent, and LED bulbs?

What other costs could a consumer have when choosing fluorescent, incandescent, and LED bulbs?

Conclusion

- What should consumers research before purchasing a product that uses energy?

- Why is cost not the only factor to consider when choosing a product?

- How can consumers conserve energy?