Name: _________________________________________________________________

Activity 5.1.2 The Source of Pollution

Purpose

Many human and natural activities result in pollution. Whether it is the fast food cup discarded in a park, an oil spill in the ocean, or nutrients running into a stream, sources of pollution can be found everywhere.

Pollution sources are categorized into two broad groups: point source and nonpoint source. Point source pollutants include industrial sources emitting pollution through stacks, chimney, drains, and pipes. Nonpoint source pollution is from large diffuse areas, such as runoff from urban areas and agricultural fields. As human populations grow, pollution from nonpoint sources increases. During the late twentieth century, regulation forced reduction of point source pollution in the United States, improving air and water quality.

There are a variety of pollutants released into the environment. Organic wastes include sewage and manure, which carry diseases and reduce oxygen levels in water. Fertilizers, such as nitrogen and phosphorus compounds, can cause excessive algae growth. Toxic chemicals, including gasoline, oil, and acids, harm organisms and disrupt food chains. Sediments in runoff interrupt photosynthesis in aquatic systems. Mercury, arsenic, and other heavy metals can cause cancer, birth defects, and nerve disorders.

How can you identify the source(s) of pollutants in a watershed? How do pollutants affect ecosystems?

Materials

| Per student: Computer with internet access ESI Notebook | Laboratory Notebook Pencil |
| --- | --- |

Procedure

Identify sources of pollution in a model watershed. Then use the Riverweb simulator to determine how land usage affects pollutants found in runoff.

Part One – River Pollution

Access the following site: http://mvhs.shodor.org/ncsa/riverweb/version5/riverweb.php.

Enter the user name ‘generic’ and the password ‘generic’ as instructed. Click Submit.

Identify the following on the map.

Seven regions within a watershed with different land uses.

Water sampling stations numbered from 0 to 7. Water sampling station 7 is at the mouth of the river.

Click on the Station 1 button, which is the logging area. You will be taken to a page displaying characteristics of a logging area and measurements from that station.

Directly under the Station 1: Logging Forest title you will see Nitrogen Concentration (Mg/L). Click on the link. The pop-up box has information about the five water pollutants (nitrogen, phosphorous, sediments, heavy metals, and toxins) and water quality indicators (water temperature, dissolved oxygen, pH, and runoff).

In your Laboratory Notebook, make a table similar to Table 1. Include all five pollutants.

| Table 1. Water Pollutant Background |  |  |
| --- | --- | --- |
| Pollutant | Source(s) | Effect(s) on a River Ecosystem |
| Nitrogen |  |  |

Complete the table for each pollutant using the information provided in the pop up box.

In your Laboratory Notebook, list the following water quality indicators.

Water temperature

Dissolved oxygen

pH

Runoff

Use the information in the pop-up box to define each indicator in your Laboratory Notebook.

Close the pop-up box when you are finished.

Part Two – Sources of River Pollution

- In your Laboratory Notebook, list and identify each area of the watershed as a point or nonpoint source of pollution. Explain why pollutants from each area would be point source, nonpoint source, or both.

Logging forest

Agricultural area

Residential area

Commercial industry

Wetland area

Urban area

In your Laboratory Notebook, draw a table similar to Table 2. Include all six areas of the watershed.

| Table 2. Sources of River Pollution |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Watershed Area | Water Pollutant |  |  |  |  |
|  | Nitrogen | Phosphorus | Sediments | Heavy Metals | Toxins |
| Logging Forest |  |  |  |  |  |

Notice the page displays Station 1 twice. At this point, you will only be using the upper display.

In the top left hand margin, find the Top Display: Select a Station buttons. Select 1. Click on Change Display. You will see Station 1: Logging Forest in the information area.

In the simulator, on the right side of the data plot, find the average runoff nitrogen concentration for a logging forest and record in Table 2.

Change the Indicator on the left side of the screen to Phosphorus. Click on Change Display.

Read the average phosphorus concentration and record in Table 2.

Complete Step 5 – 7 for Sediments, Heavy Metals, and Toxins.

Select Station 2 for the Agricultural Area and change the indicator to nitrogen concentration.

Click on Change Display.

Repeat Steps 5 – 8 for the Agricultural Area.

Repeat Steps 4 – 8 for the residential, commercial, wetland, and urban area.

For each pollutant in the table, circle the highest value and place an X over the lowest value.

Record answers to the analysis questions in your Laboratory Notebook.

In this river system, which area contributes most of the pollution?

How is agricultural pollution different from industrial pollution?

In which area will pollutants will be easier to monitor and control? Why?

Why does a watershed consist of multiple land uses?

Conclusion

- How does land use affect the types of pollutants in a watershed?

- What types of pollutants are associated with both agricultural operations and residential areas?

- Do point source pollutants affect ecosystems differently than nonpoint source pollutants? Share your reasoning.