Project 5.2.3 Local Investigation

Purpose

People around the country consume resources from various types of industrialization. Because industrialization varies by location, environmental issues are specific to local communities. Local environmental issues vary based upon industry, energy sources, and consumption preferences. Where will you find evidence of pollution in your local area?

As areas become more populated, the number of pollutants increases. All local communities are suspect to pollution from many sources. Once a researcher identifies a potentially polluted location, he or she can sample and test natural resources for evidence of pollution. After the types of pollution are known, solutions can be developed.

Researchers must collect the correct type of data to determine if pollution is an issue. You learned the process for testing air and water for pollutants during Natural Resources and Ecology. Carbon dioxide emissions and particulate pollution levels are examples of environmental data you collected from polluted air. Both are byproducts of industrialization and cause environmental problems. Polluted water can be detected by measuring pH, nitrate, phosphate, coliform, turbidity, total solids, and dissolved oxygen.

Where is pollution found in your community? How can you identify the causes of those problems and recommend solutions?

Materials

| Per pair of students: Vernier sensors Lab supplies for water and air collection Glassware for laboratory testing Sampling and testing procedures | Per student: ESI Notebook Laboratory Notebook |
| --- | --- |

Procedure

Work with your partner to investigate a local area for pollutants in air or water. Record all notes, research, materials, procedures, and data tables in your Laboratory Notebook.

Choose a location in your local area (indoor or outdoor) you believe is at risk to pollution. Suggested locations are listed below.

| Gas station Major intersection Holding pond Residential drain | Factory Local water supply Lake shore Landfill |
| --- | --- |

Research the types of pollution you may find in the selected area. Record all research. Your research should include the following information.

Potentially polluted resources

Causes of the pollution

Types of pollutants

Methods for testing air or water for pollutants

Environmental effects of the pollutants

Narrow your investigation by writing a question you will investigate. For example, “Is the local holding pond near the gas station contaminated with fuels?”

Hypothesize your answer and record your hypothesis.

Determine the pollution tests you will complete. Your teacher has the following procedural methods available upon request.

| Carbon Dioxide Particulates BOD Dissolved Oxygen Fecal Coliform Nitrates | pH Phosphates Temperature Total solids Turbidity |
| --- | --- |

Select materials and write a procedure for testing your hypothesis. The procedure should include the following information.

Sample collection method

Testing method

Collection and testing materials

Variables and controls

Design and record data tables in your Laboratory Notebook.

Go to select location(s) and collect samples and field data.

Complete your sample testing and data collection in the lab.

Work with your partner to analyze the data and write a conclusion comparing results to your hypothesis.

Individually work to formalize your research by completing a lab report using the Lab Report Template.

Your teacher will assess your report using the Lab Report Evaluation Rubric.

Conclusion

- What factors determine if the environment is at risk to pollution?

- How do people pollute our natural resources?

- How can you determine if a pollutant is a threat to a population?