Name: _________________________________________________________________

Activity 5.3.2 Murky Water

Purpose

Water is one of most precious resources needed for all living things. The health of people and ecosystems is dependent upon a quality water supply. Many pollutants have contaminated water supplies. How can you clean polluted water?

Scientists have developed many processes for purifying and treating contaminated water supplies. Treatment plants remove chemicals, microorganisms, and solids found in contaminated water. The first step in cleaning water is to remove the solids. Solid materials in water reflect, absorb, or transmit light, reducing the clarity. Turbidity is a measurement of the clarity of water. Water with high light transmission has a low turbidity, or low content of solid materials. When light is not able to transmit through water, the turbidity is high with a high solid material content.

Non-settling solids that cause a high turbidity in water are colloids. Colloids can cause filtration systems to clog quickly, costing more money for filters and increased labor costs. Water treatment plants remove colloids from water with flocculants. Flocculants are chemicals that cause colloids to bind together and settle out of water. Once the colloids settle to the bottom, they can be separated from the water. Aluminum sulfate and polymers are two types of flocculants used to treat water. Colloids in water will bind with aluminum sulfate and settle. Polymers are long molecules with many curves in them. Colloids get trapped in a polymers’ curve, causing the polymer and colloid combination to become heavy and settle out of contaminated water.

How should you use a flocculant to clean water? What factors determine the type of flocculant to use?

Materials

| Per student: PPE Pen ESI Notebook Laboratory Notebook | Per group of four students: Electronic balance |
| --- | --- |
| Per pair of students: Part One Permanent marker, fine tip Laboratory tape 2 50ml beakers Weigh dish 100ml graduated cylinder Plastic spoon Ammonium aluminum sulfate (alum) Glass stirring rod 1ml disposable graduated pipet Pool cleaner (polymer) 7 12 oz. clear plastic cups | Part Two 1 quart of unpurified water 100ml graduated cylinder Aluminum solution Polymer solution 2 glass stirring rods 2 1ml graduated pipets |

| Part Three 1 quart of unpurified water 100ml graduated cylinder Aluminum sulfate solution Polymer solution Lemon juice 2 glass stirring rods 3 1ml graduated pipets | Part Four LabQuest2 pH sensor Distilled water 250ml beaker 12 oz. plastic waste cup Rinse bottle 1ml graduated pipet Turbidity sensor Turbidity cuvette Lens paper |
| --- | --- |

Procedure

Work with a partner to purify water using different methods and then test water for clarity.

Part One – Material Preparation

Use a marker and laboratory tape to label one 50ml beaker “aluminum sulfate” and the other “polymer flocculant.”

Measure 10ml of water in a graduated cylinder and pour into a 50ml beaker. Repeat for the second beaker.

Use a plastic spoon, weigh dish, and electronic balance to measure 1g of alum.

Pour the alum into the 50ml beaker labeled aluminum sulfate and stir with a glass stirring rod.

Use a 1ml graduated pipet to transfer 0.5ml of pool cleaner into the 50ml beaker labeled polymer flocculant.

Slowly stir the polymer flocculant with the 1ml pipet.

Use a permanent marker and laboratory tape to label seven clear plastic cups with the following labels.

Cup 1: Control

Cup 2: Aluminum sulfate – not stirred

Cup 3: Aluminum sulfate – stirred

Cup 4: Polymer – not stirred

Cup 5: Polymer – stirred

Cup 6: Aluminum sulfate – acidic water

Cup 7: Polymer – acidic water

Make a table similar to Table 1 in your Laboratory Notebook. Be sure there is enough space in the observation column to record two to three sentences.

| Table 1. Water Treatments |  |  |  |
| --- | --- | --- | --- |
| Water Treatment | Observations | pH | Turbidity |
| Cup 1: Control |  |  |  |
| Cup 2: Aluminum sulfate – not stirred |  |  |  |
| Cup 3: Aluminum sulfate – stirred |  |  |  |
| Cup 4: Polymer – not stirred |  |  |  |
| Cup 5: Polymer – stirred |  |  |  |
| Cup 6: Aluminum sulfate – acidic water |  |  |  |
| Cup 7: Polymer – acidic water |  |  |  |

Part Two – Flocculant Test

- Shake one quart of unpurified water to make a homogeneous mixture.

Use a graduated cylinder to pour 100ml of unpurified water into the cups numbered 1 – 5.

Add 1ml of aluminum sulfate solution to Cup 2 and Cup 3 using a clean pipet.

Add 1ml of polymer solution to Cup 4 and Cup 5 using a clean pipet.

Use a clean glass stirring rod to mix Cup 3 for 3 minutes as your partner uses a clean glass stirring rod to mix Cup 5 for 3 minutes.

Wait 3 minutes for the colloids to settle.

Record your observations of each cup in your Laboratory Notebook.

Answer the following analysis questions in your Laboratory Notebook.

Compare treatments to the control.

How do aluminum sulfate and polymer affect the visible clarity of the water?

What affect does stirring have on the flocculants?

Part Three – Acidity

- Shake one quart of unpurified water to make a homogeneous mixture.

Use a graduated cylinder to pour 100ml of unpurified water into Cup 6 and Cup 7.

Use a clean pipet to add 1ml of lemon juice to Cup 6 and 7 to increase the acidity.

Add 1ml of aluminum sulfate solution to Cup 6 using a clean pipet.

Add 1ml of polymer solution to Cup 7 using a clean pipet.

Use a clean glass stirring rod to mix Cup 6 for 3 minutes as your partner uses a clean glass stirring rod to mix Cup 7 for 3 minutes.

Record your observations of each cup in your Laboratory Notebook.

Part Four – Quantitative Data

Test the pH and turbidity of the solutions to determine impacts on water quality.

pH Test

- Connect the pH sensor to Channel 1 of the LabQuest2.

- Place 50ml of distilled water in a 250ml beaker.

Remove the pH sensor from the solution bottle by twisting the cap loose and sliding the sensor out.

Before each use of the pH sensor, rinse the tip of the sensor thoroughly with distilled water. Hold the pH sensor above the waste cup and use the rinse bottle to rinse the sensor tip thoroughly.

Place the tip of the sensor in the control. When the reading is stable, record the pH in Table 1 of your Laboratory Notebook. It may take 30 seconds for the sensor to stabilize.

Rinse the tip of the sensor with distilled water over the waste cup.

Repeat Steps 5 – 6 for each cup.

Turbidity

- Disconnect the pH sensor and connect the turbidity sensor to Channel 1 of the LabQuest2.

On the Meter screen, tap Mode. Change the data-collection mode to Events with Entry.

When prompted, type Cup for Event Name.

Select Average over 10 seconds and select OK.

Follow the steps below to collect turbidity data.

Use the pipet to fill the cuvette with the water from the Cup 1 (control) so the bottom of the meniscus is even with the top of the white line. Take water from the surface of the cup to avoid collecting any flocculant.

Place the lid on the cuvette. Gently wipe the outside with lens paper.

Check the cuvette for air bubbles. If air bubbles are present, gently tap the bottom of the cuvette on a hard surface to dislodge them.

Holding the cuvette by the lid, place it into the turbidity sensor. Make sure that the mark on the cuvette is aligned with the mark on the turbidity sensor. Close the lid.

Start data collection.

Tap Keep. Note: Do not disturb the sensor during the 10-second sampling period.

When prompted enter the cup number.

Dispose of the water from the cuvette in a waste cup.

Rinse the pipet and cuvette with distilled water from rinse bottle.

Repeat Steps 2 – 4 for each sample treatment.

Stop data collection and tap the Table icon to view the data. Record the values in Table 1 of your Laboratory Notebook.

Answer the following analysis questions in your Laboratory Notebook.

Compare your treatments of Cup 6 and 7 to the control.

How does pH affect the efficiency of the flocculants?

Why would a treatment plant want to know the pH of a water supply before deciding on a treatment method?

Conclusion

- How can you use scientific processes to solve environmental issues such as pollution?

- How do the pollutants in a resource interact and affect the solution for cleaning the resource?

- If your local water supply is polluted and needed to be cleaned, what would you want to know about the water before you attempt to clean it?