Name: _________________________________________________________________

Project 5.3.3 Pure Water

Purpose

You have conducted multiple experiments testing water quality. Why is water quality so important? How can unclean water cause social, economic, and scientific issues?

Millions of people around the world do not have access to safe water. Unsafe water causes disease and health problems. Sewage, animal wastes, chemicals, and fertilizers are all potential contaminants of drinking water. You have learned about phosphate and nitrate contamination of water. Another common pollutant is ammonia. Ammonia is found in water containing human and animal waste and near coal extraction sites. Ammonia can cause health problems for people and is toxic to fish.

Polluted water must be tested and cleaned to be considered safe. Water purification involves multiple processes including separation, filtration, oxidation, and chemical application. Contaminants can be separated from water using flocculants and boiling. Filtration devices contain sand and charcoal to remove chemicals and bacteria. Oxidation of water helps remove odors in the water. Chemical applications, such as chlorine, can kill bacteria found in the water.

The issues surrounding water quality vary around the world. The location, contamination source, and use of the water are all factors affecting the solution to a water quality problem. People use the engineering design process to research and solve environmental problems. The first step in the process is identifying the problem. Then engineers set requirements for solving the problem. Next, they build a prototype to test. If the prototype meets the requirements, they have solved the problem.

Scientists and engineers can solve problems such as water quality, using the engineering design process. How will you use the engineering design process to solve a water quality issue?

Materials

| Per group of four students: Contaminated water sample Field report Profile analysis Various materials for construction LabQuest2 Vernier turbidity sensor Cuvette Lens paper Vernier ammonia sensor Ammonia high standard solution Ammonia low standard solution | Pipet Distilled water rinse bottle 250ml beaker Per student: PPE Pen Project 5.3.3 Evaluation Rubric ESI Notebook Laboratory Notebook Engineering Process Template |
| --- | --- |

Procedure

Your group of will receive a water sample, a field report, and an area profile. Design a water purification system that meets all scientific, economic, and social needs of the area profile. Record all research and data in your Laboratory Notebook.

Part One – Turbidity Test

Obtain your assigned contaminated water, LabQuest2, and turbidity sensor

Set up the LabQuest2 and turbidity sensor.

Use the pipet to fill the cuvette with contaminated water.

Place the lid on the cuvette. Gently wipe the outside with lens paper.

Check the cuvette for air bubbles. If air bubbles are present, gently tap the bottom of the cuvette on a hard surface to dislodge them.

Holding the cuvette by the lid, place it into the turbidity sensor. Make sure the mark on the cuvette is aligned with the mark on the turbidity sensor. Close the lid.

Wait 10 seconds for the turbidity reading to stabilize.

Record water turbidity.

Pour water back into contaminated water jar.

Rinse the pipet and cuvette with distilled water.

Disconnect the turbidity sensor.

Part Two – Ammonia Test

Ammonium Sensor Calibration

Your teacher will have the Ammonium Ion-Selective Electrode (ISE) in the ammonia high standard solution for at least 30 minutes before class. Note: Make sure the ISE is not resting on the bottom of the container, and that the small white reference contacts are immersed. Make sure no air bubbles are trapped below the ISE.

First Point Calibration

- Ensure the ammonium ISE is in the ammonia high standard solution and connect the ammonia ISE to Channel 1 of the LabQuest2.

- Choose Calibrate from the Sensors menu and select Calibrate Now.

- Enter 100 as the concentration of the standard in mg/L for Reading 1.

- When the voltage reading stabilizes, tap Keep.

Second Calibration Point

- Remove the ammonium ISE from the ammonia high standard solution.

- Rinse the ammonium ISE thoroughly with distilled water over a waste cup and gently blot it dry with a tissue or paper towel. Note: Failure to carefully rinse and dry the ISE will contaminate the standard.

| Place the tip of the ammonium ISE into the low standard solution (1 mg/L). Note: Be sure the ammonium ISE is not resting on the bottom of the bottle and the small white reference contacts are immersed as seen in Figure 1. Make sure no air bubbles are trapped below the ISE. Enter 1 as the concentration of the standard in mg/L for Reading 2. After briefly swirling the solution with ammonium ISE sensor, hold the ISE still and wait approximately 60 seconds for the voltage reading to stabilize. When the voltage reading stabilizes, tap Keep. |  |
| --- | --- |
|  | Figure 1. ISE in Bottle |

Save Sensor Calibration

- Select the Storage tab.

- Tap Save Calibration to Sensor and follow the onscreen instructions to save the calibration.

- Tap OK.

- Rinse the ammonium ISE with distilled water into a waste cup and gently dry off the sensor with paper towels.

Ammonia Test

- Place the ammonium ISE in the water sample. Be sure the white reference contacts are immersed and the sensor is not resting on bottom of container. (Figure 1.)

Wait 60 seconds for a reading to stabilize.

Record the ammonia content of water.

Use a 250ml beaker and distilled water rinse bottle to rinse the sensor.

Place the long-term storage bottle on the sensor. Be sure the tip of the sensor is not touching the sponge.

Part Three – Analyze the Issue

- Read the field report and area profile. Compare the field report and profile to your test results.

Research the issues of unclean water related to the field report and area profile.

Identify one issue related to water quality. State the issue as a non-biased question and record in your Laboratory Notebook.

Use the internet and textbook to define key terms used in the question.

Use a computer with internet access to research the history and origin of the issue.

Summarize the origin and history of the issue in a short paragraph.

Make a table similar to Table 1 in your Laboratory Notebook.

| Table 1. Issue Analysis |  |
| --- | --- |
| Risks | Benefits |

Record risks and benefits surrounding the issue in Table 1.

Record and define any additional terms people would need to understand to make a decision in regards to the issue.

Document all resources used in APA format.

Part Four – Background Research

- Research the potential means of cleaning water. Brainstorm solutions to the water quality issue that you researched in Part Three. Use the following resources to begin your research.

Learn about water: http://www.epa.gov/learn-issues/learn-about-water

Water solutions: www.water.org

Solar disinfection: http://www.cdc.gov/safewater/pdf/solar2011final.pdf

Biosand filter: http://www.sswm.info/content/biosand-filter

Ceramic filtration: http://www.cdc.gov/safewater/pdf/ceramic_2011-final.pdf

Solar distillation of water: http://www.fsec.ucf.edu/en/publications/pdf/FSEC-EN-3-80.pdf

Develop a solution to mitigate that issue. Enter your information on the Engineering Process Template. The solution should include the following items.

Problem: the issue addressed by the plan, risks, benefits, and key terms related to the issue.

Requirements: identify and describe the scientific, economic, and social requirements that must be satisfied by the solution.

Background: the social and economic importance of clean water, how water pollution occurs, and the negative effects of polluted water.

Solution: how you propose that you will clean the polluted water.

Document all resources used in APA format.

Part Five – Project Design and Construction

- Use the Engineering Process Template to organize your design.

Materials: list all materials necessary to construct a water purification system.

Sketch the design of the water purification system on a separate page.

Prototype Construction: Describe the steps for constructing the prototype.

Test and Evaluation: Determine what data needed to evaluate the prototype.

Use the field report and area profile to identify how you will measure the quality of your water after purification.

Record the equipment and procedure for testing the water.

Design a data collection table for the tests you will be conducting. Make the table in your Laboratory Notebook.

Have your teacher approve your design for construction and the water tests.

Once your teacher approves your design, begin construction of your purification system. Record the procedure for constructing your purification system prototype.

Part Six – Test and Data Collection

Use the prototype and the protocol for testing and evaluation to purify the sample water and test the sample after purification. Record and analyze your data in your Laboratory Notebook.

Part Seven – Engineering Report

Use your information and data recorded in your Laboratory Notebook to complete a formal report. Use the Engineering Process Template to type your report. Complete the Conclusion section of the Engineering Process Template by answering the following questions.

What criteria did your prototype meet?

What were possible sources of error?

What new design problems or solutions arise based on your results?

Based on the data, is your solution effective?

How does your system meet economic and social needs?

Conclusion

- What limits people’s access to clean water?

- How can you clean water with limited resources and money?

- How do social and economic issues affect solutions to environmental problems?