Activity 1.2.5 Moving Microliters

Purpose

| In biotechnology, you will find that you often have to work with very small specimens. Many times, the accuracy of the volume you transfer has direct impacts on the results of your experiment. When working with DNA and protein samples, the quantity is often so small that adding a milliliter of solution to analyze the sample dilutes it to the point of limited or no results. Thus, microliters are commonly used. A microliter is one thousandth of a milliliter and is expressed as µl. In laboratory research, measurements need to be exact and precise in order to be able to accurately replicate the procedure. Three commonly used pipets in this course are transfer, volumetric, and micropipets. How do the different pipets compare in terms of quantity and accuracy? |  |
| --- | --- |
|  | Figure 1. Volumetric pipet and pump assembly and micropipet |

Materials

| Per pair of students: Part One 5ml volumetric pipet Pipet pump Disposable transfer pipet 4 test tubes Test tube rack 400ml beaker 8 oz. paper cup 2 microcentrifuge tubes Laboratory tape Part Two 8 oz. paper cup 2-20µl micropipet Box of micropipet tips Microcentrifuge tube Sheet of wax paper Paper towel Waste container | Per class: 5 electronic balances Colored water |
| --- | --- |
|  | Per student: Sheet of wax paper Permanent marker PPE Pen Agriscience Notebook Laboratory Notebook |

Procedure

In this activity, you will share equipment with your partner, but complete each section individually. While one partner completes Part One, the other should complete Part Two. Upon completion, switch and complete the other part.

Part One – Serological and Transfer Pipets

Obtain the materials for Part One.

Fill your paper cup two-thirds full with the colored water stock.

In your Laboratory Notebook, prepare a table to record the following information.

| Samples | Data |
| --- | --- |
| 2ml | Expected mass of sample |
| 3ml | Initial mass of equipment |
| 4ml | Mass of equipment and sample |
| 5ml | Actual mass of sample |
| 0.5ml | Difference between expected and actual |
| 1ml | Percent error |

Determine the mass of an empty test tube resting inside the beaker.

Label the test tubes 2ml, 3ml, 4ml, and 5ml and place in the test tube rack.

Insert the end of the volumetric pipet into the pipet pump. When opening sterile objects, open the end you will be handling, not the end used to transfer.

Place the pipet tip into the cup of colored water and draw up 2ml of water. Stop as soon as the meniscus touches the 2ml mark.

Transfer the water to the appropriate test tube.

Repeat for the remaining samples.

Determine the mass of each sample you prepared and record your results.

Calculate the volume transferred into each test tube.

Dispose of the colored water in each test tube and rinse each with dH2O.

Determine the mass of an empty microcentrifuge tube and record.

Label the two microcentrifuge tubes 0.5ml and 1ml.

Using the disposable transfer pipet, depress the bulb at the top and insert the pipet into the colored water. Slowly release the bulb to draw water into the pipet. Allow the water to come past the 0.5ml mark, and then gently depress the bulb to push excess water from the pipet. Stop when the meniscus touches the 0.5ml mark on the pipet.

Transfer the water into the microcentrifuge tube.

Repeat for the 1ml sample.

Determine the mass of each sample you prepared and record your results in your Laboratory Notebook.

Calculate the volume transferred into each microcentrifuge tube.

Determine your accuracy for each sample prepared and discuss possible sources of error. Remember 1g of water is equal to 1ml.

Clean up, leaving equipment ready for your partner to use.

Part Two – Micropipets

- Obtain materials for Part Two.

Fill your paper cup two-thirds full with the colored water stock.

In your Laboratory Notebook, record your processes and observations.

Holding the micropipet as demonstrated by your teacher, set the display to each of the following settings to practice setting the volume.

18.5µl, 15µl, 9.5µl, 5µl, 2.5µl

Gently depress the plunger to the first stop and release. This motion is used to draw up liquid.

Depress the plunger to the second stop. This motion is used to expel the liquid.

Set the display to 20µl.

Holding the micropipet with the display facing you, insert the tapered end of the micropipet into a tip in the micropipet tip box. Tap twice to ensure the tip is securely on the micropipet.

Depress the plunger to the first stop.

Insert the tip of the micropipet into the colored water and release the plunger.

Remove the tip from the water and transfer the liquid to the wax paper. Remember to depress the plunger all the way to remove the liquid.

Observe your micropipet tip, can you see any liquid remaining? If so, you did not expel your entire sample. Try depressing the plunger over the wax paper again.

Repeat this process for each of the following quantities.

5µl, 10µl, 12.5µl, 15µl, 17.5µl

Observe the droplet size of each sample.

Transfer the following amounts of colored water into a microcentrifuge tube. Continue adding each sample to the same tube.

3µl, 2.5µl, 4.5µl, 7µl

Determine the total microliters transferred into the tube.

Set the micropipet to the total volume transferred and draw that volume from the tube. Did you remove all liquid from the tube?

Expel the sample onto the wax paper. Use a paper towel to dry the wax paper.

Set the micropipet to 2µl and draw up a sample of water.

Expel the sample onto the wax paper.

Continue adding 2µl droplets to the wax paper to write your initials.

As you work, observe the consistency of your droplet size. If you see a variance, wipe the droplets off and start again.

When you are finished, have your teacher check your work. You may wish to take a photo of your work to include in your Laboratory Notebook.

Ready the work area for your partner.

Conclusion

- Which pipets are the most accurate and why?

What could lead to inaccuracy in the volumes transferred when using a volumetric pipet?

What could lead to inaccuracy in the volumes transferred when using a micropipet?

How would you determine the accuracy of a pipet?