Name_____________________________________________________

Activity 1.2.1 Machine Settings

Purpose

Have you seen a lawn with dark and light green stripes? Or a lawn with patches of weeds? Lawns with a color variation or weed patches may be the result of improper applications of fertilizer or herbicide. Agricultural crops also require precise treatments, such as fertilizer and herbicide, to increase their production. Farmers use sprayers and dry broadcast spreaders to apply those treatments. What causes a machine to lack precision when applying fertilizer and herbicides?

The calibration and correct use of application equipment affect precision. Technicians calibrate sprayers and spreaders to apply a specific amount of a treatment to a field. Liquids sprayed on a field are measured in gallons per acre or liquid ounces per square foot. Technicians measure dry material spread on a field using pounds per acre or ounces per square foot. A combination of factors affects the application rate of equipment. The height at which the sprayer or broadcaster applies treatment on to a field affects the application width. The speed of the equipment also affects the rate. The faster the equipment travels across the field, the lower the application rate. Even if the technician calibrates the equipment for known factors, such as application height and speed, the application rate can still be affected by poor planning and improper use of equipment.

When a farmer is treating a field, he or she wants to avoid skips while limiting overlaps. A skip is an area of the field where an applicator applied little or no treatment. An overlap is an area where an application was applied more than once. Skips and overlaps can be harmful to a crop and affect production. The light green stripes you see on a lawn is an example of a skip where the broadcaster did not spread fertilizer, or a reduced amount was applied. A brown burnt area on a lawn may be where an overlap occurred with too much fertilizer. It is nearly impossible to avoid no overlap in a field while making an application. The edges of an application will have a reduced application rate. By slightly overlapping the edges of the broadcast or spray width, the producer ensures the entire field is treated.

The planning process for fertilizing and spraying a field with large equipment over hundreds of acres is the same as it is when treating a lawn with handheld equipment. How is equipment calibrated for treating a lawn with dry fertilizer and liquid herbicide?

Materials

| Per class: Bag of coarse sand Water source with hose Outdoor area with clean dry surface | Per pair of students: Stopwatch Masking tape 25′ tape measure Weigh dish Plastic spoon |
| --- | --- |
| Per group of four students: Handheld fertilizer spreader Electronic balance Handheld sprayer Measuring cup (ounces) | Per student: Agriscience Notebook Engineering Notebook Pen PPE |

Procedure

Work with your partner to determine the settings and walking speed for applying dry fertilizer and liquid herbicide to a lawn. Before starting the activity, your teacher will demonstrate how to use the fertilizer spreader and sprayer with appropriate PPE. Your teacher will assign half of the student pairs to complete Parts One and Two, while the other half complete Parts Three and Four. Finish your assigned parts and then swap equipment with another pair to complete the activity. Wear proper PPE associated with the equipment.

Part One – Broadcast Rate

The fertilizer should be spread on the lawn at a rate of 40 ounces (oz) per 1000ft2. Calculate the rate of your handheld spreader and determine how to adjust if necessary. Record all calculations, measurements, and answers to analysis questions in your Engineering Notebook. For safety purposes, you will use sand to simulate the fertilizer.

Use an electronic balance and plastic spoon to measure 5oz of sand in a weigh dish.

Fill the fertilizer spreader with the 5oz of coarse sand.

Go outside to a smooth surface as instructed by your teacher.

Use a tape measure and masking tape to measure and mark a start and finish line 10ft apart.

Hold the fertilizer spreader at the height you will use it, then have your partner measure the distance from the spreader to the ground.

Spread the sand while walking from the start to finish line.

Spread the sand with the settings at the maximum rate.

Your partner will use a stopwatch to measure the time it takes for you to walk the distance.

On the ground, examine the edges of the broadcast that have a reduced rate and need to be overlapped.

Use a tape measure to find the width of the broadcast where the rate is constant. Convert inches to decimal feet when necessary.

Multiply the constant broadcast width by the distance walked (10ft) to find the area covered by the sand.

Area = constant broadcast width × 10ft

Pour the sand remaining in the fertilizer spreader into the weigh dish.

Use an electronic balance to measure the sand in the weigh dish.

Subtract the weight of the remaining sand from 5oz to find the total oz spread over the area.

5oz - weight of remaining sand = oz spread

Find the rate of your fertilizer spreader by dividing the total oz spread by the area.

Rate = ounces spreadarea (ft2)

Use the following ratio to find the rate per 1000 square feet.

ounces spreadarea (ft2) = ounces1000ft2

Find your walking speed by dividing the distance (ft) by time (sec).

Speed = Distance (ft)Time (sec)

Answer the following analysis questions.

How does the current rate compare to the recommended rate?

How many feet apart should each path be to avoid skips while spreading fertilizer?

How can you change the application process to increase the accuracy of the application rate?

Part Two – Broadcast Improvement

Work with your partner to decide on changes to the application process so the application is closer to the recommended rate. Test your recommended changes by repeating the procedure and calculations from Part One. Record your recommendations, calculations, and results in your Engineering Notebook.

Part Three – Sprayer Rate

The herbicide mix should be sprayed on the lawn at a rate of 36 liquid oz per 1000ft2. Calculate the rate of your handheld sprayer and determine how to adjust if necessary. Record all calculations, measurements, and answers to analysis questions in your Engineering Notebook. For safety purposes, you will use water to simulate the herbicide.

- Fill the handheld sprayer approximately a third full with water.

Use masking tape to mark a start and finish line 10ft apart on a clean hard surface.

While holding the spray nozzle at the height you will use it, have your partner use a tape measure to measure the distance from the spray nozzle to the ground.

Spray the water while walking backward from the start to finish line to avoid walking through the spray.

Spray the water while holding the trigger all the way in.

Your partner will use a stopwatch to measure the time it takes for you to walk the distance.

On the ground, examine the edges of the spray that have a reduced rate needing to be overlapped.

Use a tape measure to find the width of the spray where the rate is constant. Convert inches to decimal feet when necessary.

Multiply the application width by the distance walked (10ft) to find the area covered by the spray.

Area = constant spray width × 10ft

Find the amount sprayed by spraying the water into a measuring cup for the same time it took for you to spray the area.

Find the rate of the sprayer by dividing the total ounces in the cup by the area covered.

Rate = ounces sprayedarea (ft2)

Use the following ratio to find the application rate per 1000 square feet.

ounces sprayedarea (ft2) = ounces1000ft2

Find your walking speed by dividing the distance (ft) by time (sec).

Speed = distance (ft)time (sec)

Answer the following analysis questions.

How does the current rate compare to the recommended rate?

How many feet apart should each path be to avoid skips in the spray application?

How can you change the application process to increase the accuracy of the application rate?

Part Four – Spray Improvement

Work with your partner to decide on changes to the spraying process that will increase the precision of the spraying rate. Test your recommended changes by repeating the procedure and calculations from Part Three. Record your recommendations, calculations, and results in your Engineering Notebook.

Conclusion

- How are application rates of agricultural equipment measured?

- What factors will affect the application rate of a fertilizer spreader and herbicide sprayer in a field?

- What would happen to a lawn treated with equipment that has not been calibrated?