Name_____________________________________________________

Activity 1.2.2 GIS Map

Purpose

When was the last time you used a paper map or atlas to plan a trip or find a location? You probably use more electronic maps on a computer or phone than paper maps. How are digital maps created?

A digital map is a database of information displayed visually for comparing geographic data. Computer software called GIS, geographic information systems, visually displays spatial data that is georeferenced to specific locations on the earth. Georeferencing is the alignment of geographic data to a known coordinate system so it can be viewed, queried, and analyzed with other geographic data. Latitude and longitude coordinates are used to georeference information to specific locations. Figure 1 shows the earth with latitude and longitude lines.

| Latitude lines run horizontally north and south of the equator. Latitude coordinates north of the equator are positive and those south of the equator are negative. The equator has a latitude of zero degrees. The north pole is 90° and south pole is -90°. Therefore, the range of latitudinal coordinates is 90° to -90°. Longitude lines run vertically east and west of the prime meridian, which has a value of zero degrees longitude. Longitude degree coordinates to the west of the prime meridian decrease until they are half way around the globe and have a value of -180°. Latitude degree coordinates to the east of the prime meridian increase until they are half way around the globe and have a value of 180°. The range of longitudinal coordinates is 180° to -180°. | https://en.wikipedia.org/wiki/File:Latitude_and_Longitude_of_the_Earth.svg |
| --- | --- |
|  | Figure 1. Latitude and Longitude Lines |

Geographic information systems use vectors, features, attributes, and layers to organize and display spatial data. Maps contain three types of vectors called points, lines, and polygons. Points represent a single georeferenced coordinate. Lines on a map are a connected series of coordinates. Polygons are connected lines defining a specific geographic area. Points, lines, and polygons represent features, or real-world objects on a map. Features include manmade structures such as roads, buildings and fences, along with natural objects including trees, rivers, and lakes. Attributes are information about a specific feature. For example, a lake’s attributes include its area, depth, and fish content. All attributes have values assigned. Those values may be qualitative or quantitative. Area, depth of a lake, and number of fish are quantitative and are represented with a number. The type of fish in the lake is qualitative and are represented by written description or name.

GIS displays spatial data in layers. A layer contains features represented by colors and shapes to describe the feature’s attributes. GIS maps display multiple layers of features for comparison, analysis, and reference. A base layer is the first layer used. A base layer is a map with physical features a researcher can use for reference when analyzing his or her data.

Collecting GIS data about a farmer’s field often starts with collecting soil samples and production data. Soil sample locations would be an example of a feature on a map. Soil pH is an example attribute you can collect and display on the map. Farmers can use GIS to compare soil pH to production data to determine if there is a relationship between soil pH and productivity. If there is a relationship, a plan can be made to manage the pH and increase productivity.

How is field information georeferenced in a geographic information system?

Materials

| Per pair of students: LabQuest2 USB cable Vernier pH sensor Computer with Logger Lite® software and internet access ArcGIS account Distilled water Rinse water bottle 250ml beaker Garden trowel Plastic spoon | Per class: Outdoor area with defined boundaries 10 white flags |
| --- | --- |
|  | Per student: Agriscience Notebook Engineering Notebook Pen PPE |

Procedure

Work with a partner using GPS and LabQuest2 to collect spatial data about an outdoor site. Then use GIS to display the data in the form of vectors. Record answers to your analysis questions in your Engineering Notebook.

Part One – GPS Data Collection

- Gather the following materials to bring to the site.

- LabQuest2

- pH sensor

- distilled water bottle

- Garden trowel

- 250ml beaker

- Plastic spoon

- Set up the LabQuest2.

- Turn on the LabQuest2.

- Plug in the pH sensor to Channel 1.

- Tap Sensors and select Sensor Setup.

- Choose GPS and tap OK.

- Tap Mode and choose Events with Entry.

- Type Sample Site for the Name and leave units blank.

- Tap OK.

- Press the Start button.

- Go to the site assigned to you by your teacher.

- Collect a sample of the soil at the site using a garden trowel and prepare for testing.

- Use the trowel to remove the litter and organic layer from the ground.

- Use the trowel to loosen soil beneath organic layer.

- Add approximately 100ml of distilled water to the 250ml beaker.

- Place four spoonsful of loosened soil in the 250ml beaker and stir.

- Wait approximately two minutes for soil to settle.

- Place the pH sensor in the water in the 250mL beaker to measure the pH. Do not place in settled sediment.

- While standing at the sampling site, wait for the longitude, latitude, and soil pH to stabilize.

- Tap Keep.

- Enter your sample site number.

- Remove the pH sensor from the beaker and rinse with distilled water.

- Leave the beaker with the soil solution at the site.

- As instructed by your teacher. Go to the next sampling site that has a soil solution prepared by a previous group.

- Repeat Steps 6 – 11.

- Go to each sampling site that has a prepared soil solution and repeat Steps 6 – 11.

- When you have collected data for all 10 sites, tap Stop.

- Tap File and choose Save.

- Save file as 1.2.2YourInitials and tap Done.

- Tap Save.

- Dispose of soil solution at your final site by dumping on the ground and rinsing beaker with distilled water from water bottle.

- Return to the classroom and answer the following analysis questions.

What was the feature you georeferenced at each site?

What attribute was collected at each site describing the feature?

Part Two – Data Conversion

GPS and pH Data

- Go to a computer with Logger Lite® software.

- Open Logger Lite® and attach LabQuest2 to the computer using a USB cable.

- Choose File and select LabQuest Browser >>> Open.

- Open the file titled 1.2.2YourInitials.

- Click File and choose Save.

- Type the file name as 1.2.2YourInitials and click Save.

- Choose File, Export As, and select txt Format.

- Type the file name as 1.2.2YourInitials and click Save.

Your teacher will instruct you on where to save the file.

Part Three – GIS Map

- Open a web browser, go to www.arcgis.com, and log in using the username and password provided by your teacher.

- Click on Basemap and select Imagery.

- Your teacher will provide the address of the soil sampled area. In the search bar, type in the address and click on the search button.

| - Click on Add and choose Add Layer from File, as seen in Figure 1. - Click Choose file and open 1.2.2YourInitials.txt. - Click Import Layer. You will see ten dots of different sizes on your screen. Zoom in on the map to view all dots. - On left side of screen, under Select a drawing style, scroll down, and select Location. All dots should be uniform and of the same size. - Click on Details, choose Content, and select More Options (…). |  |
| --- | --- |
|  | Figure 1. Add Layer |
| - Scroll down and choose Rename, as seen in Figure 2. - Change the Layer Name to Sample Site and click OK. - Click on Save and choose Save as. Save the GIS map as 1.2.2Yourinitals. - Click on the Show Table icon below the Layer Name, Sample Site, to display data represented in the layer. How many features are in the data table? What attributes are used to describe the features? - Change the attribute displayed in the layer to pH. Click on Change Style () found below the Layer Name, Sample Site. Select pH in the dropdown menu for Choose an attribute to show. Choose Counts and Amounts (Color). Click on Done. |  |
|  | Figure 2. Rename |

- Click on the Show Legend icon beneath Sample Site.

What is the range of pH values displayed on the map?

- Click on More Options (…) under Sample Site and choose Create Labels.

- Change the label features to display the Text (pH value) and select OK.

- Compare the pH values at each site to the locations referenced on the base layer.

What may be affecting the pH of the soil?

- In the top left corner on the screen, click on Add and choose Add Map Notes.

- Name the note Boundary and click on Create.

- Select area and make a polygon surrounding the collection sites.

- Title the area Boundary.

Click on Change Symbol and adjust the color and transparency of the area.

Click Ok.

Close the Area pop-up window.

- Click on at the top of the screen.

- Use the Measure tool to find the area of the boundary (acres) and distance between the highest and lowest pH points (feet).

Record the area and distance in your Engineering Notebook.

- Click on Save.

- Click on Print and choose Map with legend.

Print the map with legend and insert in your Agriscience Notebook for reference.

Conclusion

- How are features georeferenced?

- How is a feature different than an attribute on a GIS map?

- What does a GIS layer display on a map?

- How can a GIS map be used to analyze spatial data?