Name_____________________________________________________

Activity 1.2.4 Overlap Savings

Purpose

Today’s agricultural field equipment is designed to till, plant, spray, and harvest a field quickly due to their size and capacity. However, agricultural input costs including fuel, seed, fertilizer, and herbicide may be wasted due to inaccurate applications. For example, as a sprayer with a 60ft wide boom travels back and forth across a field applying an herbicide, part of one 60ft path may overlap with another. Where a wide overlap occurs, wasted herbicide costs a producer money. Overlap in a field can occur with tillage, planting, and harvesting equipment as well. A farmer increases input costs, such as fuel and time, if a tractor makes additional trips back and forth across a field due to overlap. How can a farmer use GPS technology to increase efficiency of equipment and prevent overlap?

Guidance systems in field equipment use GPS to prevent overlap and ensure the operator is driving a straight line. One type of guidance system is a lightbar indicating whether the operator needs to move the equipment to the left or to the right to prevent overlap of an application. Newer guidance systems have taken the controls away from the operator and automatically steer the equipment, guiding it in the most efficient path for agricultural applications by decreasing overlap.

What are the input cost savings of using a GPS guidance system?

Materials

Per student: Agriscience Notebook Engineering Notebook Calculator Pen

Procedure

Work individually to calculate the cost of tilling a field. Then find the cost savings of using a guidance system.

Part One – Traditional Overlap

A farmer is chisel plowing a 160-acre square field. The chisel plow has a width of 30 feet. Follow the steps below to calculate the labor and fuel costs of tilling the field with a 3ft overlap. Round to the nearest hundredth (0.00) when making calculations, unless noted otherwise. Record all calculations in your Engineering Notebook.

Calculate the total number of square feet of the field. (43,560 square feet = 1 acre)

Area (ft2) = 43,560ft2 x acres

The field is a square. The square root of the square feet in the field is equal to the outer dimensions of the field. Calculate dimensions of the field.

Subtract the overlap from total width of the chisel plow to find the tilled width for each path. Then find the number of trips the farmer will make across the field with the chisel plow. Round to highest whole number.

Number of trips = width of field / width tilled by chisel plow

Calculate number of miles the tractor travels to till the entire field. Refer to the field dimensions for the length of each trip. (5280ft = 1 mile)

Miles = number of trips x distance of each trip

Find the total hours it will take for the tractor and chisel plow to till the field if the tractor travels at 4.5 miles per hour.

Total hours = total distance / speed

The tractor uses 12 gallons of fuel per hour while chisel plowing. Find the total number of gallons of fuel needed to till the field.

Total gallons = total hours x gallons per hour

Calculate the total cost for fuel and labor based upon the following rates.

Fuel cost = $2.50/gallon x total gallons

Labor cost = $15/hour x total hours

Calculate the total fuel and labor cost per acre by adding the cost of fuel and labor and dividing by total acres.

Cost per acre = total costtotal acres

Part Two – Overlap Savings

The farmer has considered adding a guidance system to his tractor. Calculate the labor and fuel cost savings if the guidance system reduces the overlap to 0.5ft on a 160-acre field. The farmer owns 1600 acres (ten 160-acre fields) and plans to till all acres twice per year. Find the total fuel and labor savings the farmer will have per acre. Then calculate the cost savings for the year.

Conclusion

- How do guidance systems reduce input costs while tilling soil?

What other farming equipment could a famer use a guidance system with to save time and money?

What factors should a farmer consider before purchasing a guidance system for his or her equipment?