Name_____________________________________________________

Activity 1.3.1 Beam Design

Purpose

As you look around your classroom, you see tables, chairs, computers, desks, and cabinets that all started out as an idea. In the past, engineers would sketch on paper and then work by hand to create a detail drawing of their idea. How are engineering ideas developed today?

Besides sketching and drawing on paper, engineers use computer-aided design (CAD) software to articulate ideas in more detail. CAD software allows engineers to develop a three-dimensional model from a sketch. The model can be viewed from all angles and modified to fit an engineer’s criteria with a click of a mouse. All CAD software has similar functions, but each requires practice and may use different terminology.

When starting to model using CAD, an engineer will first make a two-dimensional sketch on a plane using basic geometric shapes such as rectangles and circles. The sketch may be a single-sided view or cross-section of an object. Figure 1 is the top view of a square that will be developed into a water tank. Once a shape is established, dimensions are added as seen in Figure 2. After dimensions are assigned, the drawing is extruded to make a three-dimensional object as seen in Figure 3.

| Figure 1. Top View of Tank | Figure 2. Dimensions | Figure 3. Extruded Tank |
| --- | --- | --- |

The three-dimensional object can be shaped using options, such as fillet, draft, or chamfer, as seen in Figure 4. Fillet rounds corners, draft changes line angles, and chamfer cuts material off in a straight line.

| Fillet | Draft | Chamfer |
| --- | --- | --- |
| Figure 4. Shaping Options |  |  |

| The block of material in Figure 4 is shown in a solid form. By removing substance from the core, the block becomes a hollow water tank, as shown in Figure 5. CAD software has many other design features to design objects according to your imagination. How can you use CAD to create a three-dimensional object from a basic sketch? |  |
| --- | --- |
|  | Figure 5. Water Tank |

Materials

| Per student: Agriscience Notebook Engineering Notebook Pen | Computer Onshape® account |
| --- | --- |

Procedure

Work individually to design four structural beams using a CAD program. Then share your designs with your teacher.

Part One – Solid Beam

Sketch a solid beam by hand in your Engineering Notebook. The dimensions of the beam are 0.50″ x 0.50″ x 7.00″. The sketch does not have to be to scale.

Log into Onshape® using your assigned username and password.

Click on Create and select Document.

In the document name, type SolidBeam_YourInitials and click on Create document.

Select the menu button at the upper left corner of the screen and choose Workspace units.

Check that units are in inches. Change to Inches if they are not.

Click on the green check mark to accept Workspace units.

On the right side of the screen you will see a cube representing the views you can use for sketching your design. Click on the arrows surrounding the cube to rotate the planes. As you rotate the cube, you will see rotating blue squares representing the planes.

Click on Sketch in the upper left corner to sketch on a plane.

Rotate the cube to view the top plane. Select the blue center square with the word ”Top” in the upper right corner. In the Sketch 1 box, you will see the Sketch plane as Top plane.

Select Rectangle in the upper left corner and choose Center point rectangle.

Click on the center of the top plane and make a rectangular shape.

Click on Dimension on the top bar and select a horizontal line on the rectangle.

Change the width of the rectangle to 7.00″.

The rectangle will extend beyond the visible blue part of the plane used for reference, the actual plane extends infinitely.

Click on Dimension on the top bar and select a vertical line on the rectangle.

Change the width of the rectangle to 0.50″.

Click on the green check mark to accept Sketch 1.

Make a three-dimensional beam by clicking on Extrude.

Select the face of Sketch 1.

Change Depth to 0.50″.

Click on the green check mark beside Extrude 1.

Hold the right mouse button to rotate and view your beam.

Round the 7-inch edges of your beam.

Select Fillet.

Change Radius to 0.05″.

Select all four 7-inch edges of the beam. You will need to rotate your beam to select on edges.

Click on the green check mark beside Fillet 1.

Use the center wheel of mouse to zoom in and view your edges.

Part Two – Hollow Beam

- Select Document Menu in the upper left corner of the screen.

Select Copy workspace.

Change the document name to HollowBeam_YourInitials and click on Create document.

Rotate the planes to view the right side of the object.

Click on Sketch and select the face of the square. The plane you will be sketching on is the right face of the beam called Face of Extrude 1.

Use the rectangle tool to sketch a rectangle inside the face of the square.

Click on Dimension tool.

Click on an outside and inside line parallel to each other and change the distance to 0.10 inches.

| Repeat Step 8 until all sides are 0.10 inches from the sides as seen in Figure 6. Click on the green check mark by Sketch 2. Click on Extrude and select the face of Sketch 2 (inner square sketched on right face of beam). In the Extrude box, select Remove and change the depth to 7 inches. Click on the green check mark by Extrude 2. Scroll and zoom in and around your hollow beam design. Select Document Menu and choose Close document. |  |
| --- | --- |
|  | Figure 6. Rectangular Dimensions |

Part Three – I-Beam

- Open SolidBeam_YourInitials.

Select Document Menu  and choose Copy Workspace.

Change the name of the document to I-Beam_YourInitials.

Change the view to the right side of the beam.

Select Sketch and choose the square end of the beam as the plane to sketch on (Face of Extrude 1).

| Make an I shape by sketching two rectangles on the Face of Extrude 1 with the dimensions shown in Figure 7. Click on the green check mark by Sketch 3. Use the Extrude option to remove 7 inches from each side of the beam to make a three-dimensional I beam. Use the fillet option to round the sharp 7-inch edges of the I-beam to a 0.05″ radius. Your beam should look like Figure 8. Select Document Menu and choose Close document. Part Four – Support Beam - Click on Create. Type in the document name SupportBeam_YourInitials and click on Create document. Click on Sketch in the upper left corner to sketch on the Right plane. |  |
| --- | --- |
|  | Figure 7. I Beam Dimensions |
|  | Figure 8. Completed I Beam |

| Use the Corner rectangle option to sketch a 0.10″x0.50″ rectangle on the Right plane as seen in Figure 9. A corner of the rectangle should be the center point of all intersecting planes. Click the green checkmark to accept Sketch 1. |  |
| --- | --- |
|  | Figure 9. Rectangle |
| Choose Sketch and select the front plane. Use the Corner rectangle option to sketch a 7.00″x0.50″ rectangle with a corner of the rectangle at the center point of all intersecting planes. See Figure 10. Click on the green check mark to complete your sketch. Hold the right mouse button to view the two sketches on different planes. |  |
|  | Figure 10. Intersecting Rectangles |

Choose Sweepand select the 0.10″ x 0.50″ rectangle as the Faces and sketch region to sweep.

Select Sweep path and click on Sketch 2 in the left side menu for the Sweep path.

Select the green check mark to complete Sweep 1.

Select Sketch and choose the top face of the beam as the Sketch plane.

Sketch a 0.10″ x 0.10″ square 0.20 inches from the top and 1.75 inches from the center of the rectangle as seen in Figure 11.

Select and highlight all sides of the 0.10″ x 0.10″ square as seen in Figure 12.

| Figure 11. Square Sketch | Figure 12. Selected Square |
| --- | --- |

Click on Linear pattern to make a copy of the shape.

Change the distance between the copy to 1.75 inches and the number of copies to 3x as shown in Figure 13.

Figure 13. Copied Squares

| Select the green check mark to complete Sketch 3. Choose Extrude and select Sketch 3 in the Features column on the left side of the screen. Change the Depth to 0.50 inches and the Direction so the extrusion extends through both long rectangles. View Figure 14. Select Document Menu and choose Close document. |  |
| --- | --- |
|  | Figure 14. Support Beam |

Part Five – Document Sharing

Share your beam designs with your teacher to be three-dimensionally printed for use later in the lesson.

- Open SolidBeam_YourInitials.

Left click on Share in the upper right corner.

Enter your teacher’s email address.

Type your name, APP number, and the type of beam in the message box.

Example: Your Name 1.3.1 Solid Beam

Repeat Steps 1 – 4 for each of the remaining beam designs: hollow beam, I-beam, support beam.

Conclusion

- What are the advantages of a computer-aided design versus a hand-drawn design?

How is CAD used to make three dimensional models?

What CAD tools can you use to alter and change shapes?

How can you use CAD for agricultural design applications?