Name_____________________________________________________

Activity 1.3.2 Design Decisions

Purpose

Engineers may have multiple designs to choose from. What factors will influence their decision when selecting a design?

Engineers research designs to determine which will be most effective. They investigate innovative technologies, visit locations where similar designs are used, and work with other members of an engineering team. Many times, experimental research is done when a team makes a decision. Experimental research involves testing designs and materials for strength, calculating economic costs of producing the product, and determining product effectiveness.

Once an engineering team has completed their experimental research, they will compare their data to determine the design solution. Identifying the solution will involve analyzing the advantages and disadvantages of each design. Choosing a design results in the team making trade-offs. A trade-off is selecting one criterion or quality for a design at the risk of sacrificing another lost.

Trade-offs considered in a design will vary based upon the criteria and constraints of the project. Economic feasibility is considered when the cost of the project is a high priority. The ability to easily manufacture and use the product can also impact costs and choices. With today’s environmental concerns, the products environmental impact may be considered as well. Engineers use a trade-off matrix, which looks like the chart in Figure 1, to organize considerations and rank designs based upon those considerations.

| Design | Economic Cost | Manufacturing Time | Environmental Impact |
| --- | --- | --- | --- |
| Design One | 1 | 1 | 3 |
| Design Two | 2 | 3 | 2 |
| Design Three | 3 | 2 | 1 |
| Figure 1. Example Trade-off Chart |  |  |  |  |

In Activity 1.3.1 Beam Design you constructed prototypes of various beams. The structural stability, cost, production, and use of the beams are all considerations related to selecting a beam design. When a structural load is placed on a beam, the beam will bend and be deflected from its straight position. The greater the displacement, the less structural stability. Material cost is another aspect considered when producing structural beams. Beams made of steel can become very heavy and expensive with excess material use, which increases production costs. The design of the beam can also impact how easy it is to use or maintain. A poor design may affect how the beam attaches to other components in the structure. In addition, certain designs may cause the pooling of water, increasing corrosion of the beam.

How are experimental research and trade-offs used to select an engineered beam?

Materials

| Per class: Vernier Structural Materials Tester (VMST) U-bolt from VMST tackle LabQuest2 Per group of four students: Electronic balance Four beams from Activity 1.3.1 Beam Design | Per student: Pen Agriscience Notebook Engineering Notebook PPE |
| --- | --- |

Procedure

Test your structural beams for strength. Then observe and record physical characteristics of each beam. Use a tradeoff matrix to organize data about each beam and choose one for application.

Part One – Strength

Observe your teacher’s demonstration of the Vernier Structural Materials Tester.

| Make a Table in your Engineering Notebook for recording the displacement and force applied to each beam. Go to Step 4 to test the strength of an assigned beam when instructed by your teacher. Move on to Part Two and Part Three while waiting. Put on safety glasses. Set the cross bars at 6cm. Place an example beam on the crossbars. Each end of the test beam should rest on the crossbars with the center of the beam over the threaded tackle as seen in Figure 1. |  |
| --- | --- |
|  | Figure 1. Beam Placement |
| Adjust the height of the threaded tackle connection by turning the displacement wheel clockwise to raise it as far as possible. Attach the U-bolt to the beam as seen in Figure 2 Attach the ring to the threaded tackle connection and slide the U-bolt through the hole. Place the U-bolt plate on top of the beam. Slide both lengths of the U-bolt through the plate and screw on both nuts. Turn the displacement wheel counter clockwise on the VSMT until the U-bolt is touching the beam and there is about to be tension between the beam and U-bolt plate. |  |
|  | Figure 2. Tackle Placement |

Turn on and set up the LabQuest2.

Plug the VSMT force sensor into Channel 1.

Plug the VSMT displacement sensor into Digital 1.

Tap on Sensor and choose sensor setup.

Select DIG 1, choose VSMT Displacement and tap on OK.

Tap on Displacement on the Meter screen and choose zero to zero out displacement.

Tap on Force on the Meter screen and choose zero to zero out displacement.

SLOWLY turn the dial on the VSMT counter clockwise until the displacement is 0.500cm.

Record the force and displacement in your Engineering Notebook and share your results with the class.

How did the shape of the beam affect its displacement?

Which beam would you choose if selecting only for strength?

Part Two – Beam Observations

- Make a table similar to Table 1 in your Engineering Notebook for recording the mass (grams) and calculating material cost of each beam.

| Table 1. Mass and Cost |  |  |  |  |
| --- | --- | --- | --- | --- |
| Beam | Mass gram | Cost per gram | Total Cost | Force to Deflect 0.500cm |
|  |  | $0.25 |  |  |

Use an electronic balance to find the mass of each beam and record the mass in your Engineering Notebook.

Calculate the material cost of each beam by multiplying the mass by the cost per gram. Record the cost for each in your Engineering Notebook.

In the table, record the force to needed to deflect each beam 0.500cm.

Which beam has lowest material cost?

What other factors could affect the cost of making each beam?

Observe the shapes and design of each beam in detail. Use your observations to answer the following questions.

Which beam would be the most difficult to manufacture? Explain why you believe this is so.

Which beams would be difficult to fasten to a structure? How would it be difficult to fasten?

Which beams are designed for easy maintenance and prevention of corrosion? What features prevent those beams from corroding?

Part Three – Trade-off Matrix

Work with your group to develop a table and scoring system to select one of the four structural beams based on its ability to support a flat roof of a building that retains its shape with limited displacement. Use the following criteria.

Economic feasibility

Strength and function

Mass

Manufacturing

Ease of use

Ease to maintain

Record your scoring system description in your Engineering Notebook. Use your table and scoring system to select your beam. Answer the analysis questions.

Which beam was chosen? Why?

Why were each of the other designs not chosen? Provide one reason for each.

How did the design table assist in the group decision-making process?

Conclusion

- What factors determine the selection of an engineer’s design?

- When would a chosen design not be considered the “best”?

- How do trade-offs impact engineering design decisions?