Name_____________________________________________________

Activity 1.3.4 Build More Parts

Purpose

As you develop more complex designs, you will need more CAD tools. How do those tools work? How does a two-dimensional concept become a three-dimensional model?

The CAD system Onshape® uses boundary representation (BREP). BREP creates solid objects from two dimensional shapes. First, a two-dimensional shape is drawn. Then the shape is extruded, swept, or revolved around a path to make a three-dimensional part. You used the sweep and extrude functions to design beams during Activity 1.3.1 Beam Design.

BREP software allows engineers to quickly change designs but requires engineers to think two- and three-dimensionally. For example, Figure 1 looks more like a T than a wheel. But if the two-dimensional shape revolves in circular motion around a path, then a wheel becomes apparent as shown in Figures 2 and 3.

| Figure 1. T Drawing | Figure 2. Half Revolution | Figure 3. Full Revolution |
| --- | --- | --- |

Often, more than one part is used to complete a design. When designing multiple parts that fit together, engineers need to consider tolerances. Tolerance is the amount of variation allowed in part dimensions that will not affect the functionality of the part. For example, a shaft with a 1.0″ diameter cannot fit into a bearing on a wheel with a 1.0″ diameter. The tolerance for the shaft must be in a range smaller than the bearing, but not too small to make the fit loose. Once all parts are designed, engineers use a single document to organize their parts to be assembled. How are BREP and tolerance used to design multiple parts for assembly?

Materials

Per student: Agriscience Notebook Engineering Notebook Handle Drawing Onshape® account Computer with internet access

Procedure

| Practice using design features of CAD software by constructing a box, shaft, wheel, and handles for a wheelbarrow similar to Figure 4. |  |
| --- | --- |
|  | Figure 4. Wheelbarrow |

Part One – Wheel

| - Start a new document on Onshape® and change the name to Wheelbarrow_YourInitials. - Right click on Part Studio 1 and choose Rename. - Change the name of Part Studio 1 to Wheel and Box. - Begin sketching a cross section of a wheel by making a 3.00″ x 1.00″ corner rectangle on the Top plane with one corner at the intersection of all three planes represented by the dot in the center as seen in Figure 5. - Choose Arc and select 3 point arc to make a 3-point arc on the right side of the rectangle with a 0.50″ radius as seen in Figure 6. Select the corners of the rectangle as the first two points. - Click on the green check mark to complete Sketch 1. - Use the three-point circle option to make a second sketch on the Front plane with a circle 1.05″ diameter. Select the intersection of the three planes as your first point. The edge of the circle must be touching the center dot representing the intersection of all planes as seen in Figure 7. - Click on the green check mark to complete Sketch 2. - Change views to verify the edge of the circle intersects a corner of the sketch made on the top plane and does not go through the sketch. Refer to Figure 8. Click on Revolve on the top bar. Select both faces of Sketch 1 as the Faces and sketch regions to revolve. Click on Revolve axis and select the edge of Sketch two (circle) as the Revolve axis. Click the green check mark to complete Revolve 1 and view your wheel similar to Figure 9. |  |
| --- | --- |
|  | Figure 5. Rectangle |
|  | Figure 6. Rectangle with Arc |
|  | Figure 7. Circle Intersecting Center |
|  | Figure 8. View of Circle and Rectangle Sketches |
|  | Figure 9. Wheel |

Part Two – Box

- Sketch a 12.00″ x 24.00″ rectangle on the Top plane of the Wheel and Box part studio.

Be sure the sketch does not intersect with your wheel.

| Click on Extrude and select the face of the rectangle. Change the Depth to 6.00 inches. Click on the green check mark to complete Extrude 1. Scroll to the bottom side of the box. Click on Sketch and select the bottom side of the box to sketch on. Sketch two 1.00″ x 3.00″ rectangles 7.00 inches apart and 1.50 inches from the 24-inch edges of the box. Use Figure 10 as a guide. Click on the green check mark to accept your sketch once completed. |  |
| --- | --- |
|  | Figure 10. Rectangles on Box |

Use the extrude option to extrude the two rectangles 6 inches outside of the box.

Make a 1.05″ diameter circle on the side of one of the two extruded rectangles. The center of the circle is 1.50 inches from the top and sides as seen in Figure 11.

Figure 11. Circle on Extrusion

| Click on Extrude and select the circle. Select Remove and choose Through all from the drop-down menu (Figure 12). Click on the green check mark to complete the extrusion. Select Draft and select the bottom of the box as the Neutral plane. Select the back side of the box furthest from the extruded rectangles as Entities to draft. Unclick Tangent Propagation so only the back side is drafted. |  |
| --- | --- |
|  | Figure 12. Remove Material |

Change Draft angle to 10 deg.

Select the green check mark to complete the draft.

Use the Draft option to change the angles of the other sides as you see fit.

Note: It is OK for the box to intersect with another part. The parts are not physically connected together.

| Use Fillet, and Chamfer options to change the shape of your box as you choose. Click on Shell and select the top face of the box for Faces to remove. Change Shell thickness to 0.25″. Click on the green check mark to complete the shell. See an example shape in Figure 13. Part Three – Shaft and Handles - Click on the “+” in the lower left corner and choose Create part studio. |  |
| --- | --- |
|  | Figure 13. Example Box |

Right click on Part Studio 2, and choose Rename.

Change the name of Part Studio 2 to Axle and Handles.

Make a round axle with a 1.00″ diameter 9.00″ length.

Make a handle matching the dimensions in the Handle Drawing.

Hint: Use the Revolve function to round the end of the handle.

Click on the side of the handle.

Select Sketch and choose the text icon represented by an “A” at the top of the screen.

Select an area to type on your Initials on the handle.

Use the extrude function to remove 0.25” of material to inscribe your initials on the handle.

Close the document.

Conclusion

- How do you use BREP to convert a sketch into a three-dimensional object?

How does BREP make it easier for engineers to design parts?

What would you like to design using BREP?