Name_____________________________________________________

Activity 1.3.5 Virtual Assembly

Purpose

After an engineer designs a set of parts for a machine, he or she needs to visualize how they will fit together. Industry saves time and resources by using CAD software to assemble parts for a machine. By assigning functions to each fastened part, such as a fixed, rotating, or sliding part, a working machine can be virtually constructed and tested before it is ever physically fabricated. What is the process to assemble virtual parts?

Materials

| Per student Agriscience Notebook Engineering Notebook Pen | Onshape® Computer with internet access |
| --- | --- |

Procedure

Assemble your wheelbarrow parts to make a working product.

Log in to Onshape® and open Wheelbarrow_YourInitials.

Right click on Assembly 1 and rename it Wheelbarrow.

Left click on Insert and click on each part to insert it on to the page. Insert the handle twice so there are two.

Close the Insert option by clicking on the red X.

| Spread out the parts by holding down the left mouse button and moving each part. Left click on the axle to view the arrows and arc as seen in Figure 1. Use the mouse to click on the arcs and arrows and move the axle into different positions. Use this option throughout the activity to assist you on viewing your parts during assembly. |  |
| --- | --- |
|  | Figure 1. Arrow and Arcs |
| Right click on the axle and choose Fix to hold it in place. Left click on Revolute mate and choose the end of the axle and the outside hole of the wheelbarrow box as seen in Figure 2. Click on the green check mark to complete the mate. Note the position of the axis to ensure the part is fastened in the correct position. If axle does not fasten in the correct position, use the undo button at the top left of the screen to undo the mate and try again. |  |
|  | Figure 2. Axle and Box Joints |
| Left click on Revolute mate and choose the center of the hole in the wheel and the center of the axle as shown in Figure 3. Click on the green check mark to complete the mate. Note the positioning of the axis to ensure the wheel is fastened in the center position. If the wheel does not fasten in the center position, use the undo button at the top left of the screen to undo the fix and try again. |  |
|  | Figure 3. Centered Mates |

Use the functions learned in Steps 5 – 6 to position the handles so they are close to where they will fasten to the wheelbarrow as seen in Figure 4.

Choose Fasten mate  and select the outside square corner of the handle by the round portion and the outside corner of the box as viewed in Figure 5.

Note the blue axis for each mate connection are parallel to each other.

Use flip primary axis  and rotate secondary axis options to mate the parts correctly.

| Figure 4. Aligned Handles | Figure 5. Parallel Mates |
| --- | --- |

Click the green check mark to complete the mate.

Repeat Steps 14 – 15 for the other handle.

Left click and hold on a handle and move the mouse up and down.

What type of motion does the wheelbarrow make?

Where does the wheelbarrow rotate?

How is a revolute mate different than a fix mate when assembling parts?

Conclusion

- How are assemblies used in a CAD drawing?

Why are different types of mates used to connect parts?

What other parts could be added to the wheelbarrow assembly?