Name_____________________________________________________

Project 1.3.3 Detail Drawing

Purpose

Engineers use drawings to communicate ideas to technicians and tradespeople. Technicians and tradespeople translate the drawing into a constructed physical product. What information needs to be included on the drawing?

During Agricultural Power and Technology, you drew isometric and orthographic drawings by hand. Engineers still sketch and draw some pictures by hand, but use CAD to make detail drawings that are precise and accurate. A detail drawing contains multiple views of an object with dimensions, symbols, and notations describing specific features. Figure 1 displays multiple types of symbols and dimensions used in a detail drawing.

| Diameter | Radius | Center Point | Hole call out | Angle |
| --- | --- | --- | --- | --- |
| Figure 1. Symbols |  |  |  |  |

| Views of orthographic drawings are in a specific order and location on a detail drawing. Figure 2 shows where the front, top, and side drawings should be located. The front view is in the lower left quadrant, the top in the upper left, and the side in the lower right. An isometric drawing providing a three-dimensional view will be in the upper right. Engineers use three types of dimensions in a drawing. Size dimensions describe the length, width, and depth of a part. Shape dimensions inform the technician about features such as angles. Location dimensions show distance between features of a part. Engineers use the following standard rules when using dimension lines. |  |
| --- | --- |
|  | Figure 2. Drawing Placement |

Place dimensions on the outside of the object

Only use dimensions necessary to dimension the object

Do not cross dimension lines

Do not duplicate dimension lines

Overall dimensions are furthest away from the object so intermediate dimension lines do not cross

Place dimensions on the view that best shows the measurement

It is the engineer’s discretion what that view will be

Show location and size of all circles and arcs

All circles and arcs in the drawing have dimensions

Identify circles using diameter

Identify arcs using radius

Locate holes using a center point, as shown in Figure 1.

All detail drawings have a title block with the drawer’s name, drawing title, scale, and date, as used in previous CASE courses. The title block may contain additional information to meet industry standards such as company name, materials used, finishes used, and technical instructions.

How is CAD used to complete a detail drawing of a design?

Materials

| Per student: Agriscience Notebook Engineering Notebook Pen | Computer with internet access Onshape® account |
| --- | --- |

Procedure

Use Onshape® to make a detail drawing for a three-dimensional part.

Open the shared document entitled MSA_Pulley as instructed by your teacher and make a copy of the document. Rename the copy MSA_Pulley_YourInitials. Observe the pulley for specific details such as angles, holes, fillets, and chamfers. Make a detail drawing for the pulley including all dimensions needed for a technician to build the pulley. Your teacher will assess the detail drawing using the following criteria as described in Project 1.3.3 Evaluation Rubric.

Use inches for dimensions

Include isometric, section, and orthographic views

Include all dimensions, angles, and radii needed to recreate the object

Follow all standard dimension rules for dimension lines

Complete the title block with your name, project name, scale, and date developed, steel as the material, and enamel paint as the finish

Print your completed drawing and submit to your teacher for assessment.

Conclusion

What are the benefits of CAD software for engineers making detail drawings?

Which factors determine the dimensions and views needed on a detail drawing?

How could detail drawings be used in the agricultural industry?