Name_____________________________________________________

Activity 2.1.1 Read and Construct

Purpose

Many construction projects use wood to frame walls. A wall has a system of methods and materials used for construction. When constructing wood wall frames, a builder needs to read detail drawings to precisely cut materials to construct the wall. What are the structural components of a wall?

The parts of a wall frame, called framing members, include sole plates, top plates, studs, headers, and sills. Studs are normally made of 2x4 or 2x6 material and are placed 16″ or 24″ apart on center (O.C.). On center is the measurement from the center of one member of a wall frame to the center of the succeeding member. Studs connect the bottom plate and top plate of a wall. Cripple studs are used where rough openings are in the wall for windows, doors, or ventilation. The sides of the rough openings are stiffened using trimmer studs. Headers are made of two studs placed together to carry the load of the structure horizontally across the opening. Windows and appliances such as ventilation fans rest on sill plates supported by cripple studs.

When walls are placed vertically, they are fastened to the floor, joined together at the corners, and have a second top plate called a double top plate overlapping two walls to increase wall stability. When planning a building, one must think three-dimensionally and consider the size of the materials they are using. For example, an 8′x10′ shed cannot have two 8′ walls and two 10′ walls constructed. The walls are 3 ½″ wide (the width of the 2x4). If two 10′ walls are constructed, the 8′ side walls will need to be 7″ shorter in length, taking account for the 3 ½″ width of the 10′ walls on both sides.

How will you use detail drawings to construct four walls for a model structure?

Materials

| Per pair of students: Activity 2.1.1 Framing Prints Balsa wood Balsa wood cutter Architect’s scale Hot glue gun Hot glue sticks | Per student: Agriscience Notebook Engineering Notebook Pen |
| --- | --- |

Procedure

Work with your partner to measure and label the parts of the wall frame and develop a bill of materials. Then construct and assemble four walls for a model building.

Part One – Wall Analysis

Review the detail drawing for each wall and label the following components of the wall on each print. If needed, use page 325 of the course text as a reference.

| Header Trimmer studs Cripple studs | Rough sill Sole plate Top plate |
| --- | --- |

The detail drawings of the wall are drawn to a 1′=1/2″ scale. Use your architect’s scale to find and label the dimensions and O.C. spacing of the components on each of the detail drawings.

Use the detail drawings to develop a materials list in your Engineering Notebook. List the number and size of each piece needed for all four walls.

Part Two – Wall Construction

- Using your bill of materials, work with your partner to cut the pieces for each size needed to make a model at a 1′=1″ scale.

Use an architect’s scale with a 1′=1″ scale and balsa wood cutter to mark and cut the pieces to size.

Use a hot glue gun to assemble each wall using the detail drawings as your guide.

Use a hot glue gun to assemble the walls together as seen on the final print.

Calculate the dimensions needed for a double top plate and place on top of the frame.

Conclusion

- Why is scale a key component to a detail drawing?

How are detail drawings used to construct a building?

What do technicians need to know about a detail drawing to make a materials list?

What information does a detail drawing not have that a builder needs to know?