Name_____________________________________________________

Activity 2.1.3 Green Sheathing

Purpose

The covering, or sheathing, of a building serves multiple purposes including insulating and structural stability. Greenhouses are designed to grow plants indoors in a controlled environment. Greenhouse sheathing transmits light and converts solar energy into thermal energy, or heat. What are the sheathing or covering choices for a greenhouse?

A variety of glass and plastic products are available for greenhouse coverings. Glass panel greenhouses provide a higher light intensity and have a longer life span than plastic. However, glass coverings are more expensive. Plastic flexible film greenhouses are less expensive but have a lower life expectancy. Sheathing on many modern greenhouses are rigid plastics such as polycarbonate. Polycarbonate has a lower cost than glass yet is rigid like glass and has greater resistance to physical damage. The type of covering impacts material and operating costs of a greenhouse. Material costs measured in cost per square foot vary based upon the quality of the material.

A builder considers heat and insulation properties when selecting material for a greenhouse. All material covering a greenhouse has insulative properties that transfer heat at different rates. Material with a low R-value, which refers to the material’s resistance to heat flow, allow heat to escape the greenhouse more quickly. Greenhouses with low R-value sheathing in cold environments require supplemental heat to maintain a constant temperature on cloudy days and during dark night hours. Heaters are rated using British thermal units (Btu). A Btu is the quantity of heat required to raise one pound of water one degree Fahrenheit. Heaters with higher Btu output ratings produce more heat.

The type of heating system needed for a greenhouse prescribes the type of heater and heating costs. Heaters with higher Btu ratings are typically more expensive and require gas or electricity to produce heat. Gas and electrical availability and costs are factors to consider before deciding on construction materials.

Geographic location is also an important factor to consider when designing a greenhouse. Greenhouses should be placed where they are exposed to the most sunlight possible. The area should be level, with limited shade. Greenhouses in northern states need to withstand snow load and require higher insulation capabilities due to lower average temperatures.

How do producers select appropriate coverings for greenhouses?

Materials

| Per group of four students: LabQuest2 2 Vernier temperature sensors 6 mil polyethylene plastic Clear tape 6 mil polyethylene, one-gallon plastic bag 6mm single wall polycarbonate 6mm double wall polycarbonate Model structure | Per student: Agriscience Notebook Engineering Notebook MSA Notebook Evaluation Rubric Computer with internet access Pen |
| --- | --- |

Procedure

Work with a group to test coverings for greenhouse properties. Then work individually to determine the total cost of each covering for sheathing a model greenhouse. Complete the activity by developing a trade-off chart to select a greenhouse covering that best fits your area. Record all calculations and tables in your Engineering Notebook.

Part One – Energy Conversion

Wrap the side walls of the model structure with 6mil polyethylene film.

Set up the LabQuest2.

Turn the LabQuest 2 on.

Plug one temperature sensor into Channel 1 and the other into Channel 2.

Tap Mode, change the duration to 240 seconds, and select OK.

| Go outside to a sunny area clear from obstructions. Set the structure outside where it will not be disturbed or shaded. Insert the temperature sensor connected to Channel 1 through the side of the of the structure as seen in Figure 1. Set the temperature sensor connected to Channel 2 beside the structure. Seal air inside the plastic one-gallon bag to simulate a double-layer plastic wall in a greenhouse. Place the bag on top of the structure to simulate a roof. Press the Start button to begin collecting data. A graph of the data will appear after data stops collecting. Copy the graph into your Engineering Notebook and record the high and low temperature for both outside of and inside the structure. Tap the file cabinet to start a new run. |  |
| --- | --- |
|  | Figure 1. Structure and Temperature Sensor |

Repeat Steps 7 – 10 for each of the following coverings. You will collect data for outside of and inside the structure for each run.

6 mil single layer polyethylene

6mm polycarbonate

6mm twin wall polycarbonate

Unplug the temperature sensors and save your data as Thermal Energy.

Which covering produced the most thermal energy?

Which covering increased the temperature at the fastest rate? How do you know?

Part Two – Light Emittance

- Set up the LabQuest2.

Tap Sensors and select Sensor Setup.

Choose Light Intensity and select OK.

Tap on Mode and select Events with Entry.

Change the Name to Plastic.

Select Average over 10 seconds and tap OK.

Make a table in your Engineering Notebook for recording light intensity for each of the following.

No plastic

6mil single polyethylene

6mil double polyethylene

Single 6mm polycarbonate

Double 6mm polycarbonate

Go outside with the plastic samples.

| Set the LabQuest2 on a flat surface with the internal light sensor facing upward. Note the location of the light sensor as in Figure 1. Press the Start button to begin collecting data. The data is recorded in lux. Press Keep. Name the event No Plastic. |  |
| --- | --- |
|  | Figure 1. Light Sensor |

Hold the polyethylene plastic over the top of the light sensor and press Keep.

Name the event Single Polyethylene.

Repeat Steps 8 – 9 for each plastic.

Press the Stop button to end collecting data.

View the table on the LabQuest2 and record the available light for each plastic type. Answer analysis questions in your Engineering Notebook.

How did the plastic covering affect the available amount of light?

How do you believe the plastic affects the plants in a greenhouse?

Part Three – Material Cost

- Calculate the total surface area (square feet) of sheathing needed to cover the actual greenhouse that your model from Activity 2.1.1 Read and Construct, represents. Use the Pythagorean theorem to find the length of the rafter on your model truss from Project 2.1.2 Raising the Roof. Use the rafter length to calculate the area of the roof. Use the following equations for finding surface area and dimensions of triangles on the gable ends of the structure.

Pythagorean theorem: a2 + b2 = c2

Surface area of square = length x width

Surface area of triangle = length x height/2

Calculate the total costs for covering your greenhouse with each of the following material costs. Record your calculations and costs in your Engineering Notebook.

6mil single polyethylene – $0.13/ft2

6mil double polyethylene – $0.26/ft2

Single 6mm polycarbonate – $1.25/ft2

Double 6mm polycarbonate – $1.67/ft2

Part Four – Heat Requirement

- Make a table to record covering types and Btu requirement data in your Engineering Notebook.

Go to the following website http://www.littlegreenhouse.com/heat-calc.shtml.

Enter the actual surface area of your structure.

Find the Minimum Outside Temperature for your location using the recommended reference on the website.

Enter the Minimum Outside Temperature.

Enter 60 degrees as the Inside Temperature to maintain in the greenhouse. This is the minimum temperature to maintain when the sun is not shining.

Enter the Heat Loss Value for the 6mil single polyethylene layer. Heat loss values are listed below the box on the website.

Click on Calculate.

Record the required Btu’s in your Engineering Notebook.

Repeat Steps 7 – 9 for the following coverings.

6mil double polyethylene

Single 6mm polycarbonate

Double 6mm polycarbonate

Use the website to select a heater that will meet the Btu needs for each type.

Record the type of heater and its cost in your Engineering Notebook.

Part Five – Select and Cover

- With your group, brainstorm factors other than energy, light, and costs to consider when selecting a covering for a greenhouse in your geographic area.

Research those factors about the coverings you tested.

Work with your group to develop a tradeoff chart and rating system for choosing a covering.

Use your data and research to complete the tradeoff chart and choose a plastic covering.

What type of plastic did you choose for your greenhouse? Why?

Turn in your Engineering Notebooks for your teacher to assess your research and calculations using the MSA Notebook Evaluation Rubric.

Conclusion

- What information would you want to know about the local environment before constructing a greenhouse?

- What factors affect sunlight intensity and thermal energy found in a greenhouse?

How does the local environment affect the materials chosen for construction a greenhouse?