Name_____________________________________________________

Project 2.1.2 Raising the Roof

Purpose

You began framing a model building in Activity 2.1.1 Read and Construct. What is the process for placing a roof on the model?

| The slope of a roof is called the pitch. The pitch is the ratio of vertical rise to horizontal run. The horizontal run is half the total span, which is the width of the structure. For example, a 6:12 pitch has a rise of six inches for every 12 inches of run (Figure 1). Roofs have a system of rafters connected to the top plate of the building frame. A common shape of a roof is a gable, which is triangular as seen in Figure 1. |  |
| --- | --- |
|  | Figure 1. Roof Span |

Engineers use the pitch and span to find the height of the roof and length of the rafter. The span is the distance the truss extends from one side of the building to the other. The run of a gable roof would be half of the span. Figure 2 shows example calculations for finding the rise, run, and length of a rafter for a structure with a 20ft span and a 6:12 pitch.

| Run The run is ½ of the 20ft span. Run = 10ft | Run=202 Run=10 |
| --- | --- |
| Rise Find the rise using the 6:12 ratio. Cross multiply and solve for x. Rise = 5ft. | Rise10ft=612 Rise =6012 Rise = 5ft |
| Rafter Length Use Pythagorean Theorem to find the rafter distance | a2 + b2=c2 |
| The rise is “a” and run is “b”. Assume the rafter length is “c”. | 5ft2 + 10ft2 = c2 25ft + 100ft = c2 125ft = c2 |
| Take the square root of each side. | √125 = √c2 |
| Rafter length = 11.18ft | 11.18ft = c |
| Figure 2. Rafter Calculations |  |

If the expected load is too large for the rafter itself to hold, a truss system is placed to support the rafters. A formula used to determine the stability of a truss is 2j = m+3 where j represents the number of joints and m represents the number of truss members. If the number of joints multiplied by 2 is larger than the number of members plus 3, the truss is unstable. See Figure 3 for an example of a stable and unstable truss.

| 2j = m+3 | 2j = m+3 |
| --- | --- |
| 2(7) = 9+3 | 2 (8) = 13+3 |
| 14 = 12 | 16 = 16 |
| Unstable Truss | Stable Truss |
| Figure 3. Truss Stability |  |

The weight of material used to make the trusses is also considered when designing a truss for the roof. The weight of the trusses will place a constant load, called a dead load, on the walls the trusses set on. In addition to the dead load, live loads, such as wind, snow, or ice, are also considered when designing a truss. The strength of the truss is found using a live load to dead load ratio. The greater the ratio, the stronger the truss.

Once an engineer has decided on a truss design, multiple trusses are made and evenly spaced on the framed walls. A common spacing for trusses is 24″ O.C.

How will you design a prototype truss for a building?

Materials

| Per pair of students: Balsa wood Architect’s scale Balsa wood cutter Hot glue Hot-glue gun | Per student: Agriscience Notebook Engineering Notebook Engineering Report Template Onshape® Account |
| --- | --- |
|  | Per class: Vernier Structural Materials Tester Vernier truss accessory LabQuest2 Electronic balance 3D printer with PLA plastic |

Procedure

An architectural firm has asked your class to design a truss for a building with the same dimensions represented by your model constructed during Activity 2.1.1 Read and Construct. They will select the truss design with the highest live to dead load ratio proven with a complete engineering report that includes comprehensive data. Cost and material are a constraint limiting your design. The trusses can be made of a composite plastic or wood. You are limited to 36 inches of plastic material or 45 inches of balsa wood for your model truss, which is the equivalent to 24 feet or 30 feet for the actual structure at a 1.5″ = 1′ scale. The firm has determined that the model prototype truss needs to withstand a minimum of 250N of force without displacing more than 0.500cm.

Work with your partner to compete with other pairs in the class to design the strongest truss using one of the assigned materials. Remember, proper recording in your Engineering Notebook is essential while developing your design. Use the engineering design process to design your truss. The architectural firm would like to see a model of your final design, a detail drawing, and completed Engineering Report. Be sure to place the truss on the materials tester simulating the span of the structure. Consider the following criteria in your design.

Pitch

Dead load

Live load

Number of joints

Number of truss members

Selected material

Conclusion

How did pitch affect the amount of material needed for the truss?

How did you determine if your design was stable?

What other factors would you research to improve your final design?