Name_____________________________________________________

Activity 2.2.3 Temperature Controls

Purpose

Have you ever heard a furnace turn on in your house? Seen a light turn on automatically? Or heard a refrigerator shut off after it had cooled? These are examples of control units operating in your home. How do control units work?

Controls in your home, such as heating and cooling, are controlled by programmable logic controllers (PLCs). A PLC is a control system with three major components. PLCs have input devices or sensors collecting data, a computer processor, or controller, reading data, and output devices directed by the computer. Technicians program PLCs to control outputs based upon the readings of an input. A thermostat is an example of a PLC in your home. The input device is the thermometer reading the temperature. The output device is the furnace. The PLC can be programmed to turn the furnace on when the thermometer reads a specific temperature or range of temperatures.

A PLC called a digital control unit (DCU) works with a LabQuest2 to read inputs from sensors and control outputs, such as motors, lights, and alarms. The DCU can be programmed using a simple threshold logic or compound logic statement. A simple threshold logic statement will turn on a device based upon a single sensor reading. Compound logic statements control devices under a variety of readings. For example, a person can program a furnace to turn on in his or her home once the temperature is below 65°F. That would be a simple threshold logic statement. A compound logic statement might tell the furnace to turn on at 65°F and shut off at 68°F.

How do you program a PLC? How do compound logic statements control electronic devices?

Materials

| Per pair of students: LabQuest2 Power adapter Temperature sensor Digital control unit (DCU) Screw terminal bridge with wire leads Electric motor | Per student: Agriscience Notebook Engineering Notebook Pen |
| --- | --- |

Procedure

Work with your partner to attach an electric motor to a digital control unit. Then use simple and compound statements to program a DCU to control an electric motor.

Part One – Digital Control Setup

Turn on the LabQuest2.

Attach the DCU to DIG1.

Choose Sensors and Select DCU Setup.

Tap on DIG1: Digital Control Unit.

Test the lines by placing a check mark by Line 1, Line 2, and Line 3. Observe the DCU.

Stop testing the lines by unchecking the boxes for each line.

Tap OK.

Attach the temperature sensor to Channel 1.

Attach the screw terminal bridge with four wire leads to the DCU.

Attach the power adapter to the DCU.

Plug the other end of the power adapter into a wall outlet to supply power.

Part Two – Logic Statement

- Choose Sensors and Select DCU Setup.

- Tap on DIG1: Digital Control Unit.

- Select Line 1.

Activate Line 1 by placing a check mark in the box.

Change the temperature to °C.

The statement should read if CH1: Temperature is ≥ 30°C.

Tap OK.

Attach the red and black wires to the corresponding wires on the electric motor.

Hold the temperature sensor in your hand to increase the temperature until it exceeds 30°C.

What happens to the motor? Why do you believe this is so?

How can you shut the motor off?

Cool the temperature sensor by releasing it from your hand.

Part Three – Compound Logic Statements

- Choose Sensors and Select DCU Setup.

- Tap on DIG1: Digital Control Unit.

- Select Line 1.

Make sure Line 1 is activated with a check mark in the box.

Tap on the second line drop down menu and choose UNTIL.

On the second line, change the equation to ≤ 29°C.

Tap OK.

Increase the temperature sensor by holding in your hand until the temperature reading exceeds 30°C and then release and cool the sensor so the reading drops below 29°C.

What does the UNTIL function do?

- Choose Sensors and Select DCU Setup.

- Tap on DIG1: Digital Control Unit.

- Select Line 1.

Tap on the second line drop down menu and choose OR.

On the second line, keep the equation ≤ 29°C.

Tap OK.

Increase the temperature by holding the temperature sensor until the motor turns on. Then release the sensor and cool until the temperature drops below 29°C.

What is the purpose of the OR function?

- Choose Sensors and select DCU Setup.

- Tap on DIG1: Digital Control Unit.

- Select Line 1.

Tap on the first line drop down equation menu and change the statement to read ≤ 30°C.

Choose AND in the second line drop down menu.

Tap on the second line drop down equation menu and change the statement to read ≥ 29°C.

Select OK.

Increase the temperature to above 30°C and then decrease to below 29°C and observe when the motor turns on and off.

When did the motor turn on and off? Why?

How is the AND function different than the OR function?

- Choose Sensors and Select DCU Setup.

Tap on DIG1: Digital Control Unit.

Select Line 1.

Deactivate Line 1 by removing the check mark in the box.

Disassemble the LabQuest2, DCU, and temperature sensor and store as instructed by your teacher.

Conclusion

- How are sensors and programable logic controls used to operate equipment?

Where would you find sensors and PLCs used in agriculture?

How are simple and compound logic statements different?

When are compound logic statements needed to program a PLC?