Name_____________________________________________________

Activity 3.1.1 Internal Combustion

Purpose

All engines operate in a specific way with a sequence of events and parts moving in harmony. When a technician works on an engine, he or she needs to understand how and why the engine works before it can be repaired. How could you use a flowchart to diagram operation of an engine?

Flowcharts show the inputs, outputs, and processes occurring in a system. Inputs include all the mechanical parts and energy going into a system. The processes include how the mechanical parts move and use the energy. There are two types of outputs in a system, desirable and undesirable. The desirable output is the work done by the machine. The undesirable outputs include wasted energy and byproducts expelled by the machine.

An engine converts the chemical potential energy found in fuel into mechanical energy. The major inputs of a four-cycle internal combustion engine are the gasoline and oxygen mixed together to make a combustible charge. Once the spark plug ignites the combustible charge, the engine does work, loses energy in the form of heat, and releases exhaust in the form of carbon dioxide and water. Work, heat, and exhaust are all examples of outputs of an engine.

The energy from the explosive charge is transferred throughout the engine by the piston, connecting rod, and crankshaft. The piston moves up and down from top dead center (TDC), position closest to the spark plug to bottom dead center (BDC), position furthest from the spark plug. In addition, valves open and close to allow fuel into and release exhaust from the engine. The processes completed by the engine components, are broken down into five major events called the intake, compression, ignition, power, and exhaust.

What processes are occurring during each event? How can the inputs, processes, and outputs of an engine be diagrammed using a flowchart?

Materials

Per student: Small Engine and Equipment Maintenance text Agriscience Notebook Engineering Notebook Poster board Colored pencils Pen

Procedure

Complete a table organizing the actions of engine components during each event of the four-stroke process. Then construct a flowchart explaining the inputs, outputs, and processes in a four-cycle engine. Record all tables and flowcharts in your Engineering Notebook.

Part One – Small Engine Inputs and Outputs

Record the purpose of each of the following components in your Engineering Notebook. Use the Small Engines Equipment and Maintenance text if needed.

Piston

Intake valve

Exhaust valve

Spark plug

Connecting rod

Crankshaft

Record the inputs and outputs of chemical combustion in your Engineering Notebook.

Part Two – Small Engine Events

- Record a copy of Table 1 in your Engineering Notebook.

| Table 1. Events |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Event | Chemical Inputs | Component Processes |  |  |  |  | Chemical Outputs |
|  |  | Intake Valve | Piston | Crankshaft | Spark Plug | Exhaust Valve |  |
| Intake |  |  |  |  |  |  |  |
| Compression |  |  |  |  |  |  |  |
| Ignition |  |  |  |  |  |  |  |
| Power |  |  |  |  |  |  |  |
| Exhaust |  |  |  |  |  |  |  |

Go to the following link on The Power Portal and watch the video on the four-stroke engine. http://www.thepowerportal.com/nA/English/PowerChannel/Foundations/FourCycleTheory.htm

Complete the events table by recording the inputs, processes, and outputs that occur during each of the five events of the four-stroke engine.

Part Three – Flowchart

Your teacher will assign you in a group of five to work on one of the five engine events.

- Develop a flowchart on a poster explaining what occurs during your assigned event in the engine. Include all inputs, processes, and engine components from Table 1 in your flowchart. Use the following shapes and colors to represent the various parts.

Green Circle – Inputs

Red Circle – Outputs

Squares – Processes (use different colors for each engine component)

Intake valve – Blue

Exhaust valve – Orange

Piston – Gray

Spark plug – Purple

Crankshaft - Yellow

Work with in your group to place the flow charts describing each event in order.

Use your flowchart to discuss the following analysis questions in your group. Record your answers  in your Engineering Notebook.

How many rotations does the crankshaft in a four-stroke engine complete for one cycle?

How could open valves during the compression stroke affect engine performance?

Which two events occur during the same stroke of the engine?

Conclusion

- What can a technician learn about an engine by reading a flowchart?

What are inputs, processes, and outputs in a small engine?

What other machines could be described using a flowchart? How?