Name_____________________________________________________

Activity 3.1.2 Engine Systems

Purpose

Engines consist of multiple systems that work together and repeat processes at a very high speed. An engine may be repeating cycles at a rate over 3000 times per minute. During Activity 3.1.1 Internal Combustion, you researched and described the major events occurring in a four-stoke cycle internal combustion engine. What systems in an engine are working together to complete those events quickly and efficiently?

In the Agricultural Power and Technology course, you learned about the different types of energy. Fuel in an engine has chemical potential energy. Once the fuel is ignited, kinetic energy is transferred throughout the engine in the forms of chemical, mechanical, electrical, and thermal energy.

Each system of an engine transfers energy to other systems. The fuel provides an energy charge to the engine. The compression system compresses the charge so it can easily ignite. The ignition system uses an electrical charge to ignite the fuel. The governor system manages the amount of fuel used by the engine. Lubrication and cooling systems remove the thermal energy from the engine to keep it cool.

How does energy flow in the systems of a small engine? How can engine components be parts of multiple systems?

Materials

| Per pair of students: Small engine Masking tape Fine tip permanent marker Engine System Cards Poster board Glue sticks Scissor | Per student: Agriscience Notebook Engineering Notebook Small Engine and Equipment Maintenance text Pen |
| --- | --- |

Procedure

Work with a partner to label and define components of a small engine in a table by identifying system inputs, processes, and outputs. Then make a flow chart showing the flow of energy throughout an engine. Record all tables and flowcharts in your Engineering Notebook.

Part One – Defining Systems

Start a table in your Engineering Notebook similar to Table 1 below. Make five rows for the ignition, compression, lubrication, fuel, and governor systems. Leave ample space between each row to record information.

| Table 1. Example Mechanical Systems Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| System(s) | Components | Process(s) | Energy Inputs | Energy Outputs |

Individually read Chapter 4, pages 44 – 62 of Small Engine and Equipment Maintenance. As you read, determine the system or systems each component listed below is a part of. List the components in the table.

| Intake valve Exhaust valve Piston Piston rings Crankcase* Muffler* Primer bulb* Head gasket | Carburetor* Air cleaner* Fuel tank* Spark plug* Armature Cooling fin* Cylinder head* Shroud* | Spark plug lead Magneto Throttle plate* Governor lever Throttle linkage* Valve springs Push rods Oil dipstick* | Timing gear Camshaft Lobes On/off switch* Oil sling Blower housing* Engine block* Rewind starter* |
| --- | --- | --- | --- |

Record the processes, energy inputs, and energy outputs for each system in the appropriate rows in the table.

Part Two – Part Identification

- Working with a partner, use a marker and masking tape to label components on the engine. Label the components marked with an (*) under Step 2.

Identify the system or system(s) each component labeled on the engine is a part of by placing one or more of the following letters on the label.

Ignition – I

Compression – C

Lubrication/Cooling – L

Fuel – F

Governor – G

Part Three – Systems Flowchart

Construct a flowchart showing the flow of potential, mechanical, electrical, and chemical energy between each system.

- Use scissors to cut out all Energy System Cards.

- Start a flow chart by placing the following system names on the poster board.

Fuel

Compression

Ignition

Lubrication and Cooling

Governor

Work with your partner to place the energy flow arrows to show where energy is flowing in and out of systems. Note that energy may flow between systems, into a system from an outside source, or out of the systems entirely.

Show your completed flow chart to your teacher once it is complete. Use a glue stick to paste the systems and arrows onto the poster board after teacher approval.

Record the flow chart in your Engineering Notebook

Conclusion

- How are components of different systems in an engine dependent upon each other?

What components of a small engine are part of multiple systems?

How could your flowchart be used to troubleshoot an engine?