Name_____________________________________________________

Activity 3.1.4 Systems Test

Purpose

You may see broken lawnmowers for free at the end of people’s driveways. Many times, those mowers can be repaired if the system problem can be identified. All systems in an engine must be in sync and operating correctly for an engine to function. What is the process for troubleshooting a system breakdown in an engine?

Engines require compression, spark, and fuel provided at the correct times to run. The compression of the charge provided by the fuel system must be timed with the spark from the ignition system for the engine to run. If there is a leak in the engine head, valve, or piston ring, there may be low compression in the engine cylinder. Without compression, the spark plug will not be able to ignite the charge. Over time, the spark plug may become corroded or damaged and not work correctly. Without a spark, the charge will not ignite, and the engine will not run.

What tools should you use to test the ignition and compression system of a small engine? What would the troubleshooting process for each system be?

Materials

| Per group of four students: Air supply Cylinder leakdown tester Leakdown clamp and 2″ bolt gauge Ignition tester 3/8″ drive 5/8″ spark plug socket 3/8″ drive 7/16″ socket 3/8″ drive 10mm socket Digital multimeter 3/8″ drive torque wrench | Per pair of students: Small engine 3/8″ drive socket wrench 2 C-clamps |
| --- | --- |
|  | Per student: Agriscience Notebook Engineering Notebook PPE Pen |

Procedure

Work with your partner to inspect and troubleshoot the compression and ignition systems of a small engine. All groups will first complete Part One. Then half of the classroom pairs will complete Part Two, while the other half of the class starts with Part Three.

Part One – Engine Preparation

Put on appropriate PPE.

Clamp the engine using two C-clamps.

Check the gas line to be sure it is OFF.

Disconnect the spark plug lead.

Remove the spark plug using the spark plug socket and socket wrench.

Part Two – Ignition Test

- A spark plug has two electrodes (one side and one center). Measure the gap between the two electrodes using a spark plug gauge.

Find the spacer that best fits between the electrode.

Record the size of the gap in your Engineering Notebook. Adjust the gap to 0.030″ if needed.

Why is a gap needed on the spark plug?

What could cause the spark plug to fail?

Use the multimeter to check the spark plug for the resistance and continuity. Use Figure 1 for identifying multimeter lead placement.

Figure 1. Spark Plug Anatomy

Resistance between the electrode terminal connection and end of center electrode.

Infinite resistance in the electrode would cause the spark plug to be faulty.

Continuity between the ground connection and ground electrode.

Continuity is needed to ground the electrode and cause a spark.

No continuity or infinite resistance between the center electrode and ground electrode.

Continuity means there is a short and a spark will not be produced.

Be sure the engine switch is in the OFF position.

Connect the spark plug wire to the long end of the ignition tester.

| Attach the ground clamp of the tester to a bolt on the compression head. Pull the recoil rope and observe the window on the ignition tester. Record your observations in your Engineering Notebook. Turn the engine switch to the ON position. Repeat Steps 7 – 9. Disconnect the oil module wires as seen in Figure 2. Check the oil module for continuity using the digital multimeter. Resistance in the module indicates it is working properly. Use a digital multimeter to check for continuity in the oil pressure switch. (See Figure 3) Record the resistance in your Engineering Notebook. Resistance indicates the spark would be grounded due to low oil. Repeat Steps 7 – 9. How do you know if your ignition system is working? Turn the switch to the OFF position. Go to Part Three if not completed. Reattach the spark plug and spark plug lead if you have completed Part Three. | Copyright Briggs and Stratton Corp. Used with permission |
| --- | --- |
|  | Figure 2. Oil Module |
|  | Copyright Briggs and Stratton Corp. Used with permission |
|  | Figure 3. Oil Pressure Switch |

Part Three – Compression Troubleshooting

| - Remove the air filter by removing the plastic nut and two metal wing nuts. - Disconnect the breather tube from the valve cover. - Position the piston of the engine at TDC by rotating the flywheel so the piston is closest to the head of the engine and the magneto of the flywheel is closest to the armature. - Place the leakdown clamp around the crankshaft and align the slot over a bolt hole in the crankcase as seen in Figure 4. Insert a 10mm bolt into the slot to hold the clamp in place. - Use a torque wrench and 7/16″ socket to torque the bolts on the leakdown clamp to 150 in-lbs. |  |
| --- | --- |
|  | Figure 4. Leakdown Clamp |
| Pull the regulator adjustment knob of the leakdown tester and turn the knob counterclockwise as far as it will go as seen in Figure 5. Check the air outlet valve on the leakdown tester to be sure it is closed. Attach the outlet hose of the leakdown tester to the spark plug hole on the engine head. Be sure the O-ring is seated to prevent air leak at spark plug hole. Connect the other end of outlet hose to the leakdown tester. Move your engine to an air source as instructed by your teacher. Attach the leakdown tester to the air supply and turn the air supply on. Turn regulator adjustment knob clockwise until the needle of the tester is on the set position. Push knob in to lock. Slowly open the air outlet valve and note position of needle on gauge. Listen for air leaks in the engine. What is the reading on the gauge? Where do you hear air leaks? |  |
|  | Figure 5. Leakdown Tester |

Record the position of the needle on the gauge in your Engineering Notebook.

What does the position of the needle tell you about the compression of the engine?

Slowly close the air outlet valve on the leakdown tester.

Shut off the main air supply.

Remove the leakdown tester and clamp.

Reattach the air filter and breather tube.

Use notes and flowcharts from Activity 3.1.1 Internal Combustion and Activity 3.1.2 Engine Systems in your Engineering Notebook to answer the following analysis questions.

How do air leaks affect engine compression?

Where could a leak be if air is flowing through the air filter?

Where could a leak be if air is flowing through the muffler?

Where could a leak be if air is flowing into the crankcase?

Go back to Part Two if not completed.

Reattach the spark plug and spark plug lead if you have completed Part Two.

Part Four – Technician Diagnostic

A lawnmower owner is unable to start his small engine. He knows the engine is getting fuel but does not know what else could be wrong. In your Engineering Notebook, record the troubleshooting procedure you would use to determine what the problem might be.

Conclusion

- How is a digital multimeter used for troubleshooting?

How can you use the leakdown and ignition test to troubleshoot system failures?

What is the process for troubleshooting an engine’s electrical and compression systems?