Name_____________________________________________________

Activity 3.1.5 SDS Protocols

Purpose

Small engines require oil for lubricant and gasoline as a fuel. Where can you access safety information about those chemicals?

Safety Data Sheets (SDS) are provided by manufacturers of nearly all chemical materials. There is even an SDS for water! SDS are designed to provide both workers and emergency personnel with the proper procedures for handling or working with a substance. SDS include information such as physical data, health effects, first aid, storage, disposal, protective equipment needed, and spill procedures. Engineers, technicians, and tradespersons use SDS if a spill or other accident occurs. SDS should be stored in an easily accessible area in the laboratory or workshop.

Gasoline used in engines is available in many forms containing alcohol in the form of ethanol. Alcohol in gasoline can be corrosive and damaging to small engine components. The alcohol content of gasoline in small engines should be less than 10%. Gasoline with unknown alcohol percentage should be tested before used.

How do you use SDS for chemicals found in a small engine?

Materials

| Per student: 2 SDS forms PPE Pen Agriscience Notebook Engineering Notebook Per class: Absorbent non-flammable material Gasoline (non-oxygenated) Liquid flammable disposal container | Per pair of students: Small engine 2 C-clamps Funnel Alcohol tester 250ml beaker Water bottle 30W oil Disposable shop towels |
| --- | --- |

Procedure

Read SDS forms to determine important safety and cleanup information about two chemicals. Use that information properly to add the chemicals to operate an engine.

Part One – SDS Review

Use one of the forms provided by your teacher to complete the information needed for Table 1 on Activity 3.1.5 Student Worksheet. Use the second form to complete the SDS review questions on the worksheet. Place Activity 3.1.5 Student Worksheet in the Safety section of your Agriscience Notebook.

Part Two – Engine Preparation

Fuel

Put on the appropriate PPE needed to work with oil and gasoline.

Place the engine, oil, and gasoline in a well-ventilated area in the shop or laboratory.

Pour approximately 200ml of gasoline into a 250ml beaker.

Remove the cap from the alcohol tester.

Fill the alcohol tester to the first line with water.

Use the beaker to fill the alcohol tester to the top with fuel.

Cap the tester and shake for 15 seconds.

Let the tester settle for 2 minutes.

Check the alcohol level of the gasoline and record in your Engineering Notebook.

Pour the gasoline from the tester into a disposal container.

Open the gas tank by removing the gas cap.

If the gasoline contains less than 10% alcohol, use a funnel to pour the remaining gasoline from the beaker into the fuel tank. Do not use the fuel and talk to your teacher if the fuel is above 10% alcohol.

Remove the funnel from the tank and seal the tank with the gas cap.

Clean the funnel and any spills following procedures listed in the SDS.

Oil

- Remove the dipstick and wipe with a clean cloth.

Insert the dipstick. Do not turn or tighten.

Remove the dipstick and check the oil level to see if it is at the top of the full indicator on the dipstick.

If oil level is low, open the engine oil fill cap.

Using the funnel, slowly pour the oil into the engine oil fill to point of overflowing. Do not overfill by tipping the engine.

Check the oil level with the dip stick.

Clean the funnel and any spills following procedures listed in the SDS. Properly dispose of all flammable rags.

Conclusion

- When should you consult a SDS when working in the laboratory or workshop?

What differences did you observe in the spill clean-up procedures for each substance?

How is an SDS used for practicing first aid?

Why are SDS forms organized in specific way?

Name_____________________________________________________

Activity 3.1.5 Student Worksheet

| Table 1. SDS Review Form |  |
| --- | --- |
| SDS Form 1 |  |
| Name of Material |  |
| Use(s) of Material |  |
| First Aid Measures |  |
| Eye Contact |  |
| Skin Contact |  |
| Inhalation |  |
| Ingestion |  |
| Fire and Explosion Hazards |  |
| Flammability of Product |  |
| Fire Fighting Media |  |
| Precautions for Safe Handling and Use |  |
| Handling a Spill |  |
| Precautions for Handling and Storage |  |
| Personal Protective Equipment Needed |  |
| Physical and Chemical Properties |  |
| Physical State and Appearance |  |
| Odor |  |
| Color |  |
| pH |  |
| Boiling Point |  |
| Melting Point |  |

SDS Analysis Questions

What is the name of this product?

What should one do if this chemical is spilled on skin?

Should you use this chemical if you are wearing contacts?

If this chemical is swallowed, what organs will most likely be damaged?

What is a hazard associated with this chemical?

Is this substance flammable?

Where and how should this product be stored?

What type of protective equipment is need when handling this product?

How should this product be disposed?

If this material spills what can be used to absorb it?