Name_____________________________________________________

Activity 3.1.6 Engine Energy

Purpose

Even though a small engine produces a large amount of power compared to its size, an engine loses 70% of its energy as thermal energy. This inefficiency means energy is lost or wasted.

When the fuel charge of an engine combusts, thermal energy is released during the chemical reaction. The thermal energy not converted to mechanical energy throughout the engine needs to be transferred to the air. If the thermal energy is not transferred, the engine may overheat. In addition, moving components have friction between them, causing additional thermal energy loss.

Thermal energy is transferred through conduction, convection, and radiation. Conduction of thermal energy occurs when two objects are in direct contact with each other. A pan on a stove conducts heat directly from the stove to the entire pan. Convection occurs when thermal energy is transferred through a liquid or gas. Water boiling in pan is an example of convection. Radiation is thermal energy transfer without a material carrier. If you place your hand over a hot stove top, you can feel radiated heat coming from the stove.

A small engine conducts thermal energy by conduction, convection, and radiation. Where does each process occur? How is an engine designed to transfer thermal energy?

Materials

| Per pair of students: Small engine 10mm wrench 2 C-clamps Disposable shop towels LabQuest2 Temperature sensor IR thermometer Oil storage bottle Funnel Baster Per class: Gas storage tank | Per student: Agriscience Notebook Engineering Notebook Employability Evaluation Rubric PPE |
| --- | --- |

Procedure

Measure the current temperature of engine components. Then follow operation procedures to run an engine and measure the operating temperature of components. Your teacher will assess your work in the shop using the Employability Evaluation Rubric.

Part One – Thermal Measurements

Turn on the LabQuest2 and plug in the IR thermometer into CH1 and the temperature sensor into CH2.

Tap on Channel 1 on the meter screen and change the units to °C.

Tap on Channel 2 on the meter screen and change the units to °C.

In your Engineering Notebook, make a table for recording the pre-run and post-run temperature of the following engine components.

Cylinder head

Muffler

Valve cover

Crankcase

Blower housing

Oil

Press the infrared button on the IR sensor. CAUTION: The laser could cause serious injury to your eyes. DO NOT point the laser at anyone or look directly into the laser.

Adjust the distance of the sensor from the cylinder head so a circle approximately 1″ in diameter projects on the center.

Record the temperature of the cylinder head in your Engineering Notebook.

Repeat Steps 6 – 7 for the muffler, valve cover, crankcase, and blower housing.

Press the infrared button on the IR sensor to shut it off.

Open the oil fill cap.

Place the tip of the temperature sensor in the oil.

Wait for the temperature reading to stabilize.

Record the temperature of the oil in your Engineering Notebook.

Close the oil fill cap.

Part Two – Engine Operation

Startup

- Disconnect the spark plug wire from the spark plug.

Place the throttle lever in the STOP position.

Make sure the fuel valve to the left of the speed control panel is in the OFF position.

Remove the dipstick and wipe with a clean cloth.

Insert the dipstick. Do not turn or tighten.

Remove the dipstick and check the oil level. It should be at the top of the full indicator on the dipstick.

Add oil if needed, use a funnel and pour the oil slowly into the engine oil fill. Fill to point of overflowing. Do not over fill by tipping the engine.

Go to the running station and secure the engine to the table using two C-clamps.

Position the C-clamps next to the drain plugs on each side of the crankcase base.

Point the muffler to the air handling vent.

Wait for your teacher to check the engine set up.

Put on ear protection.

Turn the fuel valve in front of the gas tank to the ON position.

Set the choke on the carburetor to the CHOKE position.

Connect the spark plug wire to the spark plug and move the throttle lever to the RUN position.

Turn the switch to the ON position.

Ask your teacher for permission to start the engine.

Once you receive permission, pull on the rope starter, gently and evenly. The engine should start with a few pulls of the rope starter.

When the engine is running, move the choke to the RUN position.

Let the engine run for a minimum of two minutes so the engine gets to operating temperature.

Shut down

- With the engine running, turn the fuel valve to the OFF position.

Allow the engine to run until it is out of the fuel in the carburetor and stops running.

Move the throttle lever to the STOP position.

Turn the switch to the OFF position

Remove ear protection.

Part Three – Thermal Energy

- Press the infrared button on the IR sensor. CAUTION: The laser could cause serious injury to your eyes. DO NOT point the laser at anyone or look directly into the laser.

Adjust the distance of the sensor from the cylinder head so a circle approximately 1″ in diameter projects on the center.

Record the temperature of the cylinder head in your Engineering Notebook.

Repeat Steps 2 – 3 for the muffler, valve cover, crankcase, and blower housing.

Press the infrared button on the IR sensor to shut it off.

Open the oil fill cap.

Place the tip of the temperature sensor in the oil.

Wait for the temperature reading to stabilize.

Record the temperature of the oil in your Engineering Notebook.

Close the oil fill cap.

Remove the C-clamps.

Lift the engine with your hands under the carburetor and gas tank.

Caution: Do not touch the hot muffler.

Carry the engine to your work area.

Place a funnel in an empty oil bottle.

Use the 10mm wrench to remove the oil drain plug.

Drain the oil into the oil bottle using the funnel.

Replace the plug using the 10mm wrench after the draining the engine of oil.

Cap the oil bottle and place in proper storage as instructed by your teacher.

Using a baster, remove the fuel from the tank and place it in a fuel storage container.

Open the fuel valve at the carburetor using a 10mm wrench and drain the residual fuel in the carburetor into a fuel storage container.

Clean up your work area and return your engine to your storage area.

Record answers to analysis questions in your Engineering Notebook.

How is the oil transferring thermal energy?

Where does the thermal energy from the oil get transferred too?

Where is air movement used to transfer energy?

Which areas of the engine that you measured were the hottest? Why do believe so?

After the engine is shut down, how is the thermal energy from the engine transferred?

Conclusion

- Where in an engine is thermal energy transferred using convection?

How is an engine designed to transfer thermal energy?

What would happen to an engine if thermal energy was not transferred?

How could an engine be designed to transfer thermal energy more efficiently?