Name_____________________________________________________

Activity 3.2.1 Sequential Order

Purpose

Have you ever taken something apart and put it back together? What made the task difficult? Did you have any parts left over? How could you make the process easier?

Machines are comprised of components precisely manufactured and fastened together. If they are not fastened together in the correct order, the machine may not function.

Engineers and technicians troubleshoot, maintain, and reverse engineer machines by disassembling them and putting them back together. They record steps taken, tools used, and specifications about each component in their Engineering Notebook. In addition, they sketch or photograph parts that may be confusing to put back together. For example, an engine has gaskets that help seal fastened components. The gaskets must be installed properly to function. Sketches and photos of the gaskets on the parts can help in the assembly process.

Technicians organize and label components while taking them apart to make the assembly process easier. Many machines have repair or technician manuals with specifications for each component and detailed pictures or photos noting important disassembly or assembly instructions. Specifications include torque information. Torque describes the force placed on each fastener when reassembling the engine. Technicians use the manuals as a reference during the disassembly and assembly process.

How will you use an Engineering Notebook and repair manual to document the teardown of a small engine?

Materials

| Per pair of students: Small engine Storage tote Briggs and Stratton OHV repair manual Box of sealable plastic bags Permanent marker 3/8″ drive socket wrench 3/8″ drive 13mm socket 3/8″ drive 12mm socket 3/8″ drive 5/8″ spark plug socket 3/8″ drive 8mm socket 3/8″ drive 10mm socket Pliers Fuel line pry tool Optional: digital camera | Per student: Agriscience Notebook Engineering Notebook Pen PPE |
| --- | --- |

Procedure

Start the process of disassembling a small engine. Document your process in your Engineering Notebook.

Use a table with columns similar to Table 1 to record the disassembly process. Include tasks, tools used, torque and measurement specifications. Record the section, page, and figure number from the repair manual related to each step as you disassemble the engine when applicable. Answer analysis questions related to each applicable step in your Engineering Notebook.

| Table 1. Example Engineering Notes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Step | Hardware | Tools | Torque or Specification | Manual Location |  |  |
|  |  |  |  | Sec. | Page | Fig. |

- Put on proper PPE and tie back hair.

- Remove connection to spark plug.

- Use your fingers to loosen and remove the plastic nut and one metal wing-nut from the air cleaner cover and air filter element.

Place the plastic nut and one metal wing-nut in a sealed plastic bag.

Label the plastic bag with the type and number of items.

Ex. “Air cleaner hardware – 1 plastic nut and 2 “wing nuts”.

Remove and clean the air filter by tapping on a hard service to remove dirt and debris.

Inspect the filter for tears and replace the filter if tears are present.

Store air cleaner hardware and filter in storage tote.

Find the section, page, and figure number in the engine manual for the air filter.

Note: Record the section, page, and figure number for this step and each other applicable step as you disassemble the engine.

Remove the speed control lever knob and use an 8mm wrench to remove the front engine decorative panel.

Place the plastic knob and three screws in a sealed plastic bag.

Label the plastic bag to describe the contents.

Use a 13mm socket to remove the muffler.

Find the moisture drain and deflectors on the muffler.

Why would the muffler be designed to prevent moisture build up?

Remove the spark plug using a 5/8″ spark plug socket.

Remove the air filter base using an 8mm socket for the single screw and a 10mm socket for the two nuts.

Pull the breather line from base before removing hardware

Bag and label the hardware

Position fuel valve to OFF and choke to OPEN

Note the position of the gaskets and describe or sketch in your Engineering Notebook

Inspect gaskets for tears and replace if tears are present

Remove the fuel line from the fuel tank using pliers and fuel line pry tool.

Remove the fuel tank using an 8mm socket and 12mm socket.

What is the purpose of the filter on the fuel line?

Look inside the tank, what features in the tank prevent the gas from splashing when filling it?

Inspect the linkage connecting the governor to the carburetor.

Sketch a picture of the linkage for reference when assembling. Take a picture if possible.

Remove the carburetor by hand.

Slide carburetor along studs to position the throttle shaft so you can lift the governor link up and away from the throttle shaft.

The link end at the carburetor is a “L” bend and at the governor arm, is a “Z” bend.

Sketch and label in your Engineering Notebook.

Detach link spring from throttle shaft

Slide the carburetor from the mounting studs

Slide the plastic isolator block and gaskets off

Record and sketch the orientation of gaskets

Store the carburetor pieces in a plastic bag

Place plastic bags and engine components in a storage tote. Clean area and materials as instructed by your teacher.

Conclusion

- What information should you record when disassembling a machine?

Why is it important to document each step of the disassembly process?

How will pictures, sketches, and references to manuals help a technician reassemble a machine?

Why should machine hardware and components be labeled to during the disassembly process?