Name_____________________________________________________

Activity 3.2.3 System Controls

Purpose

Have you ever mowed a lawn with a lawn mower? What lawn mower controls keep you safe? How does the lawn mower know when to increase and decrease its power based upon the thickness of the grass? Systems within a machine are designed to control or manage how it operates. A small engine in a lawn mower has electrical and mechanical controls working while you are mowing a lawn.

You learned about electrical circuits and switches in Agricultural Power and Technology. In a small engine, a single pole electrical switch in the closed position may shut the engine off by directing the electrical energy to a ground connected to the engine. When the electricity goes to the ground, the spark plug does not generate a spark and the engine stops. Additional emergency control switches have been added to small engines to prevent them from running with low oil or when the operator in an unsafe position.

Technicians test electrical systems for resistance or continuity. If a switch on the engine is in the off position, there should be little to no resistance between the switch and the ground in the circuit, which means electrical current is conducted to the ground and no spark will occur. When the switch is in the on position, there should be infinite resistance between the switch and the ground. This indicates that no electrical current can be passed to the ground and a spark can occur.

Mechanical levers are found throughout machines to transfer and control the flow of energy. A governor system in a small engine controls the speed, power, and torque. When the engine is not running, the governor spring is opening the throttle plate of the carburetor. Once the engine starts, the governor lever starts to close the throttle plate, so the engine does not run at too high of a rpm. When the engine is placed under a load such as mowing a lawn, the governor spring opens the throttle plate of the carburetor, increasing fuel flow, which prevents the engine from slowing down.

The governor system has springs and linkages connected at different locations. A technician can tune the engine for different applications by changing the location of the springs to increase or decrease tension. The tension in the springs will determine the operating speed of the engine.

How are the electrical and mechanical controls in a small engine designed? How do you test and adjust those controls?

Materials

| Per pair of students: Small engine Storage tote Briggs and Stratton OHV repair manual Permanent marker Box of plastic bags 3/8″ drive socket wrench 3/8″ drive 10mm socket 3/8″ drive 8mm socket Digital multimeter Feeler gauge Optional: digital camera | Per student: Engineering Notebook Agriscience Notebook PPE Pen |
| --- | --- |

Procedure

Work with a partner to tear down and draw a schematic of governor and ignition systems found in a small engine. Record tasks, tools used, torque and measurement specifications, and the section, page, and figure number from the service manual related to each task when applicable. Use a table similar to the one used during Activity 3.2.1 Sequential Order to record your teardown procedures. Answer analysis questions related to each step in your Engineering Notebook.

Part One – Governor System

Put on PPE and tie back long hair.

Move the governor arm back and forth.

What controls the governor arm?

What does the governor arm control?

How can the governor arm be adjusted?

Make a sketch and/or take a picture of the governor mechanism.

Note the holes in the linkage and where springs are installed.

Mark the holes with where the spring is set with a permanent marker.

Note the direction of the springs.

In which holes do the spring loops face up?

In which holes do the spring loops face down?

Use a 10mm socket to loosen the nylon locknut on the governor arm.

Remove the governor arm and pinch bolt.

Remove the governor spring.

Bag and label the arm and pinch bolt.

Use an 8mm socket to remove the two screws holding the speed bracket control.

Note and sketch/photograph the location of the red ground and two black electrical wires.

Use a permanent marker to mark and label screws if they are different sizes.

Bag and label the speed bracket control and screws.

Answer analysis questions.

What is the purpose of the governor spring?

What would happen if the spring tension was loosened or tightened?

How does the governor system maintain a constant speed?

Part Two – Ignition System

- Remove the rewind starter assembly using an 8mm socket.

Bag and label the three 8mm screws.

- Remove the blower housing using an 8mm socket.

Bag and label the four 8mm screws.

- Observe the ignition system on the engine and complete a schematic in your Engineering Notebook. Use the symbols shown in Figure 1.

| Armature | Ground | Magneto |  |
| --- | --- | --- | --- |
| Oil module | Oil switch | Switch | Spark plug |
| Figure 1. Electrical Symbols |  |  |  |

Turn the switch to the on position.

Prepare the multimeter for testing resistance.

Rotate selector to (Ohms) position.

Insert RED test lead into the V Ω receptacle in the meter.

Insert BLACK test lead into COM receptacle.

Test the switch for resistance.

Note the numbers by each terminal on the switch.

Touch the red test lead to terminal one and the black test lead to terminal two.

Record the resistance in your Engineering Notebook.

Touch the red test lead to terminal one and the black test lead to terminal three.

Record the resistance in your Engineering Notebook.

Repeat Step 7 with the switch in the off position.

Is the switch working correctly? How do you know?

- Disconnect the wiring from the switch.

Use a permanent marker to record the number on the wire to match the terminal it connects.

Sketch and/or photograph the wire routing near the governor shaft.

Disconnect the wire from the oil pressure sensor on the crankcase cover.

Use a feeler gauge to measure the air gap between the flywheel and armature.

Record the gap in your Engineering Notebook.

Record the pages in the small engine technical manual referring to the air gap.

Is the air gap correct?

How can the air gap affect how the engine functions?

Use an 8mm socket to remove the armature.

Bag and label the armature and two screws.

Find the magneto on the flywheel.

Rotate the flywheel so the magneto is directly under the armature connections.

What is the position of the piston?

Why is the relationship between piston position and magneto position important?

Remove the finger guard with the oil module using an 8mm socket.

Bag and label the speed bracket control and screws.

Why are the wires on the module different colors?

Store your engine and materials in a storage tote as instructed by your teacher.

Conclusion

- What is the purpose of controls in a machine?

How can a control prevent a machine from being damaged?

How do electrical switches control an engine?

How can controls, such as a governor, be adjusted for the type of work an engine will be doing?