Name_____________________________________________________

Activity 3.2.4 Machine Specifications

Purpose

In Activity 3.2.3 System Controls, you measured the air gap between the armature and the magneto. This is a specification that must be set precisely for the engine to produce a spark. Machines are assembled to specifications including gaps, clearances, and fastener torque. What are the specifications of your engine?

Bolts throughout an engine are tightened to a certain torque. If bolts are over-torqued, threads or the bolt may break, or the components may not move correctly. The torque on small engine bolts is measured in inch-pounds or foot-pounds. Valve clearance is another specification a technician must follow. Since the valve clearance has a range of acceptable values, it is a tolerance. The valve clearance between the rocker arm and valve stem controls how far the exhaust and intake valves open. Technicians measure the valve clearance to the nearest thousandth of an inch using a feeler gauge.

What are the specifications for the valves and engine head of a small engine? What happens when the specifications are not met?

Materials

| Per pair of students: Small engine Storage tote Briggs and Stratton OHV repair manual Box of sealable plastic bags Permanent marker 3/8″ drive socket wrench 3/8″ drive 8mm socket 3/8″ drive socket extension 14mm wrench 10mm wrench Feeler gauge 3/8″ drive 12mm socket Pencil | Per student: Engineering Notebook Agriscience Notebook PPE Pen |
| --- | --- |

Procedure

Work with your partner to practice setting the tolerances for the valves on a small engine. Then remove the valves and record the placement and torque of bolts removed. Record the task, tools used, torque and measurement specifications, and the section, page, and figure number from the repair manual related to each task when applicable. Use a table similar to one used in previous activities to record your teardown procedures. Answer analysis questions related to each task in your Engineering Notebook.

Remove the cylinder heat shield using an 8mm socket.

Remove the valve cover using an 8mm socket.

Label and store the four bolts.

Observe the breather valve inside the case designed to release gases.

Caution: Valve is calibrated, do not remove.

The valve cover is fastened at specific torque. Use the repair manual to find the torque of the valve cover bolts and record in your Engineering Notebook.

Turn the flywheel clockwise to observe the movement of the valves and position of the flywheel.

Which components are controlling the valves?

Sketch and identify each valve as intake or exhaust.

How are the intake and exhaust valves different?

Use your repair manual to find the allowable clearance for the exhaust and intake valves. Record the clearance specifications in your Engineering Notebook.

| Turn the flywheel until the magneto is below where the armature attaches, and both valves are closed. The piston should be at TDC. Use a pencil as a depth gauge. Place the pencil in the spark plug hole. Mark the pencil where it aligns with the spark plug hole. Remove the pencil, measure 1/4″ up, place a second mark. Place the pencil back in the spark plug hole and turn the flywheel until the second mark is aligned with the hole. | Copyright Briggs and Stratton Corp. Used with permission |  |  |
| --- | --- | --- | --- |
|  | Figure 1. Feeler Gauge |  |  |
| Use a feeler gauge to measure the valve clearance between each rocker arm and valve stem as seen in Figure 1. Record the clearances for the intake and exhaust valve in your Engineering Notebook. What is the current stroke of the engine? Are the clearances for each valve in the specification range you found during Step 4? On the exhaust valve, use a 14mm wrench to hold the adjusting nut while using the 10mm wrench to loosen the jam nut multiple turns. See Figure 2. The adjusting nut (14mm) sets the valve clearance. The jam (10mm) nut holds the adjusting nut in place. Mark the adjusting nut for the exhaust valve with a marker. Loosen the adjusting nut for the exhaust valve (14mm) two turns. Use the mark to count the turns. Repeat Steps 8 – 10 for the intake valve. Slowly rotate the flywheel through an entire cycle (two revolutions). Is the current valve clearance above or below the specified range? How will the changed valve clearance affect the engines operation? What would happen to the engine if the adjusting nut was too tight instead of too loose? |  | Copyright Briggs and Stratton Corp. Used with permission |
|  |  | Figure 2. Rocker Arm and Nut |

Reset both valves to the proper clearance using the following procedure.

Turn the flywheel until the magneto is below where the armature attaches and both valves are closed. The piston should be just past TDC (use the marked pencil for setting the piston).

Place the feeler gauge with the correct measurement between the rocker arm and valve stem.

Tighten the adjusting nut until the rocker arm and valve stem are touching the feeler gauge. The feeler gauge should still be free to move between the rocker arm and valve stem.

Use a 14mm wrench to hold the adjusting nut in place and tighten the jam nut with a 10mm wrench.

Turn the flywheel and check the valves to be sure they are operating correctly.

Have your teacher check your valve settings.

Loosen jam nut and adjusting nut and repeat Step 11 if settings are incorrect.

Repeat Steps 8 – 11 to loosen the rocker arms.

Turn the rocker arms to the side and remove the push rods.

Use a 13mm socket to remove the cylinder head and gasket.

Number each bolt with a permanent marker.

Remove each bolt and alignment collars, and note their locations. They should be placed back in the same hole during reassembly.

Trace the gasket in your Engineering Notebook and record where each numbered bolt will go during reassembly.

Use your technical manual to find the torque specification for the head bolts and record the specification in your Engineering Notebook.

Place cylinder head, bolts and gasket in plastic storage tote as instructed by your teacher.

Conclusion

- What will happen if a machine is not assembled using correct specifications?

- Where can machine specifications be found?

- How is a specification different from a tolerance?