Name_____________________________________________________

Activity 3.2.5 Functional Components

Purpose

Components of a machine are designed to do a specific job. One job of an engineer is to design machine components to work efficiently and aid the technician when assembling the machine. What design choices can you find in a small engine?

You explored gear ratios and metal properties during Agricultural Power and Technology. Gear ratios affect the speed and torque of a machine. The type of metal chosen for a component affects its durability and ability to transfer heat. A small engine has multiple gears meshed together to cycle the engine through each event at the correct time. Small engines contain both ferrous and nonferrous metals. Nonferrous metals, such as aluminum and zinc alloys, are used where parts need to be light weight and conduct heat. Heavier ferrous metals, such as cast iron, are used in situations where a harder, more durable metal is needed. Ferrous metals are distinguishable from nonferrous metals because of their magnetic properties. Magnets attract ferrous metals.

The shape and placement of machine components are important as well. The rings in a piston are placed in a specific order, each designed for a specific job. The compression ring seals the combustion chamber between the piston and the cylinder bore. The wiper ring and oil ring remove excess oil from the cylinder wall. All rings are marked and have a specified shape.

What types of metal and markings are found in a small engine? How is a small engine designed to function?

Materials

| Per pair of students: Small engine Storage tote Briggs and Stratton OHV repair manual Box of sealable plastic bags Permanent marker Fly wheel strap 8-inch gear puller 3/8″ drive socket wrench ½″ drive breaker bar ½″ drive 21mm deep well impact socket 3/8″ drive 8mm socket 3/8″ drive 10mm socket 3/8″ drive 17mm socket Magnet Masking tape Roll of shop towels | Per student: Engineering Notebook Agriscience Notebook Employability Evaluation Rubric PPE Pen |
| --- | --- |
|  | Per class: Ring expander |

Procedure

Work with your partner to remove the internal components of a small engine. Record the tasks, tools used, torque and measurement specifications, and the section, page, and figure number from the repair manual related to each step when applicable. Answer analysis questions related to each step in your Engineering Notebook. Your work in the shop will be evaluated using the Employability Evaluation Rubric.

| Rotate the flywheel and observe the motion of the piston. Note the notch on the top of the piston. Where is the notch pointing? Why do you believe the notch is there? Remove the flywheel guard using an 8mm socket. Use a flywheel strap, breaker bar, and 21mm socket to remove the flywheel nut. Attach the flywheel strap as seen in Figure 1. Place the 21mm socket over the flywheel nut. Push the breaker bar down in the counterclockwise direction until the flywheel nut breaks free. Remove the loose flywheel nut by hand. Use the gear puller to remove the flywheel as seen in Figure 2. Place the two jaws around the flywheel. Do not place a jaw on the magneto. Place one jaw between the arms of the armature bracket to reduce circular movement. Tighten the center bolt of the flywheel puller against the crankshaft. Continue to tighten using a 17mm socket until flywheel comes loose. Store the flywheel key in a plastic bag. Label the bag. Remove cylinder air guide using an 8mm socket. Label and store the two screws. | Copyright Briggs and Stratton Corp. Used with permission |
| --- | --- |
|  | Figure 1. Flywheel Strap |
|  | Copyright Briggs and Stratton Corp. Used with permission |
|  | Figure 2. Gear Puller |

| Remove the crankcase cover using a 10mm socket. Use a permanent marker and masking tape to number bolts before removal. Use Figure 3 as a guide for numbering the bolts. In your Engineering Notebook, sketch and record the location of each numbered bolt on the crankcase. Store and label the bolts. Remove the crankcase by hand. Do not pry. Carefully remove the gasket. If it tears, replace it. Place the flywheel nut back on the crankshaft to protect the threads. Place the gasket in between pages in the repair manual to avoid damage. | Copyright Briggs and Stratton Corp. Used with permission |
| --- | --- |
|  | Figure 3. Crankcase Bolts |

| Use shop towels to wipe clear any oil left in the engine crankcase. Use a shop towel to turn the crankshaft so the timing marks are aligned on the crankgear and cam gear as seen in Figure 4. Caution: flywheel key slot can be sharp. Count the number of teeth on the cam gear and crankgear. Calculate the gear ratio and record in your Engineering Notebook. Gear ratio = Number teeth on cam gearNumber teeth on crankgear How does the gear ratio relate to the number strokes in one cycle of the engine? Why is the gear ratio important? What would happen if the gears were not aligned with the timing marks? | Copyright Briggs and Stratton Corp. Used with permission |
| --- | --- |
|  | Figure 4. Timing Marks |

Use your hand to remove cam gear. Caution: gears may be sharp.

Lobes on camshaft are holding the tappets in place. The tappets will fall out once the cam gear and shaft are removed.

Store and label tappets.

Use a 10mm socket to remove the two bolts from the rod cap.

Sketch the cap and rod with the alignment marks.

Bag and label rod cap and bolts.

Remove the piston and crankshaft from the engine by hand.

Examine the rings on the piston.

Find the mark on the end of each ring.

Rings must be installed with marks facing the correct direction. Record which way the mark is facing.

Sketch the cross-section design of each ring.

Do not remove the rings.

What do you believe is the function of each ring?

Make a table in your Engineering Notebook similar to Table 1.

| Table 1. Components |  |  |
| --- | --- | --- |
| Component | Metal | Purpose |
| Piston |  |  |
| Piston ring |  |  |
| Crankshaft |  |  |
| Crankcase |  |  |
| Cylinder wall |  |  |
| Cooling fin |  |  |

Use a magnet to determine if each component is made of ferrous or nonferrous metals. Record your observations in the table in your Engineering Notebook.

Based upon your knowledge of ferrous and nonferrous metals, record why you believe each component is made of the metal in the purpose of column of the table.

Use a rag to clean all components coated with oil.

Clean your workstation and store your engine and parts in a plastic tote as instructed by your teacher.

Conclusion

- How does the gear ratio affect when each event occurs in the small engine?

- Why do the metals used for each engine component vary?

- How are detailed markings on components in machines used by technicians?