Name_____________________________________________________

Project 3.2.2 Fuel Efficiency

Purpose

Machines can be continually improved to become more efficient. New environmental regulations require internal combustion engines to be more fuel efficient to reduce fuel consumption and harmful emissions. What component in an engine regulates fuel usage and efficiency?

A carburetor is a passageway for fuel to move from the fuel tank to the combustion chamber of a small engine. Modern technologies and engineering ideas have improved carburetors. Improvements have been done to improve engine efficiency, generate more power, and use the optimum amount of fuel. Carburetors must maintain an optimum air-fuel ratio of 14.7 parts air to one part fuel (14.7:1). If the air-fuel ratio is too lean or too rich, the engine will not operate correctly. Lean mixtures can cause the engine to overheat. Rich mixtures may reduce the mixture’s explosiveness.

Each component in a carburetor improve the consumption of fuel at the optimum air-fuel ratio. Some of these components use Bernoulli’s principle to mix the air and fuel. You used Bernoulli’s principle in Agricultural Power and Technology to power a windmill. Bernoulli’s principle states that as the air velocity increases, the air pressure decreases.

Where is Bernoulli’s principle used in a carburetor of a small engine? How do components of a carburetor improve its efficiency?

Materials

| Per pair of students: Carburetor from small engine 2 60ml syringes 5 3″ lengths of ¼″OD. x 1/8″I.D. tubing 2 plastic 1/8″ Tees ½ cup plastic bowl with lid Blue food coloring LabQuest2 Vernier pressure sensor 10ml graduated cylinder Per class: Hand power drills Drill bits Hacksaws Hot glue guns Hot glue sticks Syringes of various sizes Duct tape Tubing Plastic tees Toothpicks Coins | Per student: Engineering Notebook Agriscience Notebook Engineering Report Template Engineering Evaluation Rubric Pen PPE |
| --- | --- |

Procedure

Work with a partner to identify the major components of a carburetor and construct and test a carburetor prototype. Then use the engineering design process to improve the prototypes fuel efficiency.

Part One – Identification

- Observe the carburetor removed from the engine and find the following parts described during the presentation: Carburetor Theory: A simple Approach to a Complex Problem.

Bowl

Venturi

Emulsion tube

Bowl vent

Throttle plate

High speed adjustment

Idle passage

Choke plate

- Sketch and label the parts in your Engineering Notebook.

- In your Engineering Notebook, explain how each part contributes to fuel usage and engine operation.

Part Two – Carburetor Prototype

| - Put on PPE. - Attach the four 3″ tubing pieces, two plastic tees, and two 60ml syringe cylinders together as shown in Figure 1. - Push the piston of the 60ml syringe all the way into the cylinder on the right. - Fill the plastic bowl with water and add 3 drops of blue food coloring. - Seal the lid on the plastic bowl. |  |
| --- | --- |
|  | Figure 1. Carburetor Tubing |

Slide the downward-facing three-inch tube through the hole in the lid so the tube is within ½″ from the bottom of the bowl.

Sketch your carburetor model in your Engineering Notebook and label the following parts.

Bowl

Emulsion tube

Venturi

Piston side of carburetor

Air cleaner side of carburetor

| Setup the LabQuest2 and pressure sensor. Turn on the pressure sensor. Place the pressure sensor plug into Channel 1. Use the last piece of ¼″ tubing to attach the pressure sensor to the second T as seen in Figure 2. Tap on Rate and change to 4 samples per second. Tap on OK. |  |
| --- | --- |
|  | Figure 2. Carburetor and Sensor |

Record the starting pressure in the venturi in your Engineering Notebook.

How does the starting air pressure relate to the optimal air-fuel ratio?

Hold the bowl and pressure sensor in place.

Press the Start button on the LabQuest2 to begin collecting data.

Have one partner pull the piston to the 60ml mark on the cylinder while the other observes the pressure reading on the LabQuest2.

Press the Stop button on the LabQuest2.

Examine the graph on the LabQuest2 and find the lowest air pressure reading.

In your Engineering Notebook, record the lowest air pressure reading as the intake pressure in the venturi.

Disconnect the syringe cylinder and piston.

Expel the water in the 60ml syringe into a 10ml graduated cylinder.

Record the volume of water in your Engineering Notebook.

Find the volume of air by subtracting the water volume from the total volume of 60ml. Record the air volume in your Engineering Notebook.

Pour the water back into the prototype bowl.

Calculate the air-fuel ratio by dividing the air volume by the volume of water.

Air fuel ratio = Air volumeWater volume

Record the air-fuel ratio in your Engineering Notebook.

What happens to the air-pressure in the venturi when the piston is pulled?

Would the engine run with the current air-fuel mixture generated by the prototype?

Which components could you add to your prototype to increase the air-fuel mixture?

Part Three – Fuel Efficiency

Use what you learned about carburetor components to modify your prototype to produce an air fuel mixture of 14.7:1. Record materials, procedures, data, and sketches of your modifications in your Engineering Notebook. Once your prototype produces a 14.7:1 air-fuel mixture, work individually to complete an engineering report using the Engineering Report Template. Your report will be assessed using the Engineering Evaluation Rubric.

Conclusion

- How can system components improve the efficiency of a machine?

How are components modified to improve efficiency?

How is the engineering process used to improve the functionality of a machine component?