Name_____________________________________________________

Activity 3.3.3 Torque and Sequence

Technicians tighten components to a specific torque when reassembling engine components. What would happen if they are torqued too much or too little? Why is torque important when assembling a machine?

Many fastened parts require a specific torque and order to ensure a proper seal. Torque is a force acting on a perpendicular radial distance from a point of rotation. Torque wrenches precisely tighten fasteners to the correct pound-inch or pound-foot. For example, a person using a one-foot wrench and applying a force of 10 pounds of force is placing 10 ft-lb of torque on the nut. If parts are torqued too much, they may break or the space for lubrication may be limited. If torqued too little, parts may come loose. The sequence of fastening is just as important as torque. When a large component is fastened with multiple fasteners, a sequential pattern is used to ensure a tight seal with no gaps.

Where are torque and sequence used in a small engine?

Materials

| Per pair of students: Disassembled small engine Briggs and Stratton OHV repair manual New crankcase gasket (if needed) New head gasket (if needed) Bottle of 30W oil 3/8″ drive in-lb torque wrench 3/8″ drive 8mm socket 3/8″ drive 10mm socket 3/8″ drive 12mm deep well socket 3/8″ drive 13mm socket 3/8″ drive 5/8″ spark plug socket | 14mm wrench 10mm wrench Briggs and Stratton ruler Feeler gauge ½″ drive ft-lb torque wrench ½″ drive 21mm deep well impact socket Flywheel strap |
| --- | --- |
| Per student: Agriscience Notebook Engineering Notebook | PPE Pen |

Procedure

Work with your partner to reassemble internal components and valves of a small engine. Be sure to install components in the correct order with the specified torque. Use your Engineering Notebook as a reference during the reassembly process. Record answers to analysis questions in your Engineering Notebook.

Part One – Rod and Rod Cap

| Line the piston rod bearing with oil. Fasten the piston rod and rod cap around the crankpin journal using a torque wrench and 8mm socket. Align marks as seen in Figure 1. What type of metal is the rod and rod cap? What are the characteristics of that metal? What will happen to the metal if it is over-torqued by the bolts? How could over-torqueing affect the movement of the piston? Torque bolts to 125in-lb. Do not over-torque. | Copyright Briggs and Stratton Corp. Used with permission |
| --- | --- |
|  | Figure 1. Rod and Cap |

Rotate the crankshaft to ensure free movement of the piston.

Part Two – Crankcase Cover

| - Install the tappets into the crankcase. Install the cam gear while holding the tappets in place. Turn crankshaft and crankgear to align the timing marks. Place the two metal dowels into the edge of the crankcase. The dowels will hold the crankcase gasket in place during installation. Place the crankcase gasket on the crankcase. Use new gasket if old one is torn. Use your finger to lubricate the bearings in the crankcase with 30W oil. Check the governor assembly to be sure the plastic thimble is in place. Have your teacher inspect the timing gear, governor, and piston to ensure proper installation. | Copyright Briggs and Stratton Corp. Used with permission |
| --- | --- |
|  | Figure 2. Crankcase Cover Sequence |

Slide the crankcase cover over the crankshaft so it fits tightly on the crankcase.

Use a 10mm socket and torque wrench to tighten the bolts on the crankcase to 75in-lb. Install using the sequence shown in Figure 2.

Repeat Step 9 at 150in-lb.

Repeat Step 9 at 210in-lb.

What pattern is the fastening sequence?

What are the benefits of using this type of pattern when fastening parts?

Part Two – Flywheel

| - Slide the flywheel and flywheel key on to the crankshaft. Align the flywheel and crankshaft keyways. Place starter cup, fan, and flywheel on crankshaft. Align the cup post to fit in the hole of the flywheel as seen in Figure 3. Tighten the flywheel nut to 65ft-lb using a 21mm socket, torque wrench, and flywheel strap. How many in-lb is 65ft-lb? (Multiply 65ft-lb by 12 inches per foot) Why do you believe the flywheel nut needs to be torqued to this amount? |  |
| --- | --- |
|  | Figure 3. Starter Cup and Flywheel |

Part Three – Valves and Valve Cover

| - Install the head gasket. Note the gasket can only go on one way to line up with bolt holes. The gasket is held in place by two dowels as seen in Figure 4. Install the engine head and valves using a 13mm socket and torque wrench. Torque to 75in-lb using the sequence shown in Figure 5. Repeat Step 3 at 150in-lb. Repeat Step 3 at 210in-lb. Why do you believe the torque is increased in multiple steps? Install the push rods with the loose rocker arms set to the side. Rotate the crankshaft of the engine until the pushrods are level with each other. Place the rocker arms over the push rods and valve stems. Set the valve clearance between the rocker arm and valve stem using a feeler gauge, 14mm wrench, and 10mm wrench. Refer to Activity 3.2.4 Machine Specifications in your Agriscience Notebook and Engineering Notebook for detailed procedures. | Copyright Briggs and Stratton Corp. Used with permission |
| --- | --- |
|  | Figure 4. Dowels |
|  | Copyright Briggs and Stratton Corp. Used with permission |
|  | Figure 5. Head Sequence |

Have your teacher check your valve settings.

Attach the valve cover using an 8mm socket and torque wrench.

Torque to 80in-lb.

Attach the heat shield using an 8mm socket and torque wrench.

Torque to 50in-lb.

Part Four – Armature

- Attach the armature using a torque wrench and 10mm socket.

Rotate the flywheel so the magneto is facing the armature brackets.

Place the Briggs and Stratton ruler on the magneto.

Place the armature on ruler and magneto.

The magneto will hold the ruler and armature tight.

Be sure the armature is facing the correct direction.

Tighten bolts to 95in-lb.

Slide Briggs and Stratton ruler out.

Rotate flywheel to ensure magneto is not hitting armature.

Part Five – Blower Housing and Wiring

- Connect the wires to the switch on the blower housing.

Adjust the wires around the engine.

Use notes from your Engineering Notebook if necessary.

- Install the fingerguard to 85in-lb using an 8mm socket and torque wrench.

- Install the blower housing to 85in-lb using an 8mm socket and torque wrench.

Use a 10mm socket and torque wrench to install the rewind starter to 30in-lb.

How are the components requiring a high torque different from those requiring a low torque?

Use a 5/8″ spark plug socket and torque wrench to install the spark plug to 180in-lb.

Clean up your area and store your engine and parts as instructed by your teacher.

Conclusion

- Why is the fastening sequence on a machine important?

- What factors affect the required torque on machine components?

- How are in-lb different from ft-lb?