Name_____________________________________________________

Activity 3.3.4 Start Your Engines

Purpose

You are about to finalize the assembly of your engine. Technicians often test and calibrate machines after reassembling them. What settings should you adjust on the small engine?

During Activity 3.2.3 System Controls, you diagramed the governor system of a small engine. The governor system needs to be set correctly during the reassembly process. A static governor setting is made where the governor arm attaches to the governor shaft. The shaft needs to move the governor arm in a direction that closes the throttle. If the static governor setting is not done correctly, the engine may always operate at WOT (wide open throttle), no matter what work the engine is doing. This will cause the engine to overheat.

The governor arm has multiple settings to adjust the placement of the linkage and governor spring. The governor spring pulls open the throttle plate and keeps it open when the engine is not running. Once the engine is operating, the governor arm pulls the linkage to close the throttle plate. The governor spring pulls in the opposite direction of the arm. The throttle plate will be set for a constant engine speed measured in rpm (revolutions per minute) where the forces of the spring and arm are equal. Technicians measure the speed of an engine with a tachometer.

The speed of an engine with no load, called the top no-load speed can be adjusted by changing the position of linkages and spring tension. A higher top no-load speed will allow the engine to work under a greater load. If the engine is under full load and is not supplied enough fuel, the RPM will decrease, and the engine will stop. Technicians carefully adjust the speed of an engine, so it can maximize the power needed to do work.

How will you start your engine and adjust its speed?

Materials

| Per pair of students: Small engine Briggs and Stratton OHV repair manual New governor arm (if needed) Vernier LabQuest2 Vernier force sensor 3/8″ drive in-lb torque wrench 3/8″ drive 8mm socket 3/8″ drive 10mm socket 3/8″ drive 12mm socket Pliers Paper clip | 3/8″ drive 13mm socket Tachometer Tang bender 2 C-clamps Funnel 30W oil Gasoline |
| --- | --- |
| Per student: Agriscience Notebook Engineering Notebook Employability Evaluation Rubric PPE Pen |  |

Procedure

Work with your partner to install the fuel and governor system on the small engine. Then operate the engine and measure the engine speed. Your work behavior will be assessed using Employability Evaluation Rubric.

Part One – Governor Installation

| Connect the governor spring to the speed bracket as seen in Figure 1. Note orientation of the spring. Use a torque wrench and 8mm socket to install the speed bracket to 85in-lb. Install the longest screw along with the red ground wire near cylinder head. Loosen the 10mm nut on the governor arm one to two turns. Spread the opening at the end of the governor arm. Slide the governor arm opening over the governor shaft. Do not tighten. Install the governor spring, linkage, and link spring as seen in Figure 2. Place the governor linkage in hole G. Place the link spring in hole F. Place the governor spring in hole 3. Part Two – Fuel System Installation - Install the carburetor and gaskets by sliding them on to the studs on the engine. Use your Engineering Notebook for referencing the correct order. Attach the governor link and link spring to the throttle shaft on carburetor as seen in Figure 3. Install the gasket and air cleaner base to the carburetor. Install screw to 50in-lb. Install nuts to 40in-lb. Complete a static governor adjustment. Rotate throttle plate to wide open. Watch rotation of the governor lever on the shaft while moving throttle to fast. Use pliers to turn the governor shaft in the same direction until it stops. Use 10mm socket and torque wrench to torque nut on governor arm to 30in-lb. Have your teacher check your static adjustment. Repeat Step 4 if adjustment is incorrect. Move the governor arm back and forth. What type of simple machine is the governor arm? |  |  |
| --- | --- | --- |
|  | Copyright Briggs and Stratton Corp. Used with permission |  |
|  | Figure 1. Spring Orientation |  |
|  | Copyright Briggs and Stratton Corp. Used with permission |
|  | Figure 2. Governor Spring Linkage |
|  | Copyright Briggs and Stratton Corp. Used with permission |
|  | Figure 3. Throttle Shaft Connection |

Sketch the governor arm in your Engineering Notebook and label the direction and forces on the governor arm needed to speed up the engine (open the throttle plate) and the direction and forces needed to slow down the engine (close the throttle plate).

Set up LabQuest2 and force sensor by following the directions below.

Attach the metal hook to the end of the force sensor.

Move the switch on the force sensor to 5N.

Turn on the Labquest2 and plug in the force sensor to Channel 1.

Turn the force sensor so the hook is pointing horizontally.

Tap on Sensor and choose Zero while the hook is pointing sideways to calibrate the sensor.

| Hook a paper clip in the hole by the letter D on the governor arm as seen in Figure 4. Hook the force sensor on the paper clip attached to letter D of the governor arm. Use the force sensor to pull on the governor arm until the throttle plate starts to move from the WOT position to the closed position. Record the force with the spring at the 3 setting in your Engineering Notebook and disconnect the force sensor. Move the governor spring to the 1 setting on the governor arm. Repeat Steps 10 – 11. |  |
| --- | --- |
|  | Figure 4. Force Sensor Connection |

Move the governor spring to the 5 setting on the governor arm.

Repeat Steps 10 – 11.

What will happen to the force needed to close the throttle plate as you change the location of the spring?

At which number will the engine have a higher top no-load speed? Why?

Adjust the tang on the speed bracket connected to the governor spring by using the tang bender to move it away from the spring.

Use the force sensor to pull on the governor arm until the throttle plate starts to move to the closed position.

Record the force in your Engineering Notebook and disconnect the force sensor.

Adjust the tang on the speed bracket connected to the governor spring by moving it towards the spring.

Repeat Step 17.

How does adjusting the tang change the force needed to close the throttle plate?

How can you use the tang to change the top no load speed of the engine?

Use the tang bender to move the tang on the speed bracket to a central position.

Install the fuel tank.

Use 8mm socket to torque screw above spark plug to 85in-lb.

Use 12mm socket to torque the two nuts above low oil module to 100in-lb.

Attach fuel line to fuel tank.

Install the air cleaner cover and filter by hand.

Use the13mm socket and torque wrench to install the muffler and gasket to 90in-lb.

Part Three – Engine Operation

- Attach the tachometer to the small engine.

Connect the tachometer red wire by wrapping it around the spark plug lead 3 to 4 times.

Attach the white ground wire of the tachometer to the red ground wire attached to the engine.

Prepare your engine for operation (Refer to Activity 3.1.6 Engine Energy when needed)

Keep the spark plug wire disconnected from the spark plug.

Place the throttle lever in the STOP position.

Add approximately 250ml of fuel to your engine.

Using the funnel, slowly pour oil into the engine oil fill to point of overflowing. Do not overfill by tipping the engine.

Go to the running station and secure the engine to the table using two C-clamps. Position the C-clamps next to the drain plugs on each side of the crankcase base.

Put on ear plugs.

Point the muffler to the air handling vent.

Wait for your teacher to check the engine setup.

Turn the fuel valve in front of the gas tank to the ON position.

Set the choke on the carburetor to the CHOKE position.

Connect the spark plug wire to the spark plug and move the throttle lever to the RUN position.

Turn the switch to ON.

Ask your teacher for permission to start the engine.

Once you receive permission, pull on the rope starter, gently and evenly. The engine should start in a few pulls of the rope starter.

When the engine is running, move the choke to the RUN position.

Adjust the throttle so the engine is idling.

Record the idle speed of the engine in your Engineering Notebook.

Adjust the throttle so the engine is at top no load speed.

Record the top no-load speed of the engine in your Engineering Notebook.

- With the engine running, turn the fuel valve to the OFF position.

Allow the engine to run until it is out of the fuel in the carburetor and stops running.

Move the throttle lever to the STOP position.

Turn the switch to OFF.

Disconnect the spark plug lead from the spark plug.

Part Four – Speed Adjustment

The engine will be used at a lower engine speed. Use what you have learned about governed speed adjustments and force to change the engine to a lower top no-load speed. Record your procedures and settings in your Engineering Notebook. Once you believe you have the settings correct, receive permission to operate your engine and test the rpm of your engine. Record your results in your Engineering Notebook. Repeat with new settings if you do not decrease the speed. After setting a lower rpm, properly drain all fuel and oil. Store your engine as instructed by your teacher.

Conclusion

- How are engine speeds adjusted?

- What factors affect an engine’s speed?

- How does the load on the engine affect how it should be adjusted?