Name_____________________________________________________

Project 3.3.1 Tool Reject

Purpose

Machinists construct engine components for a precise fit. Many of the engine components you disassembled fit together within thousandths of an inch. Components wear and break down over time causing the fit to change. If the component fit does not meet a specification, the technician replaces the component. How do you know when to reject a component?

Areas where two or more engine component surfaces slide against each other wear as the engine is used. Technicians use precision tools, such as a dial caliper, to measure these components. Then they compare their measurement to a reject size found in a repair manual. If the measurement is greater than the reject size, they keep the component. If the measurement is equal to or less than the reject size, indicating wear, they replace the component.

Some technicians are more accurate when making measurements than others. Tools can be designed to limit technician error and improve efficiency when measuring parts. For example, a tool with fixed sizes limits measurement error caused by improper use of a tool. A 13mm wrench can easily be used to determine if a nut is greater or less than 13mm by placing the wrench over the nut. If the same nut was measured with a dial caliper, it will take the technician more time to make the measurement, and there is a greater chance of user error.

How can you design a tool to increase the speed and precision for technicians measuring engine components?

Materials

| Per student: Agriscience Notebook Engineering Notebook Engineering Report Template Project 3.3.1 Evaluation Rubric Briggs and Stratton OHV repair manual Graph paper Ruler Computer with internet access Dial caliper | Per Class: 3D printer with plastic filament |
| --- | --- |

Procedure

Design a tool you can use to determine if engine components need to be replaced. Use the repair manual to find the reject sizes for the following components of your small engine and record in your Engineering Notebook.

Mag journal

PTO journal

Crankpin journal

Camshaft (PTO side)

Camshaft (Mag side)

Use the engineering design process to design a tool to identify reject sizes of engine journals. Document your design using the Engineering Report Template. Use graph paper and a ruler to draft a sketch of your tool including dimensions. Design a model of your tool using Onshape® software. Then 3D print your tool prototype as instructed by your teacher. Test your prototype for precision and record your results.

Your teacher will use Project 3.3.1 Evaluation Rubric to assess your report, sketch, drawing, and prototype and should meet the following criteria.

Report includes a problem statement, criteria, constraints, materials, procedure, analysis and conclusion.

The sketch includes all dimensions.

The prototype can be used to measure all journals on the crankshaft and camshaft of the engine.

The reject sizes on the prototype are correct for the engine model and are imprinted on the prototype.

The final drawing has all title block information, notes, views, and dimensions needed to recreate the prototype.

Conclusion

Where can you find information about reject sizes?

What engine components are measured for wear?

What factors should you consider when measuring and designing manufactured components?