Name_____________________________________________________

Activity 3.4.1 Replacement Parts

Purpose

When a machine breaks, a technician may need to order replacement parts. Identifying and ordering correct parts is an important technical skill. After a technician troubleshoots a system and identifies the problem, he or she will need to order replacement parts and determine the cost for fixing the machine. What is the process for finding the replacement costs for engine parts?

Companies use many of the same components in multiple machines. For example, the parts found in the small engine you worked on during this unit are used in multiple types of engines. Companies constantly update and change parts to meet consumer needs. Unique numbers assigned to engine and engine parts help technicians find the specific parts needed for making repairs.

The identification number on a Briggs and Stratton engine has three segments; model, type, and code. The model number describes features of the engine. The type describes how an engineer designed the engine for a specific use, such as a lawnmower for a specified company. The code describes when and where Briggs and Stratton manufactured the engine.

Once a technician knows the model number of the engine, he or she can find coded numbers and costs for specific parts. Online databases with diagrams provide information about part numbers and costs. A technician uses the part number and cost information to write a bill of materials for the customer. The bill of materials also includes the costs for labor and fees associated with fixing the engine. The customer can then decide if he or she wants to have the engine repaired. The combined cost of the engine repairs and labor may be more than purchasing a service replacement engine.

What resources are available for you to write a bill of materials for fixing an engine?

Materials

| Per student: Agriscience Notebook Engineering Notebook Briggs and Stratton Numerical Key | Pen Computer with internet access and word processing or spreadsheet software |
| --- | --- |

Procedure

Work individually to identify and decode the model number on a small engine. Then look up the coded number and costs for replacement parts. Make a bill of materials for repairing an example engine. Record all information and tables in your Engineering Notebook.

Part One – Engine Model

Find the model, type, and code on your small engine. The number is stamped on a metal plate near the rewind starter.

Use the Briggs and Stratton Numerical Key to explain the features of your engine, along with the date and location where it was built. Record the information in your Engineering Notebook.

Part Two – Part Number and Price

- In your Engineering Notebook, record a table similar to Table 1 below.

| Table 1. Parts Pricing |  |  |
| --- | --- | --- |
| Part Name | Part Number | Price |
| Crankshaft |  |  |
| Muffler |  |  |
| Carburetor |  |  |
| Head gasket |  |  |

- Login to The Power Portal and select eParts.

- Type the model, type, and code number into the search bar and click on Search.

- Find the numbers for the parts listed in the table. Record the number in your Engineering Notebook.

Go to https://shop.briggsandstratton.com/us/en/repair-parts and look up the price for each component listed above by entering the model and part number in the search bar. Record the price of each part in your Engineering Notebook.

Part Three – Cost Evaluation

A customer has asked a technician to service and repair a broken engine, with a Briggs and Stratton model and type number 19L232-0036-F1. The customer has not changed the oil in the engine and left gas sitting in the tank and carburetor over the winter. After troubleshooting the engine, the technician has found the following tasks would be required to repair the engine.

Replace all gaskets

Rebuild carburetor with carburetor rebuild kit

Replace connecting rod which has stripped threads

Replace old damaged air filter

Fill with 1 quart of oil

The technician estimates it will take four hours to complete the repairs at a cost of $36 per hour along with an additional $15 fee for waste disposal of old oil. Use word processing or spreadsheet software to estimate a bill of materials for the customer. The bill of materials should include all parts, part numbers, part costs, labor costs, and fees. Include the engine replacement cost as well so the customer can determine if the engine should be replaced or repaired. Use the database resources introduced during this activity to complete your bill of materials. Use additional web resources if necessary.

Conclusion

- How are costs found for repairing an engine?

Why is it important to know the model and part numbers of an engine?

Why should material and labor costs be considered before hiring a technician to repair a machine?