Name_____________________________________________________

Activity 4.1.1 Pulley Speed

If you look at the internal components of a machine, you may see a compound system of pulleys and belts. The pulleys and belts transfer energy. During the transfer process, pulley and shaft speeds change to work at a calculated rate. How do pulley sizes affect the speed of a drive system?

Belt drives are nonsynchronous drive systems transferring energy between two or more shafts. A nonsynchronous drive system does not have positive engagements between the parts, which means the parts can slip if resistance is greater than the friction between the belt and pulley. Machines require the correct amount of tension between the belt and pulley to function properly. Belts slip on the pulley if the tension is too loose. Increasing the belt tension reduces slippage and increases contact between the belt and pulley.

During Agricultural Power and Technology, you used ratios to calculate the rotating speed of shafts and gears in a windmill. Pulley speeds are calculated using ratios as well. The speed of the pulleys in a drive system are directly related to size of each pulley and speed of the drive gear. For example, if the drive gear connected to the engine is 4cm in diameter and the driven gear connected to the machine is 8cm, the gear ratio is 1:2 as shown in the calculation below.

Drive pulley diameterDriven pulley diameter=4cm8cm = Ratio of 1:2

A gear reduction is another way of expressing the relationship between connected pullies. The reduction is used to find the speed of the driven pulley. Find a reduction by dividing the driven pully diameter by the drive pulley diameter. This is the inverse of the ratio calculation. A pulley system with a 4cm drive pulley and 8cm driven pulley would have a reduction of 2.

Driven pulley diameterDrive pulley diameter=8cm4cm= Reduction of 2

With a 1:2 ratio, or reduction of 2, the driven pully will rotate at half the speed of the drive pully. Review the example ratio below for calculating the speed of pulleys in a mechanical drive system. Measure the pulley speed in revolutions per minute (rpm).

| - A drive pulley on an engine has a diameter of 30cm and is connected to a driven pully 150cm in diameter. - Calculate the reduction by dividing the driven pulley diameter by the drive pulley diameter. The reduction is 5. The driven gear is 5 times slower than the drive gear. | Driven pulley diameterDrive pulley diameter=150cm30cm 150cm30cm= 5 |
| --- | --- |
| - The known speed of the drive pulley is 100 rpm. - Find the speed of the driven pulley using the reduction of 5. Divide the drive rpm by 5 to find the rpm of the driven gear. | 100 rpm5 = 20 rpm |
| - The speed of the driven pulley is 20 rpm. | Driven pulley rpm = 20 rpm |
| Figure 1. Example Pulley Reduction |  |

| Compound pulley systems, as seen in Figure 2, reduce the speed of a drive system more than a two-pulley system would allow. The system in Figure 2 has an idler pulley in between the drive and driven pulleys. The calculations for the compound pulley reduction of the drivetrain in Figure 2 is shown in Figure 3. |  |
| --- | --- |
|  | Figure 2. Compound Pulley System |

| - Find the compound pulley reduction by multiplying each reduction. - Calculate the reduction between the drive and idler pulley by dividing the idler pulley diameter by the drive pulley diameter. - Calculate the reduction between the idler and driven pulley by dividing the idler pulley diameter by the drive pulley diameter. The reduction is 25. The driven gear is 25 times slower than the drive gear. | Compound reduction = 15030×15030 Compound reduction = 5 × 5 Compound reduction = 25 Reduction = 25 |
| --- | --- |
| - The known speed of the drive pulley is 100 rpm. - Find the speed of the driven pulley using the reduction of 25. | 100 rpm25 = 4 rpm |
| - The speed of the driven pulley is 4 rpm. | Driven pulley rpm = 4 rpm |
| Figure 3. Compound Pulley Ratio |  |

Drivetrains are an essential component of transferring energy. How are compound pulley drivetrains used to change speeds in a system?

Materials

| Per group of four students: Ruler LabQuest2 2 rotary motion sensors Electric motor kit with pulley Flat head screwdriver Ring stand 2 rubber bands Black wire lead Red wire lead D-size battery case D-size battery | Per student: PPE Agriscience Notebook Engineering Notebook Pen |
| --- | --- |

Procedure

Work in a group of four to calculate and measure pulley speeds using a motor powering two rotary motion sensors. Split in two pairs during Part One and work as a group for the remainder of the activity.

Part One – Pulley System

Break your group into two pairs. Assign one pair to complete Steps 1 – 9 and the other pair to complete Step 10. Record all data in your Engineering Notebook.

Use a metric ruler to measure and record the diameter (mm) of the pulley on the electric motor.

Measure and record the diameter (mm) of the small, medium, and large pulleys on the rotary motion sensor.

| Place the rotary motion sensors 3cm apart on the ring stand as shown in Figure 4. Adjust the combination pulley on the top sensor so the largest pulley is furthest from the sensor. Adjust the combination pulley on the bottom sensor so the largest pulley is closest to the sensor. Use a flat head screw driver to attach the motor mount and electric motor to the top rotary motion sensor. Place the D size battery into the battery case. Attach the black wire lead on the negative side of the battery case and the negative connection on the motor. Attach the red wire lead on the positive side of the battery case. Setup the LabQuest2. |  |
| --- | --- |
|  | Figure 4. Rotary Motion Sensors |

Turn on the LabQuest2

Plug the top rotary motion sensor connection into Digital 1.

Plug the bottom rotary motion sensor connection into Digital 2.

Tap on Dig 1, select Reset (Zero) On Collect, and change units to rev.

Select OK.

Tap on Dig 2, select Reset (Zero) On Collect, and change units to rev.

Select OK.

Tap on Duration and change the Rate to 30 and the Duration to 30 sec.

Select OK.

Part Two – Two Pulley System

- Put on PPE and tie back long hair.

- Wrap the rubber band around the small pulley on the top sensor and the pulley on the motor. You will use the second rotary motion sensor during Part Three.

- Sketch your pulley design in your Engineering Notebook.

Which pulley is the drive, and which is the driven?

What is the pulley ratio and reduction between the drive and driven (small) pulley? (Use measurements from Step 2 of Part One)

Will the drive speed be greater or less than the driven? Why?

Attach the red wire lead from the positive connection of the motor to the positive side of the battery case. The motor should start spinning.

Tap the Start button on the LabQuest2 to begin collecting data.

Once data has stopped collecting, remove the red wire lead from the battery case.

Select Analyze and choose Statistics.

Select Velocity.

What was the average velocity (speed) of the driven pulley?

Based upon the ratio of the drive and driven pulley, what is the calculated speed of the drive pulley?

Wrap the rubber band around the largest pulley on the top sensor and the pulley on the motor. Adjust the motor so the pulleys are aligned with each other.

Sketch your pulley design in your Engineering Notebook.

What is the pulley ratio and reduction for the drive and driven (large) pulley?

Based upon your reduction and calculated drive pulley speed, what is the speed of the large driven pulley?

Tap the file cabinet on the Graph screen to start Run 2.

Attach the red wire lead from the positive connection of the motor to the positive side of the battery case. The motor should start spinning.

Tap the Start button on the LabQuest2 to collect data.

Once data has stopped collecting, remove the red wire lead from the battery case.

Select Analyze and choose Statistics.

Select Velocity.

What was the average velocity (speed) of the driven pulley?

How do the results compare to your calculations?

What factors may impact your results?

Remove the rubber band from the system.

Part Three – Three Pulley System

- Connect the small pulley on the top sensor to the largest pulley on the bottom sensor.

- Connect the motor pulley to the largest pulley on the top sensor.

Sketch the design in your Engineering Notebook.

Record the ratios and reductions in the system in your Engineering Notebook.

Use the reductions and past data collected on the motor speed to predict the speed of the drive pulley, idler pulley, and driven pulley. Record your calculations in your Engineering Notebook.

Have your teacher check your calculations before moving on to Step 6.

Tap the file cabinet on the Graphing screen to start Run 3.

Attach the red wire lead from the positive connection of the motor to the positive side of the battery case. The motor should start spinning.

Tap the Start button on the LabQuest2 to begin collecting data.

Remove the red wire lead from the battery case once data has stopped collecting.

Select Analyze and choose Statistics.

Select Velocity for Dig 1.

Select Analyze and choose Statistics.

Select Velocity for Dig 2.

Record the average pulley speeds in your Engineering Notebook.

What should the speed relationship be between the two pulleys on the same shaft of the compound pulley?

How can compound pulley systems be used to reduce rpm?

How do the results compare to your calculations? What factors may impact your results?

Shut off the LabQuest2 and disconnect the rotary motion sensors.

Remove rubber bands and wires from the rotary motion sensors.

Put away materials as instructed by your teacher.

Conclusion

- What factors affect the speed of a pulley system?

- How can you reduce the driven pulley speed?

- How does the pulley diameter affect the driven speed in a compound pulley system?