Name_____________________________________________________

Activity 4.1.2 Shifting Gears

Purpose

Have you ridden in a car where the driver manually shifted gears? What happens every time the driver shifts? How can gear systems have multiple speeds? Agricultural machines, such as tractors, have gear systems that change speeds as well.

During Activity 4.1.1 Pulley Speed, you used idler pulleys to design compound pulley systems. Compound gear systems have a drive, idler, and driven gear and work similar to compound pulley systems. The number of teeth on each gear is used to calculate the gear reduction in a compound gear system. Multiple combinations of idler gears in a gearbox provide speed options for a single system. A system of gears changing speeds is called a transmission. Power required to operate the transmission will depend upon the gear reduction.

How do you design a system with shifting gears? How do gear reductions affect power requirements?

Materials

| Per team of three students: Plastic storage containers 2 12-tooth gears 2 36-tooth gears 2 60-tooth gears 3 60-tooth extra strength gears 3 square shaft inserts 1 4″ wheel 3 chassis rails, 25 holes (C-shaped) 2 chassis bumper angles, 25 holes (L-shaped) 1 2-wire motor 1 clutch Flat bearing 2 ½″ motor screws 2 12″ square shafts 1 4″ square shaft 11 shaft collars with set screws | 4 #8-32 threaded standoffs, 3″ long 8 #8-32 button head screws ¼″ long 4 #8-32 keps nuts 5 8mm spacers ¼″ wrench or nut driver 5/64″ hex wrench 3/32″ hex wrench LabQuest2 Energy sensor 7.2V battery 2 black leads with alligator clips 2 red leads with alligator clips Masking tape 9-volt battery adapter Electrical tape |
| --- | --- |
| Per student: Agriscience Notebook Engineering Notebook Guide for Building Clawbot | PPE Pen |

Procedure

Assign each person on the team as a tradesperson, technician, or engineer. Follow your assigned role and work with your teammates to assemble a gearbox. Then test the system for speed and required power.

Part One – Gearbox Assembly

Work as a team to complete Step 1. The technician and tradesperson should complete Step 2, while the engineer reads Steps 3 – 18 and makes notes in his or her engineering notebook about what needs to be assembled. Once Step 2 is complete and the engineer has read all remaining steps, work as a team to assemble the gearbox.

As a team, observe the example gearbox assembled by your teacher and shown in Figure 1 with a top, front, and side view of the gearbox.

Have the assigned technician and tradesperson gather the materials and organize in plastic storage containers as instructed by your teacher. The engineer should read the remaining steps in Part 1.

Use pages 2 – 5 of Guide for Building Clawbot to identify the parts.

Place large parts in the bottom storage area of the tool box.

Organize small pieces you may lose in small, sealable plastic storage containers.

| Front | Top | Side |
| --- | --- | --- |
| Figure 1. Gearbox Views |  |  |

| Use a 3/32″ hex wrench and ¼″ wrench to assemble the following materials to make a C shaped frame setting on four standoffs as shown in Figure 2. 2 chassis bumper angles, 25 holes, C-shaped 1 chassis rail, 25 holes, L-shaped 4 #8-32 threaded standoffs, 3″ long 4 #8-32 button head screws ¼″ long Slide the clutch on the shaft of the motor as seen in Figure 3. Attach the motor and flat bearing to the outside of the chassis rail. Line up the drive and clutch of the motor with the 5th hole on the chassis rail as seen in the Top view of Figure 1. Use the 5/64″ hex wrench to fasten two ½″ screws to the motor and chassis rail. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | Figure 2. C Frame |  |
|  |  |  | Figure 3. Motor Connection |  |
| Assemble Axle 1 using the following parts on the 4″ shaft in the order listed from left to right. Leave approximately 5mm of the shaft extending from the side with the 8mm spacer. Use Figure 4 for reference. 8mm spacer 12-tooth gear 8mm spacer 8mm spacer 36-tooth gear 8mm spacer 60-tooth gear Shaft collar |  |  |  |  |
|  |  |  |  | Figure 4. Axle 1 |
| Assemble Axle 2 using the following parts on a 12″ shaft in the order listed from left to right. Use Figure 5 for reference. Refer to this shaft as Axle 2. Shaft collar 60-tooth gear 8mm spacer 36-tooth gear 12-tooth gear Shaft collar |  |  |  |  |
|  |  |  |  | Figure 5. Axle 2 |
| Place a square shaft insert into each 60-tooth high strength gear. Assemble Axle 3 using the following parts on the second 12″ axle in the order listed from left to right. Use Figure 6 for reference. Refer to this shaft as Axle 3. Shaft collar 60-tooth high strength gear 60-tooth high strength gear 60-tooth high strength gear Shaft collar |  |  |  |  |  |
|  | Figure 6. Axle 3 |  |  |  |  |
| Place the shafts into the following rail holes. Use Figure 7 for reference. Axle 1 – Hole 5, connected to the motor clutch Axle 2 – Hole 8 Axle 3 – Hole 13 Use two ¼″ screws and keps nuts to fasten a second rail to the frame to hold the opposite side of each shaft as seen in Figure 8. Use Step 10 for hole placement of axle Tighten the rail on the 7th hole of each bumper chassis |  |  |  |  |
|  |  | Figure 7. Axle Placement |  |  |

| Place shaft collars on Axle 2 and Axle 3 to hold them in place as seen circled in Figure 8. Collars are placed on the outside of the rails. Four shaft collars are needed Slide the following parts onto Axle 3 in the following order. Shaft collar 4″ wheel Shaft collar Use two ¼″ screws and kep nuts to fasten a third rail to the frame to hold Axle 2 and Axle 3 as seen in Figure 9. |  |  |
| --- | --- | --- |
|  | Figure 8. Second Rail |  |
| Use Step 10 for hole placement Tighten the rail on the 12th hole of each chassis bumper Tighten all shaft collars except the two on the outsides of Axle 2. Use a 5/64″ hex wrench Tightening the shaft collars will keep all gears, axles and 8mm spacers in place Slide Axle 2 back and forth and observe how the idler gears connect in the system. What are the gear combinations that can drive the wheel? Predict the speeds of each combination by ranking them from fastest to slowest. Adjust Axle 2 so the 60-tooth gear of Axle 2 is in contact with drive and driven gears. |  |
|  | Figure 9. Completed Gear Assembly |

Tighten the shaft collars on the outsides of Axle 2 using a 5/64″ hex wrench.

Part Two – Power Supply

- Turn on the LabQuest2.

Attach the energy sensor to Channel 1 and Channel 2 of the LabQuest2.

| Setup the LabQuest2 Tap on Mode and choose Events with Entry Change Name to Idler Gear Select Average over 10 seconds Tap OK Attach a red and black wire lead to the source side of the sensor. Attach a red and black wire lead to the load side of the sensor. Place a small piece of electrical tape in the center of the 9-volt battery adapter as seen in Figure 10. Attach the 9-volt adapter to the electric motor wire. |  |
| --- | --- |
|  | Figure 10. 9-Volt Battery Adapter |

Attach the load side red and black wires to the adapter as seen in Figure 11. Note how connections are separated by the electrical tape to prevent a short circuit and potential electrical fire.

Attach the black wire on the source side to the black terminal of the battery.

Leave the red wire attached to the source side of the energy sensor disconnected from the battery.

Use Figure 11 as a guide for confirming energy sensor setup.

| Figure 11. Energy Sensor Connections |  |  |
| --- | --- | --- |

Part Three – Gear Speeds

- Place a piece of masking tape on the tread of the wheel.

Make a table similar to Table 1 in your Engineering Notebook for recording gear speed data.

| Table 1. Gear Speed Data |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gear Settings | Drive Teeth | Idler Teeth | Driven Teeth | Gear Reduction | Current (milliamp) | Power (milliwatt) | Wheel rpm |
| 1st Gear Setting |  |  |  |  |  |  |  |
| 2nd Gear Setting |  |  |  |  |  |  |  |
| 3rd Gear Setting |  |  |  |  |  |  |  |

In the 1st Gear Setting row, record the number of teeth of the drive, idler, and driven gear currently driving the wheel.

Calculate the compound gear reduction for the 1st Gear Setting and record in your Engineering Notebook.

Put on PPE and tie back long hair.

Press Start on the LabQuest2 to begin data collection.

Attach the red lead on the source side of the energy sensor to the red connection on the battery.

Tap Keep.

Enter the number teeth on the idler gear engaging the drive gear.

Work with your partner to count the number of revolutions the wheel makes for 15 seconds.

Have your partner time you as you count the number of times the tape on the wheel rotates.

Disconnect the red wire lead from the battery to stop the system.

Multiply the number of revolutions during 15 seconds by 4 to find the rpm of the wheel.

Number of revolutions x 4 = rpm

Record the rpm in your Engineering Notebook.

Use the 5/64″ hex wrench to adjust the idler gear so the 36-tooth idler engages the drive gear and 60-tooth idler engages the driven gear.

Repeat Steps 3 – 13 for the second gear setting.

Use the 5/64″ hex wrench to adjust the idler gear so the 12-tooth idler engages the drive gear and 60-tooth idler engages the driven gear.

Repeat Steps 3 – 13 for the third gear setting.

Press Stop on the LabQuest2.

Tap on the Table button at the top of the LabQuest2 screen.

In your Engineering Notebook, record the Current and Power data from the LabQuest2 and answer the analysis questions?

How did the gear ratio affect the speed of the wheel?

What was the relationship between the speed and current draw from the battery?

Store the gearbox and materials as instructed by your teacher. You will be using the gearbox for the next activity.

Conclusion

- What is the relationship between current draw and speed of a gear system?

- How can compound gear systems be used to change speeds?

- Why are gearboxes essential components of mobile machines, such as a tractor?