Name_____________________________________________________

Activity 4.1.3 Speed Versus Power

Purpose

Bicyclists use gears on their bike for both speed and for power. When trying to pass another bicyclist, they may shift up to smaller gears to acquire more speed. When going up a hill they may shift down to larger gears to acquire more power. How are speed and power measured in a multi-gear system?

In previous activities, you have measured the rotational speed of gears and wheels in a system using revolutions per minute (rpm). Sometimes speed is measured linearly such as feet per minute or miles per hour. The linear speed of a gear system driving a wheel is dependent upon the diameter of the wheel. As the diameter of the wheel increases, the greater distance it travels for every revolution. The distance a wheel travels for each revolution is equal to the circumference of the wheel. Find the circumference of a wheel by multiplying the diameter by pi (π) or approximately 3.14. Multiply the circumference of the wheel by the number of revolutions per minute to find a distance the wheel will travel during a set period of time. See example calculations in Figure 1 for a wheel that has a diameter of 12in and a speed of 100 revolutions per minute.

| Circumference | Speed – Inches per Minute | Speed – Miles per Hour |
| --- | --- | --- |
| Circumference = D × π Circumference = 12in × π (3.14) Circumference = 37.70in The wheel travels 37.70in for every revolution | DistanceTime=100 revolutionsminute×37.70in1 revolution DistanceTime=3770inminute | DistanceTime=3770inminute×1ft12in DistanceTime=314.16ftminute DistanceTime=314.16ftminute×1 mile5280ft×60 minutes1 hour DistanceTime=3.57 mileshour The wheel is traveling 3.57 miles/hour |
| Figure 1. Calculations |  |  |

| A wheel is a lever that revolves around an axis. The length of the lever is the radius of the wheel, as seen in Figure 2. The power of a wheel is based upon torque. Measure torque of a wheel by multiplying the weight it can lift by the radius of the wheel. For example, a wheel with a radius of 6 centimeters that can lift 10 Newtons has a torque of 60N-cm or 0.6N-m. Some gear systems have a clutch on the drive shaft to prevent the system from breaking if the system cannot handle the torque placed upon it. If the force is too great, the clutch will slip, and the gears will not move. The clutch wears down each time it slips and will eventually break. Operators should shut down the system immediately if they observe the clutch slipping. |  |
| --- | --- |
|  | Figure 2. Wheel |

What is the relationship between the gear ratio, torque, and speed in a system? What factors will cause a gear system to fail and a clutch to slip?

Materials

| Per team of three students: Ruler Calculator Gear box from Activity 4.1.2 Shifting Gears Toggle switch 3 red wire leads 2 black wire leads Electronic balance 40 ½″ washers 9oz cup 3′ string 7.2V battery | Per student: PPE Agriscience Notebook Engineering Notebook Pen |
| --- | --- |

Procedure

Work with your teammates to measure the speed and torque of a wheel driven by different gear systems. Record all data, answers to analysis questions, and calculations in your Engineering Notebook.

Part One – Speed Calculations

Use a ruler to measure the diameter of the wheel in inches.

Calculate the circumference of the wheel.

Use your rpm calculations from Activity 4.1.2 Shifting Gears to find the distance the wheel would travel in one minute for each of the three gear reductions.

Convert inches per minute to miles per hour for each of the three gear reductions. Then answer analysis questions.

How do gear reductions affect linear speed?

What would happen to the speed of the system if the wheel were smaller? Why do you believe so?

How could you increase the speed of the system without changing the gear reductions?

Part Two – Power Calculations

- Find the radius of the wheel in meters and record in in your Engineering Notebook.

Put on PPE and tie back long hair.

Attach a black and red wire lead to the 9-volt adapter on the two-wire motor.

Attach the other end of the red wire lead to a toggle switch.

Attach the other end of the black wire lead to the battery.

Attach one end of the second red wire to lead to the battery.

Attach a string to the cup.

Tie the other end of the string to the wheel as shown in Figure 2.

Hang the cup over the bracket and edge of the table.

Adjust the idler gear so the 60-tooth idler gear is engaged to both the drive and driven gears.

Attach the positive end of the battery to the second wire on the switch.

Be sure the switch is in the OFF position.

| Slowly turn the gears to wrap the string ¼ the way around the wheel so the string is at the top of the wheel as shown in Figure 2. Add washers one at a time until the wheel turns or you run out of washers, whichever comes first. Once the cup has fallen or all washers have been added, have a teammate hold on to the system so it does not fall of the edge. Turn on the motor by switching the toggle switch to the ON position. Shut OFF the system after the cup raises approximately one foot or if the clutch slips. Caution: Keep all hands and objects away from the moving gears and shafts. Use an electronic balance to find the weight in Newtons of the washers in the cup. Calculate the torque placed on the system by multiplying the weight of the washers by the radius of the wheel. |  |
| --- | --- |
|  | Figure 2. Wheel and String |

Adjust the idler gear so the 36-tooth and 60-tooth idler gears are engaged.

Repeat Steps 11 – 15.

Adjust the idler gear so the 12-tooth and 60-tooth idler gears are engaged.

Repeat Steps 11 – 15.

Answer analysis questions.

How does a compound gear design affect torque?

What is the relationship between speed and torque in a gear system?

Tear down your system and store parts as instructed by your teacher.

Conclusion

- Large tractors have engines with more power than some cars. Why is a tractor’s speed slower than a car?

- Why are vehicles designed to change gears from low speed to high speed?

- Why are speed and torque both considered when designing a drivetrain?