Name_____________________________________________________

Activity 4.1.4 Model Tractor

Purpose

Tractors are sources of power in agricultural production. You may have seen, or even driven, a tractor tilling a field, harvesting a crop, or mowing a lawn. When you sit in the driver’s seat of a new tractor you will see levers and buttons for controlling systems. A tractor is a compilation of systems working together. What are those systems and how are they controlled?

Four major systems found in a tractor include the structural system, control system, drivetrain, and manipulators. The structural system of a tractor, called the chassis, holds the drivetrain and manipulators on the tractor in place. The drivetrain is a system of parts, such as gears, sprockets, and pulleys driven by an energy source producing rotational or linear motion. The energy source for most tractors is an engine. The tractor’s drivetrain controls how it moves from one place to another. The drivetrain may also transfer energy to manipulators on the tractor, such as the power take off (PTO). The PTO controls implements for planting, managing, and harvesting crops.

Systems in a robot are very similar to those found on a tractor. Although the energy source may be different, the design of the chassis, drivetrain, and manipulators are the same, just in a smaller scale. In this activity, you will construct a model tractor using robotic parts. A model can be used to simulate activities large scale tractors do in the field.

How will you assemble systems to mobilize a model tractor?

Materials

| Per team of three students: 2 VEXnet USB keys VEX remote VEX Clawbot kit VEX Guide for Building the Clawbot Cortex 5/64″ hex wrench ¼″ wrench or ¼″ nut driver 3/32″ hex wrench Wiring tool Set of two 16ga spade connectors 7.2V Battery USB cable | Per student: PPE Agriscience Notebook Engineering Notebook 2 Employability Evaluation Rubrics Pen |
| --- | --- |

Procedure

Work with your teammates to construct a mobile chassis and drivetrain for a model tractor. Assign an engineer, technician, and tradesperson for your team. The engineer will be responsible for reading the directions in the Guide for Building the Clawbot and instructing the team how to build the model tractor. The tradesperson will handle all tools during the construction process. The technician can assist the tradesperson or engineer during the activity. Upon completion of the activity, assess your teammates using the Employability Evaluation Rubric.

Part One – Inventory Parts

Review pages 2 – 4 of Guide for Building the Clawbot.

Categorize the components shown on pages 2 – 4 in your Engineering Notebook into the following systems.

Structure

Control system

Manipulators

Drivetrain

Match the picture of each component to the components found in your robot kit.

Find the tools in your kit.

Review page 5 explaining how to use each tool.

Observe your teacher demonstrate how to use pop rivets and fasten a bearing.

Part Two – Construct the Chassis

- Review Steps 1 – 11 on pages 6 – 9 of Guide for Building the Clawbot.

- In your Engineering Notebook, make a materials list of all components and the quantity of each needed to complete Steps 1 – 11.

- Acquire all components for completing Steps 1 – 11.

- Complete Steps 1 – 11 to assemble the chassis following each of your assigned roles.

Part Three – Build the Drivetrain

- Review Steps 14 – 17 of Guide for Building the Clawbot.

- In your Engineering Notebook, make a materials list of all components and the quantity of each needed to complete Steps 14 – 17.

- Acquire all components for completing Steps 14 – 17.

- Complete Steps 14 – 17 to assemble the drivetrain following each of your assigned roles.

- Attach the battery straps to the back of the chassis as shown in Step 23.

Part Four – Add Power and Controls

- Review Step 24 of Guide for Building the Clawbot.

- In your Engineering Notebook, make a materials list of all components and the number of each needed to complete Step 24.

- Acquire all components for completing Step 24.

- Attach the wires of the left and right drive motor to the cortex.

Left drive motor – Port 1

Right drive motor – Port 10

| Use the wiring tool to cut the red wire lead of the battery in the center. Strip approximately ¼″ of the insulation off each cut end of the red wire. Use the wiring tool to crimp the male 16ga spade connector to one of the stripped ends and the female 16ga spade connector to the other end. Your teacher will demonstrate how to use the wiring tool for crimping connections. Attach the 16ga spade connectors together to ensure a proper fit. Figure 1 has a picture of a battery spliced with terminal connectors. Strap the battery onto the back of the chassis and plug in the battery to the cortex. Figure 2 shows a completed model tractor. |  |
| --- | --- |
|  | Figure 1. Battery |
|  | Figure 2. Model Tractor |

Part Five – Sync and Drive

- Plug one end of a USB cable to the remote.

Plug the other end of the USB cable to the cortex.

Turn the cortex on.

Watch for the robot and VEXnet LED’s to blink green once.

Turn the remote and cortex off.

Disconnect the USB cable from the cortex and remote.

Plug in a VEXnet USB key into the cortex.

Plug in a second VEXnet USB key into the remote.

Turn on the cortex and remote.

Wait 15 seconds for the lights to blink green.

Inform your teacher if the lights blink slowly or are colors other than green.

Use the remote to operate the tractor. Have each team member test drive the tractor.

Left joystick controls the left drive motor.

Right joystick controls the right drive motor.

Shut off the remote and cortex.

Attach each motor to the corresponding ports below. Be sure the black wire is attached to the connection furthest from the center.

Left drive motor – Port 10

Right drive motor – Port 1

- Turn on the remote and cortex.

- Use the remote to operate the tractor. Have each team member test drive the tractor.

How have the controls changed?

How do control system connections affect how the remote interacts with the tractor?

- Shut off the remote and cortex.

Attach each motor to the corresponding ports below. Be sure the black wire is attached to the connection furthest from the center.

Left drive motor – Port 1

Right drive motor – Port 10

Store model tractors as instructed by your teacher.

Complete an Employability Evaluation Rubric to assess each of your teammates. Turn them in to your teacher once complete.

Conclusion

- What systems are found in a machine or tractor?

How are systems in a machine dependent upon each other?

How are robots and tractors similar?