Name_____________________________________________________

Project 4.1.5 Tractor Pull

Purpose

Have you ever attended a tractor pull competition? A tractor pull involves people competing to see which tractor can pull the most weight, or load. Tractors pull large loads in more practical settings as well. Farmers use tractors to till fields, haul grain, and plant crops, among many other operations done on a farm. How is a tractor designed for doing these jobs? What factors affect the power needed by a tractor?

A tractor has a drawbar where implements to be pulled are hitched to the tractor. If the drawbar is not set correctly, the tractor may not be able to pull as much weight, or the tractor may lose stability. The center of gravity on a tractor will also affect its stability. The center of gravity is the point where all weight is distributed equally on an object. Ideally, that would be in the center of a tractor. However, when a tractor is pulling a large load, the center of gravity will move towards the back of the tractor, which can cause the tractor to become unstable and the front end to raise. Tractor operators can adjust for pulling large loads by adding weight to the front of the tractor to move the center of gravity.

As calculated in Activity 4.1.3 Speed Versus Torque, gear ratios can also affect a tractors power and speed. Tractors are capable of both speed and power based upon the gear it is in. Tractors have “high” gears designed for speed to move quickly from place to place on a road. “Low” gears on a tractor provide high power and torque for pulling heavy objects such as a plow when tilling a field.

When a tractor requires more power, it draws more energy in the form of a combustible fuel. Your model is drawing energy in the form of electrical current from a battery. Current, measured in amperes, multiplied by the voltage equals the total power or watts used by the model. Find the work completed by the model tractor by multiplying the power by time.

How does the drawbar design, center of gravity, and gear ratio optimize the power and work done by a model tractor?

Materials

| Per class: Various weights (2.5lb – 10lb) VEX supplies ½″ I.D. washers Per student: Agriscience Notebook Engineering Notebook Engineering Report Template Project 4.1.5 Evaluation Rubric Pen PPE | Per team of three students: Model tractor USB cable Two 8″ zip-ties Electrical tape Digital multimeter 5/64″ hex wrench ¼″ wrench or ¼″ nut driver 3/32″ hex wrench Angle gusset 2 #8-32 button head screws, ¼″ long 2 #8-32 keps nuts | VEX remote VEXnet USB Keys Empty box cover 3ft string Tape measure Masking tape Permanent marker Scissor Stop watch |
| --- | --- | --- |

Procedure

Work with your teammates to prepare a model tractor to measure power. Next, work in a group to test each pair’s model tractor by pulling a small and large load. Then work in pairs to modify a model tractor to increase its power, stability, and ability to do work. Record all data, calculations, and observations in your Engineering Notebook.

Part One – Battery and Multimeter Preparation

| Place the multimeter on the chassis of the model tractor so it does not interfere with controls or transmission as seen in Figure 1. Zip tie the multimeter to the chassis to prevent it from moving. Place the black terminal of the multimeter in the COM port of the multimeter. Place the red terminal of the multimeter in the 10ADC port of the multimeter. Disconnect the 16ga spade connectors on the battery. |  |
| --- | --- |
|  | Figure 1. Tractor with Multimeter |

Attach the red terminal of multimeter to the 16ga spade connector on the red wire connected to battery.

Attach the black terminal of the multimeter to the 16ga spade connector on the other red lead wire.

| Use electrical tape to tape the connectors together to prevent an electrical short. Use a 5/64″ wrench to attach the VEX angle gusset to the center of the back rail on the model as seen in Figure 2. Use the angle gusset as a drawbar. |  |
| --- | --- |
|  | Figure 2. Angle Gusset |

Part Two – Tractor Pull

Work with your team to test the tractors pulling power.

- Place two strips of masking tape 6ft apart. Use a marker to label one start and the other finish.

Place the box cover at the starting line.

Attach the empty box cover to the drawbar of the model tractor using a string.

Turn the cortex and the remote to the on position.

Have a teammate use the tractor to pull the box 6ft at maximum speed when you say “Go.”

Start the stopwatch when you say “Go”.

Have the third teammate observe the current draw readings on the multimeter while another drives the model.

Tell your teammate to “Stop” when the box crosses the finish line or when the box stops moving.

Stop the stopwatch timer.

Record the time and current draw in your Engineering Notebook.

Current readings will not be consistent, record an estimated reading.

Note any other stability problems with the model in your Engineering Notebook.

Place the box back at the starting line.

Repeat Steps 5 – 9 two more times so each teammate can drive the tractor.

Add ten pounds of weight to the box.

Repeat Steps 5 – 11.

Calculate the power used for each tractor pull using the following equation. Assume the voltage of the battery is constant at 7.2V.

power (watts) = current (amperes) × voltage (volts)

Calculate the work done for each tractor pull by using the following equation.

work(watts-seconds) = power (watts) × time (seconds)

Use a scissors to cut the zip-ties holding the multimeter on each model tractor.

Disconnect the multimeter from the battery.

Record answers to the analysis questions in your Engineering Notebook.

What was the relationship between power and work?

How did the load change the center of gravity of the model?

How could you improve the design to increase stability of the model and the work it can do?

Part Three – Tractor Design

Work with your teammates to redesign the tractor chassis and transmission system to increase its power. Use the available VEX gears, axles, and framing materials to complete your design. Record and sketch all notes and designs in your Engineering Notebook. The model must meet the following criteria.

Gear reduction designed for pulling heavy loads

Drawbar for attaching equipment to pull

Stable center of gravity when pulling heavy loads

Left and right drive motor controlled remotely

The following resources may help you start redesigning your model tractor.

Tractor stability: http://articles.extension.org/pages/70338/tractor-stability

Tractor stability and instability: https://extension.psu.edu/tractor-stability-and-instability

Connecting implements to the tractor: https://articles.extension.org/sites/default/files/NSTMOP%20Task%20Sheets%20Section%205%202013.pdf

Test your design using a procedure similar to Part Two. Share the remote control with another pair when testing your design. Report information about your design using the Engineering Report Template. Your design will be assessed using Project 4.1.5 Evaluation Rubric and should include the following information.

Gear ratios and reduction calculations

Energy usage

Power and work calculations

Speed calculations

Conclusion

- What factors affect energy required in a mobile system?

- How does a tractor’s design affect its stability and work performance?

- What other modifications could you make to the tractor to increase its ability to do work?