Name_____________________________________________________

Activity 4.2.3 Virtual Reality

Purpose

Have you ever been somewhere that you do not speak the language? Did you need a translator? For a robot to understand your instructions, you need to translate it into a language the robot can understand. ROBOTC is a common programming language used in industry. How are instructions written in ROBOTC?

Programmers follow specific rules when writing a program. Just like you follow certain rules when writing a paper, a program must have correct spelling, capitalization, and punctuation for the robot to understand it. The first instruction for the robot is called the task main. Instructions for task main are placed between curly brackets { }. The robot follows the instructions in the order they are written. Common instructions include when to turn a motor on and off. Motors connect to ports in the cortex. A programmer writes instructions to tell the cortex when to send power to the port, how much power to send, and how long to send it. Read the program in Figure 1.

| The instruction tells the cortex to send maximum power to port5 and port6. The power range for a VEX motor ranges from -127 to 127. 127 is maximum forward, -127 is maximum reverse, and 0 is no power. If the programmer wants the motor to move slower, he or she can change the power to 63, which is approximately half power. The end of each command has a semicolon, which is like a period in our language. It tells the cortex when one command ends, and another command begins. |  |
| --- | --- |
|  | Figure 1. Example Code |

The cortex will continue to send max power to port5 and port6 for the length of time it is programmed. Notice the port location is placed in between square brackets [ ]. Ports can be named to make it easier to program. For example, port5 could be called leftMotor and port6 could be called rightMotor. There can be no spacing for port names with multiple words. Programmers use comments to help them know what each line of code means. Two forward slash lines are used to insert comments that are not read by the robot’s cortex. Figure 2 has example comments beside the code. Comments are automatically written in green.

Figure 2. Example Comments

The wait1Msec command tells the cortex how long to keep sending power to the port. The code in Figure 1 tells the cortex to send power for 3000 milliseconds or 3 seconds before shutting off. The time to wait is placed in between parentheses ( ).

Testing computer programs can be a long tedious task. Programmers use virtual computer worlds to increase the speed of testing and rewriting programs. Virtual worlds allow programmers to visually see the behavior of a robot following programmed instructions. Once a programmer completes the robot’s virtual behavior, the program is downloaded and tested using the physical robot.

What programmable code will you use to mobilize a robot? How will you test it using a virtual world?

Materials

| Per student: Agriscience Notebook Engineering Notebook ROBOTC Reference Sheet Computer with ROBOTC and Robot Virtual Worlds Pen | Per team of three students Model tractor Shoe box USB cable |
| --- | --- |

Procedure

Work individually to complete Part One through Part Four by writing and testing computer programs for a virtual robot. Then complete Part Five with a partner by writing, downloading, and testing a program for a robot. Refer to the ROBOTC Reference Sheet for ROBOTC language information. Answer analysis questions in your Engineering Notebook as you complete the activity.

Part One – Coding Tutorial

Go to http://www.robotc.net/tutor/Cortex/cortexunits.php?platform=Cortex on a computer.

Click on ROBOTC Rules.

Read through the instructions and complete the ROBOTC Rules assignments for the ROBOTC rules tutorial.

Click on Movement Fundamentals.

Read through the instructions and complete the Movement Fundamentals assignments for the ROBOTC rules tutorial

Part Two – Drive Forward

- Open ROBOTC on a computer.

Select Robot and choose Compiler Target>>>Virtual Worlds.

Select New File.

Write the following program between the task main brackets.

| motor[port1] = 127; | //left motor full speed |
| --- | --- |
| motor[port10] = 127; | //right motor full speed |
| wait1Msec(5000); | //Motors run for five seconds |

Select Compile Program.

Save program as 4_2_3YourInitials.

Select Download to Robot.

Virtual Worlds for VEX should appear on the screen.

Select Robots and choose VEX Clawbot.

Select Utility>>>Imperial Distance Utility and choose Start Activity.

Press the Play button to observe the robot’s behavior.

Did the robot behave as you expected? Why?

What needs to be changed for the robot to move forward?

Minimize Robot Virtual Worlds so you can view your program.

Select Robot and choose Motors and Sensor Setup.

Enter the following port names.

Port1 – leftMotor

Port10 – rightMotor

Choose VEX 269 Motor from the drop-down menu for each motor Type.

Click on the reverse box for port1 and click on Apply and OK.

Why is the left motor reversed?

Change the speed of the robot by entering 63 for each port speed.

Select Compile Program.

Select Download to Robot.

Run the program on the Imperial Distance Utility.

How far did the robot travel?

How many revolutions did each wheel make? Hint: The wheel diameter is 4 inches, circumference of the wheel = Dπ.

Save and close the current ROBOTC program.

Part Three – Turning the Robot

- Enter the following ROBOTC program. Add comments for each instructional line. Change the port names, motor, and direction motor spins as you did in Steps 12 – 14 in Part Two.

wait1Msec(3000);

motor[leftMotor] = 63;

motor[rightMotor] = 63;

wait1Msec(3000);

motor[leftMotor] = 23;

motor[rightMotor] = 63;

wait1Msec(1850);

Answer the following prediction questions.

What will happen when you first start the program?

Why can you use the motor name instead of the port number?

What will be the robot’s behavior?

Select Compile Program.

Select Download to Robot.

Run the program in Robot Virtual Worlds using the Imperial Distance Utility.

How far did the robot travel?

How did the robot’s behavior compare to your predictions?

Part Four – Point and Swing-Turn

Write code and observe a point and swing turn of a virtual robot.

- Choose New File in ROBOTC.

Save the file as 4_2_3SwingTurn_YourInitials.

Convert the Swing-Turn Pseudocode below into programmable code using ROBOTC. Include comments in your code.

Run the program in Robot Virtual Worlds using the Imperial Distance Utility.

| Swing-Turn Pseudocode | Point-Turn Pseudocode |
| --- | --- |
| - Wait for three seconds. Power the right motor at half power for three seconds. Power the left motor at half power for three seconds Turn both motors off for one second. Apply half power to the right motor for three seconds. Apply no power to the left motor for three seconds. Turn both motors off. | - Wait for three seconds. Power the right motor at half power for three seconds. Power the left motor at half power for three seconds Turn both motors off for one second. Apply half power to the right motor for three seconds. Apply half power to the left motor in reverse for three seconds. Turn both motors off. |

- Repeat Steps 1 – 4 for the Point-Turn Pseudocode. Save the program as 4_2_3PointTurnYourIntials.

Record a comparison of a point-turn and swing-turn in your Engineering Notebook.

Part Five – Robot Download

Work with your teammates and follow the steps below to download 4_2_3PointTurnYourIntials program to your model tractor. Then run the program on your model tractor.

- Select File and choose Open and Compile.

Select the file 4_2_3PointTurnYourInitials.

Select Save.

Select Robot and choose Compiler Target>>>Physical Robot.

Place the model tractor on a shoe box so chassis is in contact with box and wheels are not in contact with the ground to prevent the tractor from moving during the download process.

Use a USB cable to connect the cortex to the computer.

Make sure the battery is charged and connected to the cortex.

Turn the cortex on.

Select Robot and choose Vex Cortex Communication Mode>>>USB Only.

Select Compile Program.

Select Download to Robot.

Program will download.

Do not debug. Debugging will cause the model tractor to start reading the program.

Shut off the cortex.

Disconnect USB cord from cortex and computer.

Bring the model tractor to the area assigned by your teacher.

Set the model tractor at the start location.

Turn on the model tractor and observe its behaviors. Be prepared to interfere with the model tractor if it goes outside of the designated area.

How did the virtual and physical model tractor behaviors differ? Why do you believe this is so?

Conclusion

- What is the process for programming an automated machine?

- Why is it beneficial to test programs in a virtual world?

- What factors affect the quality of a program for an automated machine?