ROBOTC Reference Sheet

Coding rules

All code starts with task main()

ROBOTC reads capitalized and lower-case letters differently – capitalization matters

All commands are found between curly brackets {}

Command statements end with a semicolon ( ; )

Statements run in the order written

Programmers use whitespace to help interpret code

Whitespace does not affect how a machine reads code

Port locations are placed between square brackets [ ]

Time is placed between parenthesis ( )

Forward (//) slashes are used for placing comments, PLCs do not read comments

Coding colors

ROBOTC will automatically change text to different colors to differentiate types of code.

Blue – control structures

Red – brackets, parenthesis, number values, and semi-colons

Green – comments

Comparison operators

Used for programming to generate a true or false statement.

> (greater than)

< (less than)

== (equal to)

Logical operators

Used for programming to combine multiple true values into one single truth.

&& (and)

|| (or)

Control structures

Used for programming to tell a machine when to run a specific command.

While loops consist of three parts

Begins with while

Condition place in parenthesis ( )

Commands placed between { }

Infinite while loops use the condition – while(1==1)

Sensor controlled loops use condition of sensor – while(SensorValue[SensorName] == 1)

Counter controlled loops have the following codes

States an initial count – int count = 0;

Checks if the count is within range before moving to command – while (count < 6)

Adds a value to the count after command is completed – count = count +1;

If – else statements

States a condition if(condition)

If condition is true, will follow commands in curly brackets { }

If condition is false, will end program or move to commands under else condition – else