Name_____________________________________________________

Activity 4.2.4 Bump and Loop

Purpose

In Activity 4.2.3 Virtual Reality, you programmed a robot to move through an open environment. What would happen if the environment were to change with obstacles the robot may hit? Sensors allow machines to be aware of their surroundings and adapt to their environment. How are sensors programmed?

Robotic machines need clear and concise instructions to follow. A robotic machine must react to yes/no or true/false conditions in an environment to perform a behavior. There is no “gray” area when programming a machine to make a decision. Statements that can only be true or false are Boolean statements. When making Boolean statements, they must be written as statements and not questions. For example, instead of asking “is the grass green”, the statement would be “the grass is green”. Then the machine would sense the truth of that statement, true (it is green) or false (it is not green).

ROBOTC uses control structures to make decisions about what code should be followed and what code should be ignored. Conditional statements such as while loops, tell a machine to make a decision based upon a Boolean statement. The statement is true or false at any moment in time. For example, “the touch button is pressed in” will either be true or false. Either the button is pressed, or it is not.

| While loops tell a machine to repeat a behavior as long as a condition is true. Figure 1 is an example of a while statement used for a touch sensor. The touch sensor has two values: 1 if pressed in, and 0 if not pressed in. The program tells the robot to continue running the motors as long as the touch sensor is not pressed in. Once the sensor is pressed, the value becomes 1, and the robot will stop. Sometimes the programmer may want the machine to repeat behaviors. This can be done with infinite loops by making a statement which is always true or false. The code in Figure 2 tells the model tractor to move forward at half speed as long as 1=1. One will always be equal to one, so the robot will never stop trying to move forward. Figure 3 shows a loop limiting the number of times a behavior will be repeated. The behavior will repeat four times and then move on to the next line of instruction. For this case, there are no additional instructions, so the program will end. |  |
| --- | --- |
|  | Figure 1. While Loop |
|  | Figure 2. Infinite Loop |

Figure 3. Behavior Count

| Besides pseudocode, programmers use flow charts to explain the flow of a program when a machine makes decisions. Figure 4 shows an example flow chart from the start to the end of a program. Rectangles represent the actions a machine makes, and diamonds represent decisions made. How does the touch sensor and while command control machine behaviors? How does a flow chart describe a program? Materials Per student: Agriscience Notebook Engineering Notebook ROBOTC Reference Sheet Pen Computer with ROBOTC and Robot Virtual Worlds software |  |
| --- | --- |
|  | Figure 4. Flow Chart |

Procedure

Work individually programming a touch sensor for a virtual robot using while loop conditional statements. Refer to the ROBOTC Reference Sheet for ROBOTC language information. Include comments in all programs so they are easy to understand. Use the Huge Table in the Utilities section of Robot Virtual Worlds for all your tests.

Part One – While Loop

Open the ROBOTC program on a computer.

Set up the ports for the left and right drive motors of the virtual robot.

Enter the following port names.

Port1 – leftMotor

Port10 – rightMotor

Choose VEX 269 Motor from the drop-down menu for each motor Type.

Click on the reverse box for Port1 and click on Apply and OK.

Write a code for the virtual robot to travel forward at full speed for ten seconds.

Save program as 4_2_4YourInitials.

Select Compile Program.

Choose Download to Robot.

Virtual Worlds for VEX should appear on the screen.

Select Robots and choose VEX Clawbot.

Click on Utility>>>Huge Table and choose Start Activity and observe the robot’s virtual behavior.

Minimize Robot Virtual Worlds.

Select Robot and choose Motors and Sensor Setup.

Click on VEX Cortex Digital Sensors 1 – 12.

Enter the following port names.

digtl6 – touchSensor

Change the Sensor Type for digtl6 to Touch and select OK.

Change the program by adding code so the robot stops once it touches the wall. Add comments to describe your program.

Compile, download, and run the program in Robot Virtual Worlds and sketch a flow chart in your Engineering Notebook explaining the decision process and actions of the robot.

Part Two – Loop

- Change the program so the robot moves forward at half speed when the touch sensor is not pressed and makes a point turn to the right for two seconds when the button is pressed.

- Record pseudocode and a flow chart in your Engineering Notebook describing the behavior.

- Compile, download, and run the program in Robot Virtual Worlds.

- Change the program so it is in an infinite loop and the Clawbot turns every time it touches a wall.

- Record a flow chart in your Engineering Notebook that describes the infinite loop behavior.

Compile, download, and run the program in Robot Virtual Worlds to test your program.

Rewrite the program and repeat Step 6 if the Clawbot does not run infinitely.

Stop the program running in Robot Virtual Worlds.

Change the program so the robot repeats the behavior five times.

Compile, download, and run the program in Robot Virtual Worlds.

Show your teacher the virtual robot following the program.

Save the final program as 4_2_4CountYourInitials.

Conclusion

- How can a machine’s behavior be programmed to stop or repeat?

- How can sensors be used to change a machine’s behavior?

- How could a sensor be used in agriculture?

- Why are sensors an essential component of programming a machine?

- How does a flow chart explain the decisions a robotic machine is making?