Name_____________________________________________________

Activity 4.2.5 Sonar Sense

Purpose

New vehicles have sensors that increase a driver’s safety by detecting objects around the vehicle. You may have heard an alarm in a car alert a driver as they were approaching another object. The sensor uses sonar technology. Sonar sensors, also called ultrasonic range finders, measure distances between the vehicle and an object by sending out an ultrasonic pulse and measuring how long it takes for the sensor to hear the pulse echoing off the object. A robot uses a sonar sensor in the same way as any other vehicle, such as a truck or tractor.

Unlike the touch sensor with only two possible readings, sonar sensors have multiple numerical readings.

Since sonar sensors compute more than two numbers, programmers use comparison operators to help the robot make a decision. ROBOTC symbols, such as >, <, and =, compare data to determine if a statement is true. Logical operators like and (&&) and or (||) are used when more than one condition is considered. Figure 1 shows a logical operator used for the sonar sensor. The robot follows the code if objects are greater than 100cm away or senses no object. No object sensed results in the sensor generating a -1 value.

Sonar sensors can be programmed using while loops or if-else statements. If-else statements allow a robot to make a decision. The robot evaluates a condition stated between parenthesis and follows the commands between braces. If the condition is not true, then the commands between the braces are ignored. If- else statements allow the robot to follow commands when a condition is false. Figure 1 has an example of if-else commands using the sonar sensor. The comments explain the logic of the program.

Figure 1. Sonar Sensor

What behaviors can you assign a robot using the sonar sensor? How does a flow chart help organize the program?

Materials

| Per team of three students: Model tractor from Lesson 4.1 Mobile Systems Sonar sensor Two #8-32 button head screws ¼″ long Two #8-32 keps nuts 5/64″ hex wrench ¼″ wrench or nut driver USB cable Shoe box | Per student: Agriscience Notebook Engineering Notebook ROBOTC Reference Sheet Pen Computer with ROBOTC and Robot Virtual Worlds software |
| --- | --- |

Procedure

Work individually to program a virtual robot to sense its surroundings with the sonar sensor. Refer to the ROBOTC Reference Sheet for ROBOTC language information. Record all programming notes, pseudocode, and flowcharts in your Engineering Notebook. Then work with a partner to download and test the program using the model tractor.

Part One – Sonar Program

Open ROBOTC on a computer.

Select Robot and choose Compiler Target>>>Virtual Worlds.

Select Robot and choose Motors and Sensor Setup.

Setup the ports for the left and right drive motors of the virtual robot.

Enter the following port names.

Port1 – leftMotor

Port10 – rightMotor

Choose VEX 269 Motor from the drop-down menu for each motor Type.

Click on the reverse box for port1 and click on Apply and OK.

Name dgtl8 sonarSensor.

Change dgtl8 Sensor Type to Sonar (in).

Write a program using the sonar sensor and while statements to stop the robot once it gets within 20 inches of an object.

Select Compile Program and save as 4_2_5SonarYourInitials.

Select Download to Robot. Virtual Worlds for VEX should appear on the screen.

Select Robots and choose VEX Clawbot.

Select Utility>>>Huge Table and choose Start Activity.

Press the Play button to observe the robot’s behavior.

Press the view sensor button in the lower right corner to view the sonar sound waves.

View the robot behaviors to ensure it is working properly.

Check the program for errors if it is not working.

Ask your teacher for help if you cannot get your program to work after the third try.

Change the program so the robot turns 90 degrees once it comes within 20 inches of an object and then moves forward again. Use if-else statements to repeat the behavior. Record the pseudocode and flowchart for the program in your Engineering Notebook.

Save the program and close ROBOTC on the computer.

Part Two – Sonar Download and Test

Work with your teammates to download and test the program with your model tractor.

- Use the hex wrench and ¼″ wrench to attach the sonar sensor to the front of the model tractor.

Use button head screws and nuts to attach the sensor.

Plug the sonar sensor into the cortex ports assigned in the program. Negative wire lead on each cable should be furthest from the center of the cortex.

Input cable to digital8

Output cable to digital9

Select File and choose Open and Compile.

Select the file 4_2_5SonarYourInitials.

Click Save.

Select Robot and choose Compiler Target>>>Physical Robot.

Place the model tractor on a shoe box so the chassis is in contact with the box and wheels are not in contact with the ground to prevent the tractor from moving during the download process.

Use a USB cable to connect the cortex to the computer.

Make sure the battery is charged and connected to the cortex.

Turn the cortex on.

Select Robot and choose Vex Cortex Communication Mode>>>USB Only.

Select Compile Program.

Select Download to Robot.

Program will download.

Do not debug. Debugging will cause the model tractor to start reading the program.

Shut off the cortex.

Disconnect USB cord from cortex and computer.

Bring the model tractor to the area assigned by your teacher.

Set the model tractor at the start location.

Turn on the model tractor and observe its behaviors. Be prepared to interfere with the model tractor if it goes outside of the designated area.

Record the virtual tractors behaviors and any changes you would make to the program in your Engineering Notebook.

Conclusion

- What is the purpose of the sonar sensor?

- What commands did you use to successfully complete the challenge?

What are some agricultural uses for a sonar sensor?