Name_____________________________________________________

Activity 4.3.2 Pneumatic Schematic

Purpose

When engineers plan a fluid power system, they draw the system in the form of a schematic. Fluid power schematics are similar to electrical schematics used in Lesson 2.2 Electrical Systems.

Fluid power schematics have symbols representing the components found in the system. Common components found in a fluid power system include a compressor, valves, hoses, and actuators. Solid lines in a fluid power schematic represent hoses connecting each component. A variety of symbols represent how valves direct fluid through a system. Actuators convert the energy from the fluid into rotational or linear mechanical energy. Motors produce rotational movement, while cylinders produce linear movement. The type of actuator used will affect the valve chosen for the system.

Valves have another type of actuator used to move the valve into different positions. Valve actuators are activated manually or mechanically. Manual valve actuators include push buttons and switches. Mechanical valve actuators include springs, solenoids, and air pilots. The valves direct air through ports and ways. Ports and ways direct air internally through the system or exhaust it out of the system.

Why are different types of valves and actuators used in a pneumatic system?

Materials

Per student: Computer with Pneumatic Simulator software Agriscience Notebook Engineering Notebook Pen

Procedure

Use a pneumatic simulator to design fluid power systems with actuators, valves, and hoses. Assume the supply source is a compressor consisting of an accumulator, regulator, and pressure gauge. Draw a schematic of each system in your Engineering Notebook.

Part One – Single Acting Cylinder

Open Pneumatic Simulator on your computer.

Review the symbols at the bottom of the screen. Use Table 1 to determine what each symbol represents.

| Table 1. Pneumatic Program Symbols |  |
| --- | --- |
| Symbol | Representation |
|  | Delete tubes |
|  | Draw tubes |
|  | Drag items |
|  | Activate |
| Images from VMS Pneumatic Animations |  |

Select the Info button at the bottom of the screen and review each component available to use in the animation.

Sketch and label each component in your Engineering Notebook.

Click and drag a single-acting, spring-return cylinder on to the work area.

The white space to the left of the symbols is the work area to complete your schematic.

At the bottom of the screen select Draw tubes.

| Click and hold the mouse button on the air supply port and drag a line to the port on the single-acting cylinder. Your schematic should look similar to Figure 1. Click the green button at the bottom of the screen to activate the system. Note the color of the hose. Red represents pressurized air in the system and blue represents non-pressurized air exhausted from the system. Click select Reset at the bottom of the screen. Place a single-acting spring-return cylinder and two-position, push-button spring-return, three-way valve on the work space. Sketch the schematic in your Engineering Notebook. Circle the valve position when the button is not activated. On the workspace and in your Engineering Notebook, connect hoses to the valve so the cylinder extends when the button is pushed. |  |
| --- | --- |
|  | Figure 1. Single Acting Cylinder Image from VMS Pneumatic Animations |

Hint: Connect hoses to the flow box closest to the spring.

Click the green button at the bottom of the screen to activate the system.

Record the behavior of the system in your Engineering Notebook.

Click the green button to deactivate the system.

Select the Drag items button at the bottom of the screen.

Add an exhaust to the open port on the valve so air can be exhausted when the button is not pushed.

Add the exhaust port on your schematic in your Engineering Notebook.

Activate and test the system using the green button.

Select Reset once you have constructed your system and it works correctly.

Redesign the system so the cylinder actuator is extended when the push-button is not pressed and is retracted when the push button is pressed.

Sketch the schematic in your Engineering Notebook.

Click Reset once you have constructed and tested your system.

Part Two – Double Acting Cylinder

Use Pneumatic Simulator to design a system that has a double acting cylinder controlled by a dual push button valve. The actuator should extend when pressing the button on the right and retract when pressing the button on the left. Then, in your Engineering Notebook, draw a schematic of your system and record the type of components used. Test your system using the Pneumatic Simulator.

Part Three – Dual Control

Design a system that has a double acting cylinder with one button controlling the extension of the actuator and another button controlling the retraction. Each button is found in a different location. Use the following components.

One double acting cylinder

Two two-position, push-button, spring-activated, three-way valves with one exhaust port

One two-position, double-pilot, four-way valve with two exhaust ports

In your Engineering Notebook, draw a schematic of your system. Then test your system using the pneumatic simulator.

Conclusion

- How is a schematic used to design a fluid power system?

- What do the arrows in the valve schematic represent?

- How are three-way and four-way valves different? When are each of them used?

- What information can you obtain from a fluid power schematic?